3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230809

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3260 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC RRC testcase 8.2.3.17.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current TTCN implementation for Steps 1 and 6 is not inline with 38.523-1 Table 8.2.3.17.1.3.3-3. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN to be inline with Prose |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not inline with Prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.17.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | Function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Current TTCN implementation for Steps 1 and 6 is not inline with 38.523-1 Table 8.2.3.17.1.3.3-3. This needs to be corrected. |
| Summary of change | Updated the TTCN to be inline with Prose |
| TTCN module | \ENDC\8_2_3\RRCMeasurment_ENDC_EUTRA.ttcn |

function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { …… //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(neighborCells, cs_ReportQuantityNR_r15(false, false, false)); //@sic R5-222272 sic@ //Table 8.2.3.17.1.3.3-2 …… //@siclog "Step 6-7" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(pSCell); v_MeasConfig.reportConfigToAddModList := cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 8" siclog@ …… }

After Change:

function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { …… //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(neighborCells, cs_ReportQuantityNR_r15(true, false, false)); //@sic R5-222272 sic@ // WI=884135 //Table 8.2.3.17.1.3.3-2 …… //@siclog "Step 6-7" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(pSCell, cs_ReportQuantityNR_r15(false, false, false)); //// WI=884135 v_MeasConfig.reportConfigToAddModList := cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 8" siclog@ …… }