3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230818

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3149 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MDT test case 8.1.6.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For RRCSetupComplete at step 14 the contents of ue-MeasurementsAvailable-r16 is given as “UE-MeasurementsAvailable-r16 with condition LOG, specified in 38.508-1 table 4.6.5-15. Other fields apart from “logMeasAvailable-r16” are specified as “not checked” and may be reported by the UE if it has stored information for e.g. previous RLF failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use default value (*) for all fields of cr_UE_MeasurementsAvailable_r16 except p_LogMeasAvailable_r16 (instead of omit) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_3_NR5GC_TestBody() |
| --- | --- |
| Reason for change | For RRCSetupComplete at step 14 the contents of ue-MeasurementsAvailable-r16 is given as “UE-MeasurementsAvailable-r16 with condition LOG, specified in 38.508-1 table 4.6.5-15. Other fields apart from “logMeasAvailable-r16” are specified as “not checked” and may be reported by the UE if it has stored information for e.g. previous RLF failure. |
| Summary of change | Use default value (*) for all fields of cr_UE_MeasurementsAvailable_r16 except p_LogMeasAvailable_r16 (instead of omit) |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_3_NR5GC_TestBody() runs on NR5GC_PTC { ………………………………… //@siclog "Steps 11-19" siclog@ // Steps 1 to 9 of the generic radio bearer establishment procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_,omit,omit,omit,omit)))); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Step 20" siclog@ // The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, cs_TimingInfo_Now, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_))); // UEInformationRequest - condition Logged MDT ……………………………… }

After Change:

function f_TC_8_1_6_1_2_3_NR5GC_TestBody() runs on NR5GC_PTC { ………………………… //@siclog "Steps 11-19" siclog@ // Steps 1 to 9 of the generic radio bearer establishment procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell11); f_NR_RRCSetup_Def(nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_)))); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_ReceivedMsg); //@siclog "Step 20" siclog@ // The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, cs_TimingInfo_Now, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_))); // UEInformationRequest - condition Logged MDT …………………………………… }