3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230820

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4751 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The current function, fl_NBIOT_CheckSupportedBands, can let a non-conformant UE pass the testcase when the first reported band matches the first band configured via PICS (eg. UE supports band 3 only while PICS bands are set as 3, 4, 5, 12). - The current function, fl_NBIOT_CheckSupportedBands, will also not evaluate the correct PICS band because of one of the variables, v_MaxBandsReported, reporting the length of the list and not the maximum band reported and can cause the TC to not report a failure for a band that should have triggered a failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Rewritten function to not use for loops inbedded in for loops for clarity. Use of comparison of two bitstrings (v_SuppBandsFromPICS and v_SuppBandsReported) to determine if a mismatch between reported bands and configured bands from PICS. Additionally, a check on if the UE did report any bands at all. Please see below for details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A non-conformant UE may pass the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

  - Correction to function fl_NBIOT_CheckSupportedBands in f_TC_22_4_13

| Function name | fl_NBIOT_CheckSupportedBands |
| --- | --- |
| Reason for change | - The current function, fl_NBIOT_CheckSupportedBands, can let a non-conformant UE pass the testcase when the first reported band matches the first band configured via PICS (eg. UE supports band 3 only while PICS bands are set as 3, 4, 5, 12). - The current function, fl_NBIOT_CheckSupportedBands, will also not evaluate the correct PICS band because of one of the variables, v_MaxBandsReported, reporting the length of the list and not the maximum band reported and can cause the TC to not report a failure for a band that should have triggered a failure. |
| Summary of change | - Rewritten the function to not use for loops inbedded in for loops for clarity. Use of comparison of two bitstrings (v_SuppBandsFromPICS and v_SuppBandsReported) to determine if a mismatch between reported bands and configured bands from PICS. Please see below for details. Creation of a new variable named v_MaxBandReported to hold an integer of the final (largest) band reported by the UE. Creation of a new variable named v_NumBandsReported to hold the number of bands reported by the UE. Creation of a new variable v_NumSupportedBands to hold the number of supported bands from tsc_NBIOT_SupportedBands Additionally, a check on if the UE did report any bands at all is implemented. |
| TTCN module | NBIOT_RRC_ControlPlane.ttcn |
| MCC160 Comment |  |

Before

function fl_NBIOT_CheckSupportedBands(SupportedBandList_NB_r13 p_SupportedBandList)

{

var integer i;

var integer j;

var integer v_Band_r13;

var integer v_BitStringIndex;

var bitstring v_SuppBandsFromPICS := f_DeriveSuppNBIoTBandsFromPics ();

var integer v_MaxBandsReported := lengthof(p_SupportedBandList);

var bitstring v_SuppBandsReported := int2bit(0, v_MaxBandsReported); // 1... maxbands

// There may be more bands reported than there are PICS defined for bands

for (i:= 0; i < lengthof(p_SupportedBandList); i:= i+1) {

if (tsc_NBIOT_SupportedBands[i] == '1'B) { // Don't check anything if band not supported @sic R5s170290 sic@

if (v_SuppBandsFromPICS[i] =='1'B) { // PICS for band i

for (j := 0; j < v_MaxBandsReported; j := j + 1) { // check against all reported bands

v_Band_r13 := p_SupportedBandList[j].band_r13;

v_BitStringIndex := v_Band_r13 - 1;

if (i == v_BitStringIndex) {

if (v_SuppBandsReported[v_BitStringIndex] == '1'B) {

f_SetVerdict(fail, __FILE__, __LINE__, "support of band nr. " & int2str(v_Band_r13) &  " is reported more than once"); // error: duplicate found

} else {

v_SuppBandsReported[v_BitStringIndex] := '1'B;

}

}

else {// do nothing

}

}

if (v_SuppBandsReported[i] != '1'B) { // PICS set,  but expected band was not in the list

f_SetVerdict(fail, __FILE__, __LINE__, "expected (per PICS) band nr. " & int2str(i + 1) &  " is not reported"); // error: PICS set, but expected band not reported

}

}

else { // PICS is not set => non support

for (j := 0; j < v_MaxBandsReported; j := j + 1) { // check against all reported bands

v_Band_r13 := p_SupportedBandList[j].band_r13;

v_BitStringIndex := v_Band_r13 - 1;

if (i == v_BitStringIndex) {

f_SetVerdict(fail, __FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_Band_r13)); // PICS mismatch

}

else { // do nothing

}

}

}

} // if band not supported

}

}

After

function fl_NBIOT_CheckSupportedBands(SupportedBandList_NB_r13 p_SupportedBandList)

{

if (lengthof(p_SupportedBandList) == 0){

f_SetVerdict(fail, __FILE__, __LINE__, "No bands were reported by the UE"); // error: No bands reported by the UE

}//WA#117600

var integer i;

var integer v_Band_r13;

var integer v_BitStringIndex;

var bitstring v_SuppBandsFromPICS := f_DeriveSuppNBIoTBandsFromPics ();

var integer v_NumBandsReported := lengthof(p_SupportedBandList); // Number of bands reported by UE // There may be more bands reported than there are PICS defined for bands

var integer v_MaxBandReported := p_SupportedBandList[v_NumBandsReported - 1].band_r13; // Final band reported, to compute the length of bitstring

var integer v_NBIOT_NumSupportedBands := lengthof(tsc_NBIOT_SupportedBands); // The number of supported bands to test

var bitstring v_SuppBandsReported := int2bit(0, v_MaxBandReported);

for (i := 0; i < v_NumBandsReported; i := i + 1) { // setup of bitstring of UE's reported bands to compare with PICS bitstring bands

v_Band_r13 := p_SupportedBandList[i].band_r13;

v_BitStringIndex := v_Band_r13 - 1;

if (v_SuppBandsReported[v_BitStringIndex] == '1'B) { // if UE reports the same band twice

f_SetVerdict(fail, __FILE__, __LINE__, "support of band " & int2str(v_Band_r13) &  " is reported more than once"); // error: duplicate found

}

v_SuppBandsReported[v_BitStringIndex] := '1'B

}

//if v_MaxBandReported is less than v_NBIOT_NumSupportedBands, append 0 bits to v_SuppBandsReported bitstring until the same size

if (v_MaxBandReported < v_NBIOT_NumSupportedBands){

for(i := v_MaxBandReported; i < v_NBIOT_NumSupportedBands; i := i + 1) {

v_SuppBandsReported[i] := '0'B;

}

}

// For every supported band check band set in PICS against band reported

for (i := 0; i < v_NBIOT_NumSupportedBands; i:= i+1) {

if (v_SuppBandsFromPICS[i] != v_SuppBandsReported[i]) { // if UE reported bands and PICS set bands don't match

f_SetVerdict(fail, __FILE__, __LINE__, "PICS and reported band list mismatch: PICS and UE eband: " & int2str(i+1) & " is mismatched. UE reported: " & bit2str(v_SuppBandsReported[i]) & ", while PICS was set to: " & bit2str(v_SuppBandsFromPICS[i])) // error: PICS and UE band list do not match

}

}

}