3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230826

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0918 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 11.4.x and 10.x test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TTCN is modified to update DRBMUX before receiving INVITE from UE in case of emergency Call |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.x and 10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	9

2.5	Change 5	12

2.6	Change 6	14

2.7	Change 7	15

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 of IMS_NR5GC_EmergencyCallTestcases.ttcn and IMS_Procedures_5G.ttcn

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required.

## Change 1

| Function name | f_NR5GC_EstablishEmgCall_IMS2 |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_EstablishEmgCall_IMS2(boolean p_GeolocationUsage := false, boolean p_CheckGeolocationAbsent := false) runs on IMS_PTC return SipUrl { /* IMS implementation for PDN2 - Emergency registration plus emergency call establishment */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := p_GeolocationUsage; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ var boolean v_CheckGeolocationAbsent := p_CheckGeolocationAbsent; f_IMS_IMS_WaitForTrigger(OtherPDN); <<skipped code>> //@siclog "Step 6" siclog@ //Step 1 of annex A.6 (emergency call) //Check: Does the UE send a correctly composed INVITE request? v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A7, v_GeolocationUsage); // @sic R5s220531 sic@ // absence of geolocation is to be checked v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; if (v_CheckGeolocationAbsent) {f_IMS_CheckGeolocationAbsent(v_InviteRequest);} // @sic R5-223467 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); …………………

After Change:

function f_NR5GC_EstablishEmgCall_IMS2(boolean p_GeolocationUsage := false, boolean p_CheckGeolocationAbsent := false) runs on IMS_PTC return SipUrl { /* IMS implementation for PDN2 - Emergency registration plus emergency call establishment */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := p_GeolocationUsage; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ var boolean v_CheckGeolocationAbsent := p_CheckGeolocationAbsent; f_IMS_IMS_WaitForTrigger(OtherPDN); <<skipped code>> //@siclog "Step 6" siclog@ //Step 1 of annex A.6 (emergency call) //Check: Does the UE send a correctly composed INVITE request? f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // Anritsu CR for 10.x v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A7, v_GeolocationUsage); // @sic R5s220531 sic@ // absence of geolocation is to be checked v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; if (v_CheckGeolocationAbsent) {f_IMS_CheckGeolocationAbsent(v_InviteRequest);} // @sic R5-223467 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 7" siclog@ //Step 2 of annex A.6 (emergency call) //@siclog "Step 8" siclog@ //Step 3 of annex A.6 (emergency call) //@siclog "Step 9" siclog@ //Step 4 of annex A.6 (emergency call) //@siclog "Step 10" siclog@ //Step 5 of annex A.6 (emergency call) //Check: Does the UE send ACK? f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); …..

## Change 2

| Function name | function f_TC_10_3_NR5GC_IMS2 |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_10_3_NR5GC_IMS2() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var IMS_DATA_REQ v_IMS_DATA_REQ; var SipUrl v_ContactUrlUE; <<skipped code>> // Emergency call setup: 34.229-5 Annex A.6 // @siclog "Step 11" siclog@ // Check: Does the UE send correctly composed INVITE request for emergency call? (Step 1 of Annex A.6 v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(); // INVITE for an Emergency call @sic R5s220215 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12-14" siclog@ // SS sends 100 Trying, followed by 180 Ringing, followed by 200 OK. (Steps 2-4 of Annex A.6) // @siclog "Step 15" siclog@ // Check: Does the UE acknowledge? (Step 5 of Annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); // @sic R5s220215 sic@ // Inform NR: Emergency Call established f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15"); …..…………….

After Change:

function f_TC_10_3_NR5GC_IMS2() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var IMS_DATA_REQ v_IMS_DATA_REQ; var SipUrl v_ContactUrlUE; <<skipped code>> // Emergency call setup: 34.229-5 Annex A.6 // @siclog "Step 11" siclog@ // Check: Does the UE send correctly composed INVITE request for emergency call? (Step 1 of Annex A.6) f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // Anritsu CR for 10.x v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(); // INVITE for an Emergency call @sic R5s220215 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12-14" siclog@ // SS sends 100 Trying, followed by 180 Ringing, followed by 200 OK. (Steps 2-4 of Annex A.6) // @siclog "Step 15" siclog@ // Check: Does the UE acknowledge? (Step 5 of Annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); // @sic R5s220215 sic@ // Inform NR: Emergency Call established f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15"); …..…………….

## Change 3

| Function name | function f_TC_10_6_NR5GC_IMS2() |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | SS is now configured for the CRNTI based RACH procedure with the CRNTI value of the cell2 prior to step 24. |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

…… function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var IMS_DATA_REQ v_IMS_DATA_REQ; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (present) SDP_Message v_SDP_Message_Step1; var template (value) SDP_Message v_SDP_Message_Step4; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 2-11" siclog@ // Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 are performed // EXCEPTION: In parallel to the events described in steps 12-16 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // @siclog "Steps 12-16" siclog@ // Steps 1-5 of Annex A.6 happens f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17-18" siclog@ //Steps 1 -2 of Annex A.8 happens v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); // @sic R5s220555 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 18"); f_IMS_CC_EndFirstOfMultipleCalls(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall);// @sic R5-225426 sic@ f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //EXCEPTION: In parallel to the events described in steps 23-27 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. //@siclog "Steps 23-27" siclog@ //Step 1 of Annex A.6 happens //Check: Does UE send INVITE message for emergency call? //Steps 2-5 of Annex A.6 happens v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); /* @sic R5s130988 change 6/7: has to be done after the INVITE has been received and the address information has been initialised even for A_2_1_A6 sic@ */ v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1); /* @sic R5w140112 sic@ sic@ */ v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfo(v_SDP_Message, {AMR_16000}); /* @sic R5-222021: f_IMS_SDP_GetAudioCodecInfo sic@ */ // Step 2 of Annex A.6. SS sends a 100 Trying provisional response. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine100, f_IMS_InviteResponse_100_MessageHeaderTX(v_InviteRequest)))); // Step 3 of Annex A.6. SS sends a 180 Ringing. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine180, f_IMS_InviteResponse_180_MessageHeaderTX(v_InviteRequest, -, -, -, v_EmergencyType)))); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_EmergencyCall_NormalService, -, IPCAN_SpeechAndEmergencyCall); // @sic R5-225426, R5s221136 sic@ …..

After Change:

…… function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var IMS_DATA_REQ v_IMS_DATA_REQ; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (present) SDP_Message v_SDP_Message_Step1; var template (value) SDP_Message v_SDP_Message_Step4; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 2-11" siclog@ // Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 are performed // EXCEPTION: In parallel to the events described in steps 12-16 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // @siclog "Steps 12-16" siclog@ // Steps 1-5 of Annex A.6 happens f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17-18" siclog@ //Steps 1 -2 of Annex A.8 happens v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); // @sic R5s220555 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 18"); f_IMS_CC_EndFirstOfMultipleCalls(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall);// @sic R5-225426 sic@ f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //EXCEPTION: In parallel to the events described in steps 23-27 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. //@siclog "Steps 23-27" siclog@ //Step 1 of Annex A.6 happens //Check: Does UE send INVITE message for emergency call? //Steps 2-5 of Annex A.6 happens v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); /* @sic R5s130988 change 6/7: has to be done after the INVITE has been received and the address information has been initialised even for A_2_1_A6 sic@ */ v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1); /* @sic R5w140112 sic@ sic@ */ f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_EmergencyCall_NormalService, -, IPCAN_SpeechAndEmergencyCall); // Anritsu CR for 10.x v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfo(v_SDP_Message, {AMR_16000}); /* @sic R5-222021: f_IMS_SDP_GetAudioCodecInfo sic@ */ // Step 2 of Annex A.6. SS sends a 100 Trying provisional response. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine100, f_IMS_InviteResponse_100_MessageHeaderTX(v_InviteRequest)))); // Step 3 of Annex A.6. SS sends a 180 Ringing. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine180, f_IMS_InviteResponse_180_MessageHeaderTX(v_InviteRequest, -, -, -, v_EmergencyType)))); //f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_EmergencyCall_NormalService, -, IPCAN_SpeechAndEmergencyCall); // @sic R5-225426, R5s221136 sic@ …..

## Change 4

| Function name | function f_TC_10_9_NR5GC_IMS2() |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

…… function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // @siclog "Steps 2-9" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 11-12" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); // 4.9.11.2.2-2 of TS 38.508-1 @sic R5-225362 sic@ //@siclog "Step 12" siclog@ //UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 13" siclog@ //SS sends 401 Unauthorized(Step 2 of Annex A.3) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 14" siclog@ //UE sends REGISTER(Step 3 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; //@siclog "Step 15" siclog@ //SS sends 403 Forbidden //The following messages are exchanged on non protected port. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_GlobalRelease (keepUnprotected); // @sic R5s220609 - Additional changes: keepUnprotected instead of oldestProtectedContextOnly sic@ // EXCEPTION: In parallel to steps 16-20 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");

After Change:

…… function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // @siclog "Steps 2-9" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 11-12" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); // 4.9.11.2.2-2 of TS 38.508-1 @sic R5-225362 sic@ //@siclog "Step 12" siclog@ //UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 13" siclog@ //SS sends 401 Unauthorized(Step 2 of Annex A.3) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 14" siclog@ //UE sends REGISTER(Step 3 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; //@siclog "Step 15" siclog@ //SS sends 403 Forbidden //The following messages are exchanged on non protected port. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_GlobalRelease (keepUnprotected); // @sic R5s220609 - Additional changes: keepUnprotected instead of oldestProtectedContextOnly sic@ // EXCEPTION: In parallel to steps 16-20 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // Anritsu CR for 10.x v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");

## Change 5

| Function name | function f_TC_10_10_NR5GC_IMS2() |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

…… function f_TC_10_10_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var SipUrl v_ContactUrl; var boolean v_CondA7 := true; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 0-8" siclog@ // UE is made to initiate an emergency call. @sic R5-224683 sic@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 9-10" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); // 4.9.11.2.2-2 of TS 38.508-1 @sic R5-225362 sic@ // UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); // @siclog "Step 12" siclog@ // SS sends 403 Forbidden // The following messages are exchanged on non protected port. // The SS responds with a failure. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_PTC_SetRanType(NR); // Ran Type Condition A28 f_IMS_GlobalRelease(keepUnprotected); // @sic R5s220609 - Additional changes: keepUnprotected instead of oldestProtectedContextOnly sic@ // EXCEPTION: In parallel to steps 13-17 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. // @siclog "Step 13" siclog@ // Check: Does the UE send a correctly composed INVITE request? // Condition A28 checked v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6); // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13");

After Change:

…… function f_TC_10_10_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var SipUrl v_ContactUrl; var boolean v_CondA7 := true; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 0-8" siclog@ // UE is made to initiate an emergency call. @sic R5-224683 sic@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 9-10" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); // 4.9.11.2.2-2 of TS 38.508-1 @sic R5-225362 sic@ // UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); // @siclog "Step 12" siclog@ // SS sends 403 Forbidden // The following messages are exchanged on non protected port. // The SS responds with a failure. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_PTC_SetRanType(NR); // Ran Type Condition A28 f_IMS_GlobalRelease(keepUnprotected); // @sic R5s220609 - Additional changes: keepUnprotected instead of oldestProtectedContextOnly sic@ // EXCEPTION: In parallel to steps 13-17 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. // @siclog "Step 13" siclog@ // Check: Does the UE send a correctly composed INVITE request? f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // Anritsu CR for 10.x // Condition A28 checked v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6); // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13");

## Change 6

| Function name | function f_IMS_EmergencyCallSetup_NR5GC_A_6 |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | Common\IMS\IMS_Procedures_5G.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_EmergencyCallSetup_NR5GC_A_6(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@ template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request { var IMS_DATA_REQ v_IMS_DATA_REQ; v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri); return f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); // @sic R5s210875 sic@ }

After Change:

function f_IMS_EmergencyCallSetup_NR5GC_A_6(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@ template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request { var IMS_DATA_REQ v_IMS_DATA_REQ; f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // Anritsu CR for 10.x and 11.4.x v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri); return f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); // @sic R5s210875 sic@ }

## Change 7

| Function name | function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd |
| --- | --- |
| Reason for change | UE sends RTP/RTCP packets on QFI:2 which is not expected as per current TTCN configuration. UE generates RTP/RTCP packets upon reception of 200 OK for emergency call, but all-match IP Packet filter#1 allows UE to send RTP/RTCP packets on QFI:2(configured in PDU SESSION ESTABLISHEMENT REQUEST). Therefore, after transmission of 200 OK and before configuration of QFI: 7 (via PDU MODIFICATION COMMAND), SS can’t handle RTP/RTCP packets which are sent over QFI:2 as packet filter allows it. So, we modify TTCN to update the DRBMUX for RTP/RTCP loopback of QFI:2 just before waiting for INVITE. |
| Summary of change | TTCN to DRBMUX before receiving INVITE from UE in case of emergency Call test cases |
| TTCN module | Common\IMS\IMS_Procedures_5G.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(IMS_DATA_REQ p_IMS_DATA_REQ) runs on IMS_PTC return INVITE_Request // @sic R5-220214 sic@ { /* @sic R5-231989: p_GeolocationInfoAvailable removed sic@ */ var INVITE_Request v_INVITE_Request; f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ v_INVITE_Request := f_IMS_EmergencyCallSetup_Step2ToEnd (p_IMS_DATA_REQ); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s220531, R5-225362 sic@ return v_INVITE_Request; }

After Change:

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(IMS_DATA_REQ p_IMS_DATA_REQ) runs on IMS_PTC return INVITE_Request // @sic R5-220214 sic@ { /* @sic R5-231989: p_GeolocationInfoAvailable removed sic@ */ var INVITE_Request v_INVITE_Request; //f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ v_INVITE_Request := f_IMS_EmergencyCallSetup_Step2ToEnd (p_IMS_DATA_REQ); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s220531, R5-225362 sic@ return v_INVITE_Request; }