3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230832

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0924 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS WLAN XCAP testcase G.15.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test, MTSI_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add testcase G.15.8 to the ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case G.15.8 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to IMS_WLAN testsuite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.15.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6889	3

5	References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case G.15.8 which is part of the IMS_WLAN test suite in ATS iwd-TTCN3-B2022-09_D23wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

1.1 Verification Test Summary

Test Case: 			G.15.8
ATS Version:			iwd-TTCN3-B2022-09_D23wk24
System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR
UE used:			MediaTek MTK 6889
Verification Status:	 		PASS

# Corrections required

## Same as R5s230616(Addition of IMS WLAN XCAP testcase G.15.1)

# Branches executed

N/A.

# Execution Log Files

## Mediatek MT6889 UE

The Mediatek MT6889 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_G_15_8_LOG.html

- PICS/PIXIT parameter file:

TC_ G_15_8_PIXIT.xml

# References

[1] R5s230627: Supporting information for addition of IMS WLAN test case G.15.8