3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230840

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3185 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SIB1 is scheduled to transmit on even SFN. If f_NR_GetNextSendOccasion_DL returns even SFN then there will be collision to transmit DTCH for DL data and SIB1 on same SFN/Subframe/Slot for reason stated below: CORESET#0 and CORESET#1 occupy same set of RBs. However, CORESET#0 is interleaved while CORESET#1 is non-interleaved. If SI-RNTI DCI and C-RNTI DCI are scheduled on the same slot, then, SS can’t schedule C-RNTI DCI as 12 consecutive RBs are not available for 4 CCEs(AL4). Example, Coreset size is 24 RBs with two symbols. If SI_RNTI(4 CCEs, AL4) occupies RBs#0-5 and RBs#12-17 due to interleaving, then, C-RNTI also requires 4CCEs(AL4) non-interleaved but this requires 12 consecutive RBs which are not available. To solve this issue, it is proposed to use odd SFN to avoid this collision |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added if condition for changing DL transmission timing to odd sfn. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_2_2_NR_TestBody () |
| --- | --- |
| Reason for change | SIB1 is scheduled to transmit on even SFN. If f_NR_GetNextSendOccasion_DL returns even SFN then there will be collision to transmit DTCH for DL data and SIB1 on same SFN/Subframe/Slot for reason stated below: CORESET#0 and CORESET#1 occupy same set of RBs. However, CORESET#0 is interleaved while CORESET#1 is non-interleaved. If SI-RNTI DCI and C-RNTI DCI are scheduled on the same slot, then, SS can’t schedule C-RNTI DCI as 12 consecutive RBs are not available for 4 CCEs(AL4). Example, Coreset size is 24 RBs with two symbols. If SI_RNTI(4 CCEs, AL4) occupies RBs#0-5 and RBs#12-17 due to interleaving, then, C-RNTI also requires 4CCEs(AL4) non-interleaved but this requires 12 consecutive RBs which are not available. To solve this issue, it is proposed to use odd SFN to avoid this collision |
| Summary of change | Added if condition for changing DL transmission timing to odd sfn. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_2_2_NR_TestBody( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_SYSTEM_IND v_SYS_IND; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_1_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing,cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('011'B)), cs_NR_RedundancyVersionList_Def); //Anritsu TTCN CR R5s230068 MCC var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 0.4; //Configure DL DCI as per Table 7.1.1.2.2.3.3-3: Physical layer parameters for DCI format 1_1 (Steps 3, 7, Table 7.1.1.2.2.3.2-1) //Set K1=5 f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); //By default in TS 38.508-1, the Time Alignment timer is set to infinity //configure SS for no Auto Time alignement - no grant needed f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5s221035 sic@ //Config SS to report: NR_SystemRequest_Type -> NR_System_IndicationControl_Type -> UL_HARQ //NR_SYSTEM_IND -> NR_SystemIndication_Type-> UL_HARQ: ack/nack f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s221035 sic@ v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_36B))); //@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // @siclog "Steps 3-4" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); //added here v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; //added here } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 5" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. (Note3) // SS already configured to do it - PDSCH aggregation // @siclog "Step 6" siclog@ // Check: Does the UE transmit a HARQ NACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack))-> value v_SYS_IND { v_Nack_Count := v_Nack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Nack_Count == 1) { //@sic R5s230068 sic@ //Anritsu TTCN CR R5s230068 MCC f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); } // @siclog "Step 7" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // @siclog "Step 8" siclog@ // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. //Set CRC error as normal mode f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_Now)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); //added here v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // @siclog "Step 9" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. (Note3) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 10" siclog@ // Check: Does the UE transmit a HARQ ACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))-> value v_SYS_IND { v_Ack_Count := v_Ack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Ack_Count == 1) { //@sic R5s230068 sic@ //Anritsu TTCN CR R5s230068 MCC f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } // Configure SS to stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //@sic R5s221035 sic@ }

After Change

function f_TC_7_1_1_2_2_NR_TestBody( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_SYSTEM_IND v_SYS_IND; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_1_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing,cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('011'B)), cs_NR_RedundancyVersionList_Def); var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 0.4; //Configure DL DCI as per Table 7.1.1.2.2.3.3-3: Physical layer parameters for DCI format 1_1 (Steps 3, 7, Table 7.1.1.2.2.3.2-1) //Set K1=5 f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); //By default in TS 38.508-1, the Time Alignment timer is set to infinity //configure SS for no Auto Time alignement - no grant needed f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5s221035 sic@ //Config SS to report: NR_SystemRequest_Type -> NR_System_IndicationControl_Type -> UL_HARQ //NR_SYSTEM_IND -> NR_SystemIndication_Type-> UL_HARQ: ack/nack f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s221035 sic@ v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_36B))); //@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // @siclog "Steps 3-4" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); if (((v_TimingDL.SFN.Number) mod 2) == 0){ //conflict with SIB scheduling v_TimingDL.SFN.Number := v_TimingDL.SFN.Number + 1; } v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 5" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. (Note3) // SS already configured to do it - PDSCH aggregation // @siclog "Step 6" siclog@ // Check: Does the UE transmit a HARQ NACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack))-> value v_SYS_IND { v_Nack_Count := v_Nack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Nack_Count == 1) { //@sic R5s230068 sic@ //Anritsu TTCN CR R5s230068 MCC f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); } // @siclog "Step 7" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // @siclog "Step 8" siclog@ // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. //Set CRC error as normal mode f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_Now)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); if (((v_TimingDL.SFN.Number) mod 2) == 0){ //conflict with SIB scheduling v_TimingDL.SFN.Number := v_TimingDL.SFN.Number + 1; } v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // @siclog "Step 9" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. (Note3) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 10" siclog@ // Check: Does the UE transmit a HARQ ACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))-> value v_SYS_IND { v_Ack_Count := v_Ack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Ack_Count == 1) { //@sic R5s230068 sic@ //Anritsu TTCN CR R5s230068 MCC f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } // Configure SS to stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //@sic R5s221035 sic@ }