3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230850

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3192 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The v_GSM_MobilityInfo could be rewritten in step 3Ca1 in the function f_Check_NG_PDUSessionReleaseReq which would effectivelly set the PDNindex to omit and hence the information would be lost and cause runtime error later when calling f_IP_RtpRtcpLoopback with omitted value. - In steps 3Fa / 3Fb the old timer is still referenced. - At step 3E, UE can send deregister request immediately after sending PDU session release complete to start normal service once Emergency call end’s. f_NR5GC_PDUSessionRelease function may time to complete its all subfunction’s like f_IP_Handling_StopPDN to stop all DHCP and ICMPv6 servers. - Since the f_NR_RRCSetup_Def (called by f_NR5GC_RRC_Idle_Steps3_20) will not start the UL grants and send the RRC message for non-early contention tests it has to be added in step 9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Stored PDNindex separately and used its value when calling f_IP_RtpRtcpLoopback. - In steps 3Fa / 3Fb corrected the typo in timer name. - Added default variable to handle deregistration request as Note 2 before function f_NR5GC_PDUSessionRelease. And removed corresponding function of Note 2 from alt statement after step3F. - Added start of UL grant and sending of RRC setup in step 9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_4_1_TestBody |
| --- | --- |
| Reason for change | - The v_GSM_MobilityInfo could be rewritten in step 3Ca1 in the function f_Check_NG_PDUSessionReleaseReq which would effectivelly set the PDNindex to omit and hence the information would be lost and cause runtime error later when calling f_IP_RtpRtcpLoopback with omitted value. - In steps 3Fa / 3Fb the old timer is still referenced. - At step 3E, UE can send deregister request immediately after sending PDU session release complete to start normal service once Emergency call end’s. f_NR5GC_PDUSessionRelease function may time to complete its all subfunction’s like f_IP_Handling_StopPDN to stop all DHCP and ICMPv6 servers. - Since the f_NR_RRCSetup_Def (called by f_NR5GC_RRC_Idle_Steps3_20) will not start the UL grants and send the RRC message for non-early contention tests it has to be added in step 9. |
| Summary of change | - Stored PDNindex separately and used its value when calling f_IP_RtpRtcpLoopback. - In steps 3Fa / 3Fb corrected the typo in timer name. - Added default variable to handle deregistration request as Note 2 before function f_NR5GC_PDUSessionRelease. And removed corresponding function of Note 2 from alt statement after step3F. - Added start of UL grant and sending of RRC setup in step 9. |
| TTCN module | 11_4\Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { … //@siclog "Step 3" siclog@ //Make the UE release the emergency call. //@siclog "Step 3A" siclog@ //The Generic test procedure for IMS MO Emergency call release f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5-233779 sic@ // Don't need this anymore as next step is no longer a DL message f_Delay(0.1);//@sic R5s230143 sic@ :ensure that the Ack for PDU Session Release Complete is sent in DL. //@sic R5-233779 sic@:Step3B-Step3F // Get this for the emergency PDN now in case UE doesn't send a PDU Release Req v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfo, Emergency_PDN); v_GSM_MobilityInfo := valueof(v_GSM_MobilityInfo_IMS); v_PDUSessionId := v_GSM_MobilityInfo.SessionId; //@siclog "Step 3B" siclog@ //SS starts timer T1 = 5 seconds. t_Wait.start; //EXCEPTION: Steps 3Ca1 to 3Cb1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. alt { [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) { repeat} //@siclog "Step 3Ca1" siclog@ //The UE transmits a PDU SESSION RELEASE REQUEST message. [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; // Step 3Ca2 if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } } //@siclog "Step 3Cb1" siclog@ //Timer T1 expires. [] t_Wait.timeout { //Do nothing } } //@siclog "Step 3D" siclog@ //The SS transmits a PDU SESSION RELEASE COMMAND message. //@siclog "Step 3E" siclog@ //Check: Does the UE transmit a PDU SESSION RELEASE COMPLETE message? v_PDUSessionId := v_GSM_MobilityInfo.SessionId; f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionId); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3E"); //@siclog "Step 3Fa1-3Fb3" siclog@ t_Wait2.start; //EXCEPTION: EXCEPTION: Steps 3Fa1-3Fb3 describe optional behaviour that depends on the UE implementation. the "lower case letter" identifies a step sequence that take place if a UE specific behavior takes place. alt { //@siclog "Step 3Fa2" siclog@ //Check: Does the UE transmit RRCSetupRequest on NR Cell 1? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(?, ?))) { t_Wait.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3Fa2"); //@siclog "Step 3Fa3" siclog@ //SS locally releases the RRC connection. f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)) } //@siclog "Step 3Fb1" siclog@ //Timer T2 expires [] t_Wait.timeout { //@siclog "Step 3Fb2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3Fb3" siclog@ //Check: Does the UE transmit RRCSetupRequest on NR Cell 1? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(?, ?))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3Fb2"); } //@siclog "NOTE 2" siclog@ [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) -> value v_ReceivedAsp { repeat; } } // PDU session definitely gone now, so can stop can stop looping back RTP/RTCP f_IP_RtpRtcpLoopback(IP, v_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, omit); //@sic Rs230583 sic@ //@siclog "Step 4A-4B" siclog@ //Void //@sic R5-233779 sic@ //@siclog "Step 9A1-9A2" siclog@ //Void //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, -, Initial_NoSecurity); //@siclog "Step 9A22" siclog@

After Change:

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { … var PDN_Index_Type v_PDNindex; var default v_MyDefaultVar := null; //@siclog "Step 3" siclog@ //Make the UE release the emergency call. //@siclog "Step 3A" siclog@ //The Generic test procedure for IMS MO Emergency call release f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5-233779 sic@ // Don't need this anymore as next step is no longer a DL message f_Delay(0.1);//@sic R5s230143 sic@ :ensure that the Ack for PDU Session Release Complete is sent in DL. //@sic R5-233779 sic@:Step3B-Step3F // Get this for the emergency PDN now in case UE doesn't send a PDU Release Req v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfo, Emergency_PDN); v_GSM_MobilityInfo := valueof(v_GSM_MobilityInfo_IMS); v_PDNindex := v_GSM_MobilityInfo.PdnIndex; v_PDUSessionId := v_GSM_MobilityInfo.SessionId; //@siclog "Step 3B" siclog@ //SS starts timer T1 = 5 seconds. t_Wait.start; //EXCEPTION: Steps 3Ca1 to 3Cb1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. alt { [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) { repeat} //@siclog "Step 3Ca1" siclog@ //The UE transmits a PDU SESSION RELEASE REQUEST message. [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; // Step 3Ca2 if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } } //@siclog "Step 3Cb1" siclog@ //Timer T1 expires. [] t_Wait.timeout { //Do nothing } } //@siclog "Step 3D" siclog@ //The SS transmits a PDU SESSION RELEASE COMMAND message. //@siclog "Step 3E" siclog@ //Check: Does the UE transmit a PDU SESSION RELEASE COMPLETE message? v_PDUSessionId := v_GSM_MobilityInfo.SessionId; //@siclog "NOTE 2" siclog@ v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell1)); f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionId); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3E"); //@siclog "Step 3Fa1-3Fb3" siclog@ t_Wait2.start; //EXCEPTION: EXCEPTION: Steps 3Fa1-3Fb3 describe optional behaviour that depends on the UE implementation. the "lower case letter" identifies a step sequence that take place if a UE specific behavior takes place. alt { //@siclog "Step 3Fa2" siclog@ //Check: Does the UE transmit RRCSetupRequest on NR Cell 1? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(?, ?))) { t_Wait2.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3Fa2"); //@siclog "Step 3Fa3" siclog@ //SS locally releases the RRC connection. f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)) } //@siclog "Step 3Fb1" siclog@ //Timer T2 expires [] t_Wait2.timeout { //@siclog "Step 3Fb2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3Fb3" siclog@ //Check: Does the UE transmit RRCSetupRequest on NR Cell 1? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(?, ?))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3Fb2"); } } deactivate(v_MyDefaultVar); // PDU session definitely gone now, so can stop can stop looping back RTP/RTCP f_IP_RtpRtcpLoopback(IP, v_PDNindex, tsc_IMS_MediaPort_M1, omit); //@sic Rs230583 sic@ //@siclog "Step 4A-4B" siclog@ //Void //@sic R5-233779 sic@ //@siclog "Step 9A1-9A2" siclog@ //Void //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR_ULGrantConfiguration_Start(nr_Cell1); SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def))); f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, -, Initial_NoSecurity); //@siclog "Step 9A22" siclog@