3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230851

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3195 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_508_ENDC_RbEst_Steps3_10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In current TTCN implementation of function f_EUTRA38_508_ENDC_RbEst_Steps3_10 based, v_DrbConfigList is not having an entry with cs_OneDRB_ConfigAM(v_DrbForInternet) when the UserPlaneIntegrity is set to False ( In case of non-UPIP ). As TTCN is communicating about v_DrbForInternet in the Peer Reconfiguration message to the UE, it is required to configure the same for the Local end NW configuration as well. - TTCN implementation has to also take care of filling v_DrbForInternet configuration in v_DrbConfigList[1] when there is already another DRB Configuration coming in as part of the function call f_EUTRA38_508_ENDC_RbEst_Steps3_10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Initialized v_DrbConfigList in f_EUTRA38_508_ENDC_RbEst_Steps3_10 when UserPlaneIntegrity is set to False. - Populated v_DrbConfigList[1] when there is already another DRB Configuration coming in as part of the function call f_EUTRA38_508_ENDC_RbEst_Steps3_10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.1.ENDC, 7.1.3.2.2.ENDC, 7.1.3.2.3.ENDC, 7.1.3.3.1.ENDC, 7.1.3.3.2.ENDC, 7.1.3.3.3.ENDC, 8.2.1.1.1.ENDC, 8.2.2.2.1.ENDC, 8.2.2.3.1.ENDC, 8.2.2.6.1.ENDC, 8.2.2.8.1.ENDC, 8.2.3.13.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_EUTRA38_508_ENDC_RbEst_Steps3_10	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_EUTRA38_508_ENDC_RbEst_Steps3_10

| Function name | f_EUTRA38_508_ENDC_RbEst_Steps3_10 |
| --- | --- |
| Reason for change | - In current TTCN implementation of function f_EUTRA38_508_ENDC_RbEst_Steps3_10 based, v_DrbConfigList is not having an entry with cs_OneDRB_ConfigAM(v_DrbForInternet) when the UserPlaneIntegrity is set to False ( In case of non-UPIP ). As TTCN is communicating about v_DrbForInternet in the Peer Reconfiguration message to the UE, it is required to configure the same for the Local end NW configuration as well. - TTCN implementation has to also take care of filling v_DrbForInternet configuration in v_DrbConfigList[1] when there is already another DRB Configuration coming in as part of the function call f_EUTRA38_508_ENDC_RbEst_Steps3_10. |
| Summary of change | - Initialized v_DrbConfigList in f_EUTRA38_508_ENDC_RbEst_Steps3_10 when UserPlaneIntegrity is set to False. - Populated v_DrbConfigList[1] when there is already another DRB Configuration coming in as part of the function call f_EUTRA38_508_ENDC_RbEst_Steps3_10. |
| TTCN module | ENDC/Common_EUTRA/EUTRA38_ENDC_CommonProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_508_ENDC_RbEst_Steps3_10(EUTRA_CellId_Type p_CellId, template (omit) octetstring p_SCGConfig, template (omit) octetstring p_RBConfig, template (omit) P_Max p_MaxEUTRA, template (omit) NAS_MSG_RequestList_Type p_NAS_MsgList, template (omit) RadioBearerList_Type p_DrbConfigList, template (omit) RadioResourceConfigDedicated p_RadioResourceConfigDedicated, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) octetstring p_RBConfig2 := omit, template NAS_UL_Message_Type p_ULNASMsg := omit, ENDC_DRB_Configuration_Type p_DRB_Type := MCG_SCG, boolean p_UserPlaneIntegrity := false) runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var integer v_SK_Counter; var DRB_Identity v_DrbForInternet := f_EUTRA_NB_EpsBearerAssociatedDRB(tsc_EpsDefaultBearerId2ndPDN); var template (omit) RadioBearerList_Type v_DrbConfigList := p_DrbConfigList; v_NasInd := f_EUTRA38_508_ENDC_RbEst_Steps3_5(p_CellId); if (p_UserPlaneIntegrity) { // @sic R5-235451 sic@ if (not isvalue(p_DrbConfigList)) { // if a specific DRB config hasn't been included // need to configure UPIP v_DrbConfigList := {cs_RadioBearer_Reconfig_DRBonEUTRAwithUPIP(v_DrbForInternet)}; } } /* step 6-7 of procedure in 36.508 cl. 4.5.3.3 */ v_AuthParams := f_EUTRA38_RRC_ActivateSecurity(p_CellId, v_AuthParams, v_NasInd.SecurityProtection.NasCount, v_DrbConfigList, p_UserPlaneIntegrity); // @sic R5s190476, R5-235451 sic@ << SKIPPED CODE >>

After Change:

function f_EUTRA38_508_ENDC_RbEst_Steps3_10(EUTRA_CellId_Type p_CellId, template (omit) octetstring p_SCGConfig, template (omit) octetstring p_RBConfig, template (omit) P_Max p_MaxEUTRA, template (omit) NAS_MSG_RequestList_Type p_NAS_MsgList, template (omit) RadioBearerList_Type p_DrbConfigList, template (omit) RadioResourceConfigDedicated p_RadioResourceConfigDedicated, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) octetstring p_RBConfig2 := omit, template NAS_UL_Message_Type p_ULNASMsg := omit, ENDC_DRB_Configuration_Type p_DRB_Type := MCG_SCG, boolean p_UserPlaneIntegrity := false) runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var integer v_SK_Counter; var DRB_Identity v_DrbForInternet := f_EUTRA_NB_EpsBearerAssociatedDRB(tsc_EpsDefaultBearerId2ndPDN); var template (omit) RadioBearerList_Type v_DrbConfigList := p_DrbConfigList; v_NasInd := f_EUTRA38_508_ENDC_RbEst_Steps3_5(p_CellId); if (not isvalue(p_DrbConfigList)) { if (p_UserPlaneIntegrity) { // if a specific DRB config hasn't been included // need to configure UPIP v_DrbConfigList := {cs_RadioBearer_Reconfig_DRBonEUTRAwithUPIP(v_DrbForInternet)}; } else { v_DrbConfigList := {cs_OneDRB_ConfigAM(v_DrbForInternet)}; } } else { if (p_UserPlaneIntegrity) { // if a specific DRB config hasn't been included // need to configure UPIP v_DrbConfigList[1] := cs_RadioBearer_Reconfig_DRBonEUTRAwithUPIP(v_DrbForInternet); } else { v_DrbConfigList[1] := cs_OneDRB_ConfigAM(v_DrbForInternet); } } /* step 6-7 of procedure in 36.508 cl. 4.5.3.3 */ v_AuthParams := f_EUTRA38_RRC_ActivateSecurity(p_CellId, v_AuthParams, v_NasInd.SecurityProtection.NasCount, v_DrbConfigList, p_UserPlaneIntegrity); // @sic R5s190476, R5-235451 sic@s } << SKIPPED CODE >>