3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230855

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3264 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.6.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - While building Measurement report configuration need to provide delay_DRBlist_r16 on template cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup established DRB id values. Else test will fail if UE is setup to IMS mode because with IMS, first DRB id value IMS is 1 and next 2 (second and third) DRB id will 2 and 3. delay_DRBlist_r16 set always DRBid 3 and DRB id 4. However, for Non-IMS UE setup, it will be working without these changes because DRB ID value list will be generated for it as 2,3,4. - Need to pass established DRB ID values for second and third DRB ID for the reason mentioned on change 1. - Need to pass DRB ID values for second and third DRB ID for the reason mentioned on change 1. - Added arguments on cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup for DRB ID’s and update delay_DRBlist_r16 according to correctly established second and third DRBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added DRB id’s for second and third DRB on template cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup. - Arguments added for second and third DRB on f_TC_8_1_6_1_1_2_NR5GC_TestBody. - Arguments added for second and third DRB on cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup. - Updated variables for delay_DRBlist_r16 instead constant 3 and 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

2.1	Change 2	5

2.1	Change 3	5

2.1	Change 4	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_1_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | While building Measurement report configuration need to provide delay_DRBlist_r16 on template cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup established DRB id values. Else test will fail if UE is setup to IMS mode and for IMS first DRB id value IMS is 1 and next 2 (second and third) DRB id will 2 and 3. delay_DRBlist_r16 set always DRBid 3 and DRB id 4. However, for Non-IMS UE setup it will be working without these changes because DRB ID value list will be generated for it as 2,3,4. |
| Summary of change | Added DRB id’s for second and third DRB on template cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup. |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_1_2_NR5GC_TestBody(integer p_PduSessionId) runs on NR_BASE_PTC { var PhysCellId v_PhysCellId; var RSRP_Range v_AbsThreshSS_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; -----------------skip code--- v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup(tsc_NR_IdReportConfigId1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_DefaultRef := activate(a_NR_AddDefault_IgnorePeriodicalMeasurmentReport(nr_Cell1,cr_NR_MeasurementReport_Any)); //@sic R5-224191, R5s220961 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); -----------------skip code--- }//end of f_TC_8_1_6_1_1_2_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_1_2_NR5GC_TestBody(integer p_PduSessionId, integer i, integer j) runs on NR_BASE_PTC { var PhysCellId v_PhysCellId; var RSRP_Range v_AbsThreshSS_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; -----------------skip code--- v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup(tsc_NR_IdReportConfigId1, i, j)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_DefaultRef := activate(a_NR_AddDefault_IgnorePeriodicalMeasurmentReport(nr_Cell1,cr_NR_MeasurementReport_Any)); //@sic R5-224191, R5s220961 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); -----------------skip code--- }//end of f_TC_8_1_6_1_1_2_NR5GC_TestBody

## Change 2

| Function name | f_TC_8_1_6_1_1_2_NR5GC () |
| --- | --- |
| Reason for change | Need to pass established DRB ID values for second and third DRB ID for the reason mentioned on change 1 |
| Summary of change | Arguments added for second and third DRB on f_TC_8_1_6_1_1_2_NR5GC_TestBody. |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_1_2_NR5GC() runs on NR5GC_PTC { //Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB var Octet_Type v_IP_PDU_delay := '05'O; var IntegerList_Type v_DRBList; var integer v_FirstDRB; var integer v_SecondDRB; var integer v_ThirdDRB; -------skip code------- //Get DRB identities v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB v_FirstDRB := v_DRBList[0]; v_SecondDRB := v_DRBList[1]; v_ThirdDRB := v_DRBList[2]; -------------skip code---------------- f_TC_8_1_6_1_1_2_NR5GC_TestBody(v_PduSessionId); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); //@sic R5s220691 sic@ } //end of f_TC_8_1_6_1_1_2_NR5GC

After Change

function f_TC_8_1_6_1_1_2_NR5GC() runs on NR5GC_PTC { //Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB var Octet_Type v_IP_PDU_delay := '05'O; var IntegerList_Type v_DRBList; var integer v_FirstDRB; var integer v_SecondDRB; var integer v_ThirdDRB; var integer v_PduSessionId; -------skip code------ //Get DRB identities v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB v_FirstDRB := v_DRBList[0]; v_SecondDRB := v_DRBList[1]; v_ThirdDRB := v_DRBList[2]; --------skip code---- f_NR_TestBody_Set(true); f_TC_8_1_6_1_1_2_NR5GC_TestBody(v_PduSessionId, v_SecondDRB, v_ThirdDRB); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); //@sic R5s220691 sic@ } //end of f_TC_8_1_6_1_1_2_NR5GC

## Change 3

| Function name | template cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup () |
| --- | --- |
| Reason for change | Need to pass DRB ID values for second and third DRB ID for the reason mentioned on change 1 |
| Summary of change | Arguments added for second and third DRB on cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup. |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC |
| MCC160 Comment |  |

Before Change

template (value) ReportConfigToAddMod cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup(ReportConfigId p_ReportConfigId) := /* @status APPROVED (NR5GC) */ cs_NR_ReportConfigToAddMod_NR(p_ReportConfigId, cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup);

After Change

template (value) ReportConfigToAddMod cs_NR_ReportConfig_Periodical_UlDelayValueConfig_r16_setup(ReportConfigId p_ReportConfigId, integer i, integer j) := /* @status APPROVED (NR5GC) */ cs_NR_ReportConfigToAddMod_NR(p_ReportConfigId, cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup(i, j) );

## Change 4

| Function name | template cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup () |
| --- | --- |
| Reason for change | Added arguments on cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup for DRB ID’s and update delay_DRBlist_r16 according to correctly established second and third DRBs |
| Summary of change | Updated variables for delay_DRBlist_r16 instead constant 3 and 4 |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC |
| MCC160 Comment |  |

Before Change

template (value) ReportConfigNR cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup := { /* @status APPROVED (NR5GC) */ reportType := { periodical := { rsType := ssb, reportInterval := ms480, //Acc Table 4.6.3-109 TS 38.508-1 reportAmount := infinity_, reportQuantityCell := cs_NR_MeasReportQuantity_All, maxReportCells := 8, reportQuantityRS_Indexes := omit, maxNrofRS_IndexesToReport := omit, includeBeamMeasurements := false, useAllowedCellList := false, /* @sic R5s211526 BASELINE MOVING 2021 sic@ */ measRSSI_ReportConfig_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeCommonLocationInfo_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeBT_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeWLAN_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeSensor_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ ul_DelayValueConfig_r16 := { setup := { delay_DRBlist_r16 := {3, 4} //@sic R5-224191, R5s220961 R5s221179 BASELINE MOVING 2022 rel-17 sic@ } }, reportAddNeighMeas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ ul_ExcessDelayConfig_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ coarseLocationRequest_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ reportQuantityRelay_r17 := omit /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ } } };

After Change

template (value) ReportConfigNR cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup (integer i, integer j) := { /* @status APPROVED (NR5GC) */ reportType := { periodical := { rsType := ssb, reportInterval := ms480, //Acc Table 4.6.3-109 TS 38.508-1 reportAmount := infinity_, reportQuantityCell := cs_NR_MeasReportQuantity_All, maxReportCells := 8, reportQuantityRS_Indexes := omit, maxNrofRS_IndexesToReport := omit, includeBeamMeasurements := false, useAllowedCellList := false, /* @sic R5s211526 BASELINE MOVING 2021 sic@ */ measRSSI_ReportConfig_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeCommonLocationInfo_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeBT_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeWLAN_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ includeSensor_Meas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ ul_DelayValueConfig_r16 := { setup := { delay_DRBlist_r16 := {i, j} //@sic R5-224191, R5s220961 R5s221179 BASELINE MOVING 2022 rel-17 sic@ } }, reportAddNeighMeas_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ ul_ExcessDelayConfig_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ coarseLocationRequest_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ reportQuantityRelay_r17 := omit /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ } } };