3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230858

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3267 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Since the K2 value is different for initial BWP and for other BWPs it is essential that the correct K2 is used in UL grant timing calculations depending on which BWP the test currently running. In case of the test switching only between BWP#0 and #1 the K2 is not updated to value of initial BWP before UL grant calculations on BWP#0. So this change is correcting this by updating cell info with the current K2. - The BWP switch command to SS in step 16 needs to be sent with explicit timing which is same as step timing for 16 DRB data, otherwise the switch may happen separately from step 16 data which would not correspond to the test prose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Cell info is updated with current K2 value in two instances of the f_TC_7_1_1_8_1_NR_CommonTestBody function. - Added explicit timing to the BWP switch command SS configuration which is same as the timing of step 16 data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| TC name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | - Since the K2 value is different for initial BWP and for other BWPs it is essential that the correct K2 is used in UL grant timing calculations depending on which BWP the test currently running. In case of the test switching only between BWP#0 and #1 the K2 is not updated to value of initial BWP before UL grant calculations on BWP#0. So this change is correcting this by updating cell info with the current K2. - The BWP switch command to SS in step 16 needs to be sent with explicit timing which is same as step timing for 16 DRB data, otherwise the switch may happen separately from step 16 data which would not correspond to the test prose. |
| Summary of change | - Cell info is updated with current K2 value in two instances of the function. - Added explicit timing to the BWP switch command SS configuration which is same as the timing of step 16 data. |
| TTCN module | NR_TC_Common\7_1_1\MAC_BWP_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 R5s230634 sic@ f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_3); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1); } // @siclog "Step 10" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @siclog "Step 11A" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1400); // 1400ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; var PUSCH_TimeDomainResourceAllocation.k2 v_K2; var PUSCH_TimeDomainResourceAllocation.k2 v_K2_initial:=4; // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); v_K2 := f_NR_CellInfo_GetK2(p_NR_CellId); f_NR_CellInfo_SetK2(p_NR_CellId, v_K2_initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); f_NR_CellInfo_SetK2(p_NR_CellId, v_K2); //@sic R5s230220 change 9.2 sic@ v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 R5s230634 sic@ f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_3); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1); } // @siclog "Step 10" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @siclog "Step 11A" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; //@sic R5s230634 sic@ // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1400); // 1400ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); }