3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230887

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3196 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per RAN1 CR R1-2202898, clarification has been provided that the PUCCH resource determination for the SPS-PDSCH “associated with the corresponding activation DCI” follows the RRC-configured n1PUCCH-AN, or SPS-PUCCH-AN-List like all the subsequent SPS-PDSCHs do. Hence, this would mean that UE would use pucch-ResourceID = 0 as configured in TS 38.523-1 Table 7.1.1.6.1.3.3-2 However, as per Prose CR R5-201195, the PUCCH resource indicator in DCI 1_x is set to “110”. Hence, the SS implementation would decode the HARQ Feedback from the UE as DTX for the activation DCI sent on PDCCH and data on PDSCH using CS-RNTI leading to testcase failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the pucch-ResourceID = 6 mirroring the PUCCH Resource Indicator in the DCI 1_x |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC and 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A prose CR will be raised for this change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to template cs_SPS_Config_Common	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required

  - Correction to template cs_SPS_Config_Common

| Template name | cs_SPS_Config_Common |
| --- | --- |
| Reason for change | As per RAN1 CR R1-2202898, clarification has been provided that the PUCCH resource determination for the SPS-PDSCH “associated with the corresponding activation DCI” follows the RRC-configured n1PUCCH-AN, or SPS-PUCCH-AN-List like all the subsequent SPS-PDSCHs do. Hence, this would mean that UE would use pucch-ResourceID = 0 as configured in TS 38.523-1 Table 7.1.1.6.1.3.3-2 However, as per Prose CR R5-201195, the PUCCH resource indicator in DCI 1_x is set to “110”. Hence, the SS implementation would decode the HARQ Feedback from the UE as DTX for the activation DCI sent on PDCCH and data on PDSCH using CS-RNTI leading to testcase failure |
| Summary of change | Changed the pucch-ResourceID = 6 mirroring the PUCCH Resource Indicator in the DCI 1_x |
| TTCN module | NR_TC_Common\7_1_1\MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) SPS_Config cs_SPS_Config_Common (SPS_Config.periodicity p_Periodicity := ms160, template(omit) SPS_Config.mcs_Table p_MCS_Table := omit, template(omit) SPS_Config.sps_ConfigIndex_r16 p_SPS_ConfigIndex_r16 := omit, template(omit) SPS_Config.harq_ProcID_Offset_r16 p_Harq_ProcID_Offset_r16 := omit) := { /* @status APPROVED (ENDC, NR5GC) */ periodicity := p_Periodicity, nrofHARQ_Processes := 8, n1PUCCH_AN := 0, mcs_Table := p_MCS_Table, sps_ConfigIndex_r16 := p_SPS_ConfigIndex_r16, harq_ProcID_Offset_r16 := p_Harq_ProcID_Offset_r16, periodicityExt_r16 := omit, harq_CodebookID_r16 := omit, pdsch_AggregationFactor_r16 := omit, sps_HARQ_Deferral_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ n1PUCCH_AN_PUCCHsSCell_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ periodicityExt_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ nrofHARQ_Processes_v1710 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ harq_ProcID_Offset_v1700 := omit /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ }; v_ConfigGrant_Period,

After Change:

template (value) SPS_Config cs_SPS_Config_Common (SPS_Config.periodicity p_Periodicity := ms160, template(omit) SPS_Config.mcs_Table p_MCS_Table := omit, template(omit) SPS_Config.sps_ConfigIndex_r16 p_SPS_ConfigIndex_r16 := omit, template(omit) SPS_Config.harq_ProcID_Offset_r16 p_Harq_ProcID_Offset_r16 := omit) := { /* @status APPROVED (ENDC, NR5GC) */ periodicity := p_Periodicity, nrofHARQ_Processes := 8, n1PUCCH_AN := 6, mcs_Table := p_MCS_Table, sps_ConfigIndex_r16 := p_SPS_ConfigIndex_r16, harq_ProcID_Offset_r16 := p_Harq_ProcID_Offset_r16, periodicityExt_r16 := omit, harq_CodebookID_r16 := omit, pdsch_AggregationFactor_r16 := omit, sps_HARQ_Deferral_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ n1PUCCH_AN_PUCCHsSCell_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ periodicityExt_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ nrofHARQ_Processes_v1710 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ harq_ProcID_Offset_v1700 := omit /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ };