3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230896

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3287 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.7.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.7.2 to the IWD_23wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.7.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.7.2	4

2.1	f_TC_11_7_2_NR5GC_TestBody	4

2.2	f_GetTestcaseAttrib_EarlyContentionResolution	10

2.3	f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle	11

2.4	f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive	13

2.5	cs_NG_NAS_EDRXParams	14

3	Branches executed	15

4	Execution Log Files	15

4.1	Hisilicon Balong 5000	15

5	References	16

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.7.2 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D23wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.7.2.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required to test case 11.7.2

## f_TC_11_7_2_NR5GC_TestBody

| Function name | f_TC_11_7_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation: - For Steps 19Aa1-20 the function f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); is called. This does not handle the case when pc_noOf_PDUsNewConnection >0. This needs to be handled. - At Step 19, the TTCN needs to use the latest PDCP counts and refresh the AS security with those sequence numbers otherwise the rrcResume message may not be received by the UE. This needs to be addressed. Same change is also needed after Steps 27 and 34. - In Step 27, the eDRX cycle length is set to 2.56s, whereas a\ccording to the Prose it should be 5.12s. This needs to be corrected. |
| Summary of change | - For Steps 19Aa1-20, instead of calling f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); called function f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); This takes care of both situations. - Called the function f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); after Steps 19, 27 and 34. - Updated the TTCN at Step 27 to use correct eDRX cycle length. |
| TTCN module | EDRX_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_7_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.2.3.3-2: REGISTRATION ACCEPT // @sic R5-233412 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) SuspendConfig v_SuspendConfig; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE? //Check: Does the UE transmit a REGISTRATION COMPLETE message? f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-18" siclog@ //Void //@siclog "Steps 19Aa1-20" siclog@ //Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf512); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1); //@siclog "Step 22" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 22"); //@siclog "Step 23" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 24" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 25" siclog@ //Void //@siclog "Step 26" siclog@ //The SS transmits an RRCRelease message with suspend. v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf256); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 27" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 0); //@siclog "Step 28" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 28"); //@siclog "Step 29" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 30" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message with suspend. f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 32" siclog@ //The SIB1 is updated according to Table 11.7.2.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_CellInfo_SetSIB1_hyperSFN_r17(nr_Cell1, omit); // return value by default f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(nr_Cell1); f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //Modify Sysinfo in SS //@siclog "Step 33" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 35" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 35"); //@siclog "Step 36" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 37" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); }//end of f_TC_11_7_2_NR5GC_TestBody

After Change:

function f_TC_11_7_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.2.3.3-2: REGISTRATION ACCEPT // @sic R5-233412 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) SuspendConfig v_SuspendConfig; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE? //Check: Does the UE transmit a REGISTRATION COMPLETE message? f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-18" siclog@ //Void //@siclog "Steps 19Aa1-20" siclog@ //Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //WA#11_7_2 COMMENTED OUT f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); //WA#11_7_2 v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf512); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 22" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 22"); //@siclog "Step 23" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 24" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 25" siclog@ //Void //@siclog "Step 26" siclog@ //The SS transmits an RRCRelease message with suspend. v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf256); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 27" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1 /*WA#11_7_2 -->1*/); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 28" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 28"); //@siclog "Step 29" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 30" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message with suspend. f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 32" siclog@ //The SIB1 is updated according to Table 11.7.2.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_CellInfo_SetSIB1_hyperSFN_r17(nr_Cell1, omit); // return value by default f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(nr_Cell1); f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //Modify Sysinfo in SS //@siclog "Step 33" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 35" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 35"); //@siclog "Step 36" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 37" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); }//end of f_TC_11_7_2_NR5GC_TestBody

## f_GetTestcaseAttrib_EarlyContentionResolution

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | In the function f_TC_11_7_2_NR5GC_TestBody at Step 22 upon reception of the rrcResumeREQ, a preconfigured rrcSetup will be transmitted. This is incorrect, and needs to be addressed.. Hence it is proposed that the testcase 11.7.2 be incuded in this function, after which the preconfigured rrcSetup will not be transmitted. |
| Summary of change | Included testcase 11.7.2 in function f_GetTestcaseAttrib_EarlyContentionResolution |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } <<SKIPPED CODE>>

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_2_NR5GC") { return true; } //WA#11_7_2 case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } <<SKIPPED CODE>>

## f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle

| Function name | f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle |
| --- | --- |
| Reason for change | The current TTCN implementation only allows to set the IE eDRX_AllowedIdle_r17 to TRUE, but it does not allow it to set to OMIT. This needs to be corrected. |
| Summary of change | Updated the function so that depending on the parameter p_EDRX_AllowedIdle_r17 the IE eDRX_AllowedIdle_r17 is set to TRUE or OMIT. Please see below. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedIdle_r17 p_EDRX_AllowedIdle_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdle_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := valueof(p_EDRX_AllowedIdle_r17); } } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedIdle_r17 p_EDRX_AllowedIdle_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdle_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := valueof(p_EDRX_AllowedIdle_r17); } } else { //WA#11_7_2 v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := omit; } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive

| Function name | f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive |
| --- | --- |
| Reason for change | The current TTCN implementation only allows to set the IE eDRX_AllowedInactive_r17 to TRUE, but it does not allow it to set to OMIT. This needs to be corrected. |
| Summary of change | Updated the function so that depending on the parameter p_EDRX_AllowedIdle_r17 the IE eDRX_AllowedInactive_r17 is set to TRUE or OMIT. Please see below. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedInactive_r17 p_EDRX_AllowedIdleInactive_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdleInactive_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := valueof(p_EDRX_AllowedIdleInactive_r17); } } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedInactive_r17 p_EDRX_AllowedIdleInactive_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdleInactive_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := valueof(p_EDRX_AllowedIdleInactive_r17); } } else { //WA#11_7_2 v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := omit; } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## cs_NG_NAS_EDRXParams

| Function name | cs_NG_NAS_EDRXParams |
| --- | --- |
| Reason for change | In the current TTCN implementation, the iel is set incorrectly, this needs to be corrected. |
| Summary of change | Changed the iel to ‘02’O. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) ExtdDRXParams cs_NG_NAS_EDRXParams (B4_Type p_PTW, B4_Type p_EDRXVal, template (omit) B8_Type p_ExtdPTW := omit) := { /* @status */ iei := '6E'O, iel := '01'O, pagingTimeWindow := p_PTW, eDRXValue := p_EDRXVal, extdPTW := p_ExtdPTW };

After Change:

template (value) ExtdDRXParams cs_NG_NAS_EDRXParams (B4_Type p_PTW, B4_Type p_EDRXVal, template (omit) B8_Type p_ExtdPTW := omit) := { /* @status */ iei := '6E'O, iel := '02'O, // length //WA#11_7_1 01 --> 02 (Also needed for 11_7_2, 11_7_3 and 8_1_5_2_3) pagingTimeWindow := p_PTW, eDRXValue := p_EDRXVal, extdPTW := p_ExtdPTW };

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

## Hisilicon Balong 5000

The Hisilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

tc_11_7_2_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230897: Supporting information for addition of NR5GC testcase 11.7.2 in FR1 |
| --- | --- |