3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230904

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3292 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A conformant UE would request for an UL grant immediately to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the OnSR UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |  |  |  |  |  |  |  |  |
| Summary of change: |  | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody and fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC, 7.1.1.9.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | A conformant UE would request for an UL grant to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |
| Summary of change | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ <<SKIPPED CODE>> // @siclog "Step 4" siclog@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); <<SKIPPED CODE>>

After Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ //WA# var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ <<SKIPPED CODE>> // @siclog "Step 4" siclog@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" WA# //Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList, -, v_IsDataRecvd); //WA#

## Change 2

| Function name | fl_TC_7_1_1_9_1_ENDC_TestBody |
| --- | --- |
| Reason for change | The change mentioned in Change 1 necessitates the handling of the optional MAC PDU at Step 4A within function fl_TC_7_1_1_9_1_ENDC_TestBody as f_TC_7_1_1_9_1_Step5to9A is common between NR5GC and ENDC components |
| Summary of change | Moved the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s230860 sic@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode); //@sic R5s200855 R5s220838 sic@ <<SKIPPED CODE>>

After Change:

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ //WA var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s230860 sic@ // Initialization of RLC variables WA# v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode, -, v_IsDataRecvd); //@sic R5s200855 R5s220838 sic@ WA# <<SKIPPED CODE>>

## Change 3

| Function name | f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | A conformant UE would request for an UL grant to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |
| Summary of change | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody and fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { //@sic R5-206320 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true, boolean p_IsDataRecvd //WA# ) runs on NR_BASE_PTC return boolean { //@sic R5-206320 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := p_IsDataRecvd; //WA# /*REMOVED var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } }*/ <<SKIPPED CODE>>