3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230023

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2882 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-16 DAPS Mobilty Enh test case 8.1.4.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 8.1.4.3.5 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.4.3.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	7

4	Execution Log Files	8

4.1	MediaTek MT6879	8

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.3.5 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.4.3.5

ATS Version:	iwd-TTCN3-B2020-09_D22wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	MediaTek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig() |
| --- | --- |
| Reason for change | - For DAPS bearer from 38.331, 2> If any DAPS bearer is configured: 3> create a MAC entity for the target cell group with the same configuration as the MAC entity for the source cell group. 3> for each DAPS bearer: 4> establish an RLC entity or entities for the target cell group, with the same configurations as for the source cell group. So SS should keep same LogicalChannelId for target cell. - Configurations on target cell should be activated immediately (cs_TimingInfo_Now), as target can miss its actvation time as UE can send RRCreconfigurationcomplete immediately after getting DAPS HO configuration while SS might be busy in activating configuration for radio bearer, pdcp count, security, SR activation at target cell according to their time. |
| Summary of change | - Commented LogicalChannelId modification. - Modified target cell activation time with cs_TimingInfo_Now. |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | - Accepted. - This change affects the DAPS HO sequence endorsed in R5w210214r1. Accepted in principle but alternative solucion proposed in R5w230004 (R5-230116), where step 4 a) of the procedure is done before T with activation Now. |

Before Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return template (value) NR_RadioBearerList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); //v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.1.3.3-4 //Anritsu TTCN CR R5s221151 MCC if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // (R5w210214r1 Step 1. Target Cell: Configuration of DRBs) v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof (v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId) + 1; //Configure DRB for DAPS f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 3. Source Cell: Send RRCReconfiguration) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); // (R5w210214r1 Step 4. Transfer of the PDCP Count for the DAPS DRB and SRBs (if applied) from source to target cell // a) Source Cell: Get PDCP COUNT. // b) Target Cell: Set PDCP COUNT. // NOTE 2: For DAPS DRB the PDCP count is maintained. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, v_RadioBearerList, -, cs_TimingInfo_NR(v_Timing2), false); //p_RBConfig_KeyChange := true, p_IncrementSRB1 := false // (R5w210214r1 Step 5. Target Cell: Inform the SS about the HO and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // (R5w210214r1 Step 6. Target Cell: Activate security. // RBConfig_KeyChange =true -> ActivateSecurity_Current f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, tsc_NR_38508_NextHopChainingCount, cs_TimingInfo_NR(v_Timing2)); // (R5w210214r1 Step 7. Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_NR(v_Timing2), -, -, cs_UL_GrantConfig_OnSR); // (R5w210214r1 Step 8. Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_NR(v_Timing2), -, cs_NR_UplinkTimeAlignment_Start, -); //return list of bearers to be used later in the procedure return v_RadioBearerList; }

After Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return template (value) NR_RadioBearerList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); // @sic R5s221151 sic@ Deleted "v_RACH_ConfigGeneric.preambleTransMax := n200;" if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // (R5w210214r1 Step 1. Target Cell: Configuration of DRBs) //v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof (v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId); //Configure DRB for DAPS f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, cs_TimingInfo_Now); // (R5w210214r1 Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_Now); // (R5w210214r1 Step 3. Source Cell: Send RRCReconfiguration) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); // (R5w210214r1 Step 4. Transfer of the PDCP Count for the DAPS DRB and SRBs (if applied) from source to target cell // a) Source Cell: Get PDCP COUNT. // b) Target Cell: Set PDCP COUNT. // NOTE 2: For DAPS DRB the PDCP count is maintained. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, v_RadioBearerList, -, cs_TimingInfo_Now, false); //p_RBConfig_KeyChange := true, p_IncrementSRB1 := false // (R5w210214r1 Step 5. Target Cell: Inform the SS about the HO and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // (R5w210214r1 Step 6. Target Cell: Activate security. // RBConfig_KeyChange =true -> ActivateSecurity_Current f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, tsc_NR_38508_NextHopChainingCount, cs_TimingInfo_Now); // (R5w210214r1 Step 7. Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, -, cs_UL_GrantConfig_OnSR); // (R5w210214r1 Step 8. Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); //return list of bearers to be used later in the procedure return v_RadioBearerList; }

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_4_3_5_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_4_3_5_PIXIT.xml

# References

| [1] | R5s230024 : NR5GC FR1 : Supporting information for addition of Rel-16 Mobility enhancement test case 8.1.4.3.5 |
| --- | --- |