3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230031

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2885 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 6.1.2.15a in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 6.1.2.15a to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.15a required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.15a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_TC_6_1_2_15a_NR5GC()	4

3	Branches executed	7

4	Execution Log Files	7

4.1	Qualcomm	7

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.1.2.15a which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	6.1.2.15a.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform and Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System

Verification Status:	PASS

# Corrections required

## Function f_TC_6_1_2_15a_NR5GC()

| Function name | f_TC_6_1_2_15a_NR5GC() |
| --- | --- |
| Reason for change | - Template cs_38508_SIB1_Def within fl_NR_InitSIB1() can only handle one PLMN_IdentityInfo IE. So, current implementation will not be able to create and populate two PLMN_IdentityInfo IE within the PLMN-IdentityInfoList as per Table 6.1.2.15a.3.3-2 SIB1 for NR Cell 11 requirement and will lead to crash on accessing index greater than 1 in the list via f_NR_CellInfo_SetPLMN, f_NR_CellInfo_SetTAC, f_NR_CellInfo_SetRANAC, f_NR_CellInfo_SetCellIdentity - As per 6.1.2.15a.3.2 Test procedure sequence Max reference Power for NR Cell11 is incorrect. |
| Summary of change | - Added a new PLMN_IdentityInfoList template and filled it with desired information as per Table 6.1.2.15a.3.3-2 requirement and removed the existing implementation via f_NR_CellInfo_SetPLMN, f_NR_CellInfo_SetTAC, f_NR_CellInfo_SetRANAC, f_NR_CellInfo_SetCellIdentity. Also, configured NR Cell 11 NAS_Parameters.PLMN_Identity separately as f_NR_CellInfo_SetPLMN is no longer used to configure it. - Corrected the Max reference Power for NR Cell11 as per the test prose. |
| TTCN module | 6_1_2\Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | - Accepted but implemented differently. See MCC160 implementation below. - Accepted |

Before Change:

function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_001_02); // Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_Def, 0, 1); // Table 6.1.2.15a.3.3-2: RAN Area Code f_NR_CellInfo_SetRANAC(nr_Cell11, 2); f_NR_CellInfo_SetRANAC(nr_Cell11, 1, 1); // Table 6.1.2.15a.3.3-2: cellIdentity f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000110'B & '0000001011'B); f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000101'B & '0000001010'B, 1); // Table 6.1.2.15a.3.3-2: trackingAreaCode f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(2,24)); f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(3,24),1); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -75, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC

After Change:

function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities var template(omit) PLMN_IdentityInfoList v_PLMN_IdentityInfoList := omit; //WA#WI=1041047 f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 // Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 //WA#WI=1041047 v_PLMN_IdentityInfoList := { cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_001_02, int2bit(2,24), 2, '00000000000000000000000110'B & '0000001011'B), cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_Def, int2bit(3,24), // TAI ID 1, '00000000000000000000000101'B & '0000001010'B) } var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell11); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := tsc_NR_HPLMN_Def; v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList := valueof(v_PLMN_IdentityInfoList); f_NR_CellInfo_Set(nr_Cell11, v_NR_CellInfo); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -82); // WA#WI=1041047 Max Power level - Table 6.1.2.15.3.2-1: // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC

MCC160 implementation:

module Idle_CellReSelection_NR5GC

function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities //@sic R5s230031 sic@ var template (value) PLMN_IdentityInfoList v_PLMN_IdentityInfoList; f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 v_PLMN_IdentityInfoList := { cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_001_02, int2bit(2,24), 2, '00000000000000000000000110'B & '0000001011'B), cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_Def, int2bit(3,24), // TAI ID 1, '00000000000000000000000101'B & '0000001010'B)}; f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell11, v_PLMN_IdentityInfoList );// Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC

module NR_CellInfo. New function.

function f_NR_CellInfo_SetPLMNIdentityInfo_List (NR_CellId_Type p_NR_CellId, template (value) PLMN_IdentityInfoList p_PLMN_IdentityInfoList, PLMN_Identity p_PLMN_Identity := tsc_NR_HPLMN_Def, template(omit) NPN_IdentityInfoList_r16 p_NPN_IdentityInfoList_r16 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := p_PLMN_Identity; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList := valueof(p_PLMN_IdentityInfoList); // SNPN if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16)){ //Handle one SNPN entry by default v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16 := valueof(p_NPN_IdentityInfoList_r16); } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

# Branches executed

## This NR5GC testcase was executed on NR Primary Band “n41” with NEA1 ciphering and NIA1 integrity algorithms.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon X70 5G Modem-RF System passed this test case on R&S® 5G Protocol Conformance Test platform and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S:
tc_6_1_2_15a_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

Keysight:

TC_6_1_2_15a_LOG.html

TC_6_1_2_15a_PIXIT.xml

# References

| [1] | R5s230032: Supporting information for addition of NR5GC test case 6.1.2.15a in FR1. |
| --- | --- |