3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230035

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2887 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT testcase 11.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN implementation of function f_TC_11_4_12_NR5GC_EUTRA the following functions are called in this sequence : f_EUTRA38_NR5GC_PreambleAndSwitchOff(eutra_Cell1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR There is no need to call the highlighted coordination messages above, as these are already called within function f_EUTRA38_NR5GC_PreambleAndSwitchOff. Calling them twice derails the TTCN flow and causes testcase failure. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the calls to functions : f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR from the function f_TC_11_4_12_NR5GC_EUTRA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_11_4_12_NR5GC_EUTRA	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Correction to the Function f_TC_11_4_12_NR5GC_EUTRA

| Function name | f_TC_11_4_12_NR5GC_EUTRA |
| --- | --- |
| Reason for change | In current TTCN implementation of function f_TC_11_4_12_NR5GC_EUTRA the following functions are called in this sequence : f_EUTRA38_NR5GC_PreambleAndSwitchOff(eutra_Cell1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR There is no need to call the highlighted coordination messages above, as these are already called within function f_EUTRA38_NR5GC_PreambleAndSwitchOff. Calling them twice derails the TTCN flow and causes testcase failure. This needs to be corrected. |
| Summary of change | Removed the calls to functions : f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR from the function f_TC_11_4_12_NR5GC_EUTRA |
| TTCN module | NR5GC_IRAT_Emergency_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_11_4_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA, USE_BIG_GRANTS); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff(eutra_Cell1); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR // Ok now registered on both RATs f_EUTRA_TestBody_Set(true); fl_TC_11_4_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

function f_TC_11_4_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA, USE_BIG_GRANTS); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff(eutra_Cell1); //WA#11_4_12 COMMENTED OUT f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); //Trigger to register on NR //WA#11_4_12 COMMENTED OUT f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); //Successfully registered on NR // Ok now registered on both RATs f_EUTRA_TestBody_Set(true); fl_TC_11_4_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }