3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230068

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2908 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to TS 38.214 (Release15) section 5.1.2.1, PDSCH aggregation uses Table 5.1.2.1-2: Redundancy version when pdsch-AggregationFactor is present, the SS stack will use first row of this table, with RV sequence 0, 2, 3, 1. Hence, there is no need to configure for the DL DCI a list of redundancy versions (0,2,3,1) - As per the ASP definition for SymbolTimingInfo_Type, the IE FirstSymbol is used in REQ ASPs only. In this case for the IND ASP on the UL, the Symbol IE should be set to “Any” as symbol level information is not relevant in this TC for the Uplink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set the Redundancy Version List to None - Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.2.ENDC, 7.1.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | f_TC_7_1_1_2_2_NR_TestBody () |
| --- | --- |
| Reason for change | - According to TS 38.214 (Release 15) section 5.1.2.1, PDSCH aggregation uses Table 5.1.2.1-2: Redundancy version when pdsch-AggregationFactor is present, the SS stack will use the first row of this table, with RV sequence 0, 2, 3, 1. Hence, there is no need to configure for the DL DCI a list of redundancy versions (0,2,3,1) - As per the ASP definition for SymbolTimingInfo_Type, the IE FirstSymbol is used in REQ ASPs only. In this case for IND ASP on the UL, the Symbol IE should be set to “Any” as symbol level information is not relevant in this TC for the Uplink. |
| Summary of change | - Set the Redundancy Version List to None - Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | 1. Accepted, implemented differently using cs_NR_RedundancyVersionList_SingleRV(0), so that there is no retransmission at step 2. See R5s230065 and see proposed implementation below. |

Before Change:

function f_TC_7_1_1_2_2_NR_TestBody( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation <CODE SKIPPED> var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_1_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing,cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('011'B)), cs_NR_RedundancyVersionList_Def); <CODE SKIPPED> // @siclog "Steps 3-4" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } <CODE SKIPPED> // @siclog "Step 7" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // @siclog "Step 8" siclog@ // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. //Set CRC error as normal mode f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_Now)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } <CODE SKIPPED>

After Change:

function f_TC_7_1_1_2_2_NR_TestBody( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation

<CODE SKIPPED> var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_1_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing,cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('011'B)), cs_NR_RedundancyVersionList_None); //WA# <CODE SKIPPED> // @siclog "Steps 3-4" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing v_TimingUL.Symbol := valueof(cs_SymbolTimingInfo_Any); //WA# select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } <CODE SKIPPED> // @siclog "Step 7" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // @siclog "Step 8" siclog@ // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. //Set CRC error as normal mode f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_Now)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing v_TimingUL.Symbol := valueof(cs_SymbolTimingInfo_Any); //WA# select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } }

New Template:

template (value) SymbolTimingInfo_Type cs_SymbolTimingInfo_Any := { Any := true };

MCC160 proposed implementation

function f_TC_7_1_1_2_2_NR_TestBody( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_SYSTEM_IND v_SYS_IND; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_1_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing,cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('011'B)), cs_NR_RedundancyVersionList_Def); var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 0.4; //Configure DL DCI as per Table 7.1.1.2.2.3.3-3: Physical layer parameters for DCI format 1_1 (Steps 3, 7, Table 7.1.1.2.2.3.2-1) //Set K1=5 f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); //By default in TS 38.508-1, the Time Alignment timer is set to infinity //configure SS for no Auto Time alignement - no grant needed f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5s221035 sic@ //Config SS to report: NR_SystemRequest_Type -> NR_System_IndicationControl_Type -> UL_HARQ //NR_SYSTEM_IND -> NR_SystemIndication_Type-> UL_HARQ: ack/nack f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s221035 sic@ v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_36B))); //@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // @siclog "Steps 3-4" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 5" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC error on UE side. (Note3) // SS already configured to do it - PDSCH aggregation // @siclog "Step 6" siclog@ // Check: Does the UE transmit a HARQ NACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack))-> value v_SYS_IND { v_Nack_Count := v_Nack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Nack_Count == 1) { //@sic R5s230068 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); } // @siclog "Step 7" siclog@ // The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE, the rv_idx is 0. // @siclog "Step 8" siclog@ // The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. //Set CRC error as normal mode f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_Now)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); v_TimingUL := v_TimingDL; //Assign DL and UL timing select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 4; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 1; v_TimingUL.Subframe.Number := 1; v_TimingUL.Slot.SlotOffset.Numerology1 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // @siclog "Step 9" siclog@ // In the following 3 consecutive slots, the SS transmits on the same downlink assignment a MAC PDU including a RLC PDU, The CRC is calculated in such a way, it will result in CRC pass on UE side. (Note3) t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@sic R5s221035 sic@ // @siclog "Step 10" siclog@ // Check: Does the UE transmit a HARQ ACK on slot n3+k1? (Note 4) alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))-> value v_SYS_IND { v_Ack_Count := v_Ack_Count + 1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } if (match(v_SYS_IND.Common.TimingInfo, cr_TimingInfo(v_TimingUL.SFN.Number, v_TimingUL.Subframe.Number, -, v_TimingUL.Slot)) and v_Ack_Count == 1) { //@sic R5s230068 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } // Configure SS to stop indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //@sic R5s221035 sic@ }