3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230074

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2913 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC CAG selection test case 6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR5GC CAG TC 6.5.2.1 is failing on 22WK49 TTCN at step 24 as the UE is unable to find the NR Cell 4 for manual CAG selection. This is due to the SIB1 for Cell 4 is missing the manualCAGselectionAllowed-r16 IE set to true. NR Cell2 SIB1 is also missing the manualCAGselectionAllowed-r16 IE as per the Prose |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList() and related function to allow set the manualCAGselectionAllowed-r16 to true_ for Cell 2 and 4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	6

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | f_TC_6_5_2_1_NR5GC |
| --- | --- |
| Reason for change | NR5GC CAG TC 6.5.2.1 is failing on 22WK49 TTCN at step 24 as the UE is unable to find the NR Cell 4 for manual CAG selection. This is due to the SIB1 for Cell 4 is missing the manualCAGselectionAllowed-r16 IE set to true. NR Cell2 SIB1 is also missing the manualCAGselectionAllowed-r16 IE as per the Prose |
| Summary of change | In function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList() passed a new boolean parameter ‘true_’ for NR cell 4 In function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList() passed a new boolean parameter ‘true_’ for NR cell 2 |
| TTCN module | CAG_Selection_NR5GC |
| MCC160 Comment | Accepted in principle, but implemented differently |

Before Change

function f_TC_6_5_2_1_NR5GC() runs on NR5GC_PTC { //CAG Selection in Manual Mode var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR5GC_Init(NR_2); f_NR_CellInfo_SetPLMN(nr_Cell2, tsc_NR_HPLMN_003_21);//PLMN3 - @sic R5-223381 sic@ f_NR_CellInfo_SetPLMN(nr_Cell4, tsc_NR_HPLMN_002_11);//PLMN2 //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 - @sic R5s220768 sic@ f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_);//@sic R5-225305 sic@ f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_);//@sic R5-225305 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -88, -88); //FR2 FFS … … … }

After Change

function f_TC_6_5_2_1_NR5GC() runs on NR5GC_PTC { //CAG Selection in Manual Mode var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR5GC_Init(NR_2); f_NR_CellInfo_SetPLMN(nr_Cell2, tsc_NR_HPLMN_003_21);//PLMN3 - @sic R5-223381 sic@ f_NR_CellInfo_SetPLMN(nr_Cell4, tsc_NR_HPLMN_002_11);//PLMN2 //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 - @sic R5s220768 sic@ f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2, -, true_); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2, true_); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_);//@sic R5-225305 sic@ f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_);//@sic R5-225305 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -88, -88); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -88, -88); //FR2 FFS … … … }

  - Change 2

| Function name | f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList |
| --- | --- |
| Reason for change | See change 1 |
| Summary of change | Passed a new parameter in function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList with default value ‘omit’ |
| TTCN module | NR_CellInfo |
| MCC160 Comment | Accepted in principle, but implemented differently |

Before Change

function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList( NR_CellId_Type p_NR_CellId, integer p_Cag_Identity := 1 ) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList ; //v_NPN_IdentityInfoList := f_NR_GetPNI_NPN_IdentityInfoList(v_NR_CellInfo.NAS_Parameters, p_Cag_Identity); f_NR_CellInfo_SetSIB1_SetNPN_IdentityInfoList(p_NR_CellId, valueof(v_NPN_IdentityInfoList)); };

After Change

function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList( NR_CellId_Type p_NR_CellId, integer p_Cag_Identity := 1, template (omit) CAG_IdentityInfo_r16.manualCAGselectionAllowed_r16 p_ManualCAGselectionAllowed := omit ) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList ; v_NPN_IdentityInfoList := f_NR_GetPNI_NPN_IdentityInfoList(v_NR_CellInfo.NAS_Parameters, p_Cag_Identity, p_ManualCAGselectionAllowed); f_NR_CellInfo_SetSIB1_SetNPN_IdentityInfoList(p_NR_CellId, valueof(v_NPN_IdentityInfoList)); };

  - Change 3

| Function name | f_NR_GetPNI_NPN_IdentityInfoList |
| --- | --- |
| Reason for change | See change 1 |
| Summary of change | Passed a new parameter in function f_NR_GetPNI_NPN_IdentityInfoList with default value ‘omit’ |
| TTCN module | NR_CellInfo |
| MCC160 Comment | Accepted in principle, but implemented differently |

Before Change

function f_NR_GetPNI_NPN_IdentityInfoList( template(value) NR_NAS_Parameters_Type p_NAS_Parameters, integer p_Cag_Identity := 1) return template(omit) NPN_IdentityInfoList_r16 { var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList := omit; v_NPN_IdentityInfoList := {cs_NPN_IdentityInfo_Def(cs_NPN_IdentityCAG ( valueof(p_NAS_Parameters.PLMN_Identity), valueof(cs_CAG_IdentityInfo(int2bit(p_Cag_Identity,32)))), p_NAS_Parameters.TrackingAreaCode, p_NAS_Parameters.Ranac, p_NAS_Parameters.CellIdentity) } return v_NPN_IdentityInfoList; }

After Change

function f_NR_GetPNI_NPN_IdentityInfoList( template(value) NR_NAS_Parameters_Type p_NAS_Parameters, integer p_Cag_Identity := 1, template (omit) CAG_IdentityInfo_r16.manualCAGselectionAllowed_r16 p_ManualCAGselectionAllowed := omit) return template(omit) NPN_IdentityInfoList_r16 { var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList := omit; v_NPN_IdentityInfoList := {cs_NPN_IdentityInfo_Def(cs_NPN_IdentityCAG ( valueof(p_NAS_Parameters.PLMN_Identity), valueof(cs_CAG_IdentityInfo(int2bit(p_Cag_Identity,32), p_ManualCAGselectionAllowed))), p_NAS_Parameters.TrackingAreaCode, p_NAS_Parameters.Ranac, p_NAS_Parameters.CellIdentity) } return v_NPN_IdentityInfoList; }

  - Change 4

| Function name | cs_CAG_IdentityInfo |
| --- | --- |
| Reason for change | See change 1 |
| Summary of change | Passed a new parameter in template cs_CAG_IdentityInfo with default value ‘omit’ and used it to initialize the IE manualCAGselectionAllowed_r16 |
| TTCN module | NR_SysInfo_Templates |
| MCC160 Comment | Accepted in principle, but implemented differently |

Before Change

template(value) CAG_IdentityInfo_r16 cs_CAG_IdentityInfo(B32_Type p_CAG_Identity) := { /* @status APPROVED (NR5GC) */ cag_Identity_r16 := p_CAG_Identity, manualCAGselectionAllowed_r16 := omit };

After Change

template(value) CAG_IdentityInfo_r16 cs_CAG_IdentityInfo(B32_Type p_CAG_Identity, template (omit) CAG_IdentityInfo_r16.manualCAGselectionAllowed_r16 p_ManualCAGselectionAllowed := omit) := { /* @status APPROVED (NR5GC) */ manualCAGselectionAllowed_r16 := p_ManualCAGselectionAllowed };

MCC160 implementation

template(value) CAG_IdentityInfo_r16 cs_CAG_IdentityInfo(B32_Type p_CAG_Identity) := { // Acc to 38.508-1 Table 4.6.3-84AA: NPN-Identity with condition CAG /* @status APPROVED (NR5GC) */ cag_Identity_r16 := p_CAG_Identity, manualCAGselectionAllowed_r16 := true_ //@sic R5s230074 sic@ };