3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230077

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2915 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC test case 8.2.2.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13. |
| Summary of change | Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |
| TTCN module | RRC_Reconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment | Alternative solution implemented: add DRB2 to v_ConfigDrbSS at steps 7, 10, 13 and 16 so that it is released, then re-configured in f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange |

Before Change

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ ... // @siclog "Steps 13 - 14" siclog@ // SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ ... }

After Change

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ ... // @siclog "Steps 13 - 14" siclog@ // SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); //New Change v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ ... }

#### MCC160 Implementation

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ //@sic R5-198928 sic@

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

var template (value) RadioBearerList_Type v_SrbDRBList;

var template (value) octetstring v_NR_SCGConfig;

var template (value) octetstring v_NR_RBConfig;

var SubFrameTiming_Type v_Timing;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();  //@sic R5s200317 sic@

// @siclog "Steps 1 - 2" siclog@

// SN to MN terminated SCG

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 4-5" siclog@

// modify MN terminated SCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 6" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 7 - 8 " siclog@

// modify SN terminated Split DRB with security key s-KgNB to MN terminated Split DRB with security key change to keNB

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)};  //@sic R5s200171 change 4.2 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)};

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max());

v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1, -, -, -,{cds_DRB_ToAddMod_r15(tsc_DRB2, tsc_DRB2 + 4, -, cs_508_RLC_Config_DRB_AM, {cs_ReestablishRLC_r15}, cs_508_LogicalChannelConfig_DRB_AM)});

//@sic R5s200171 change 4.2 sic@

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-, -, -,

v_RadioResourceConfigDedicated

);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 9" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 10 - 11 " siclog@

// modify MN terminated Split DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB.

//@sic R5-201101 sic@

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Release(v_NR_RBConfig,

v_Security_Params.SK_Counter);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)};  //@sic R5s200171 change 4.2 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)};

//@sic R5s200171 change 4.2 R5s221053 sic@

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-,-, -, -,

cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// @siclog "Step 12" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 13 - 14" siclog@

// SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)};  //@sic R5s200171 change 4.2 sic@

// NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@

//@sic R5s200171 change 4.2 sic@

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (-,

v_NR_RBConfig,

v_Security_Params.SK_Counter,

fl_NR_GetP_Max());

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,  //@sic R5s221053 sic@

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-,-, -, -,

cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def) );

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14");

// @siclog "Step 15" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 16 - 17" siclog@

// modify MN terminated MCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)};  //@sic R5s200171 change 4.2 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)};

//@sic R5s200171 change 4.2 sic@

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max());

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,    //@sic R5s221053 sic@

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17");

// @siclog "Step 18" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 19 - 20" siclog@

// modify SN terminated Split DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2);

v_ConfigDrbSS := {cs_DRB_Release(2)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

// @siclog "Step 21" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 22 - 23" siclog@

// modify MN terminated SCG DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB.

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1,

v_ConfigDrbSS, //RadioBearerList_Type

v_RadioResourceConfigDedicated, //RadioResourceConfigDedicated

-,

v_NR_RBConfig,

-, // p_NAS_MsgList

-, // p_SK_Counter

-, // p_MaxEUTRA

-, // RRC_TransactionIdentifier

-, // MeasConfig p_MeasurementConfiguration

-, // MobilityControlInfo

-, // p_ULNASMsg

cs_TimingInfo_EUTRA_NB(v_Timing),

-,-,-,true);

f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR);  //default security process during PSCell change

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 23");

// @siclog "Step 24" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 25 - 26" siclog@

// modify SN terminated MCG DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB.

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2);

v_ConfigDrbSS := {cs_DRB_Release(2)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 26");

// @siclog "Step 27" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// Test body now finished

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

}