3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230087

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2924 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MAC is being put into TM too early causing the reception of the BSR to TTCN where there is no handling for this. The reception of the BSR happens because of the reconfigurationWithSync procedure due to which RACH procedure is triggered on the NR side, the resulting UL DCI to resolve Contention Resolution to finish up the RACH procedure is used by the UE to send the BSR. As MAC is already in TM mode, this gets sent to TTCN which then fails the TC as the BSR gets handled in the default handler. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MAC is initially configured in Normal Mode to finish the configuration of the SCG DRB and then moved into TM before the TestBody. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | f_TC_7_1_1_6_1_ENDC_NR () |
| --- | --- |
| Reason for change | MAC is being put into TM too early causing the reception of the BSR to TTCN where there is no handling for this. The reception of the BSR happens because of the reconfigurationWithSync procedure due to which RACH procedure is triggered on the NR side, the resulting UL DCI to resolve Contention Resolution to finish up the RACH procedure is used by the UE to send the BSR. As MAC is already in TM mode, this gets sent to TTCN which then fails the TC as the BSR gets handled in the default handler |
| Summary of change | MAC is initially configured in Normal Mode to finish the configuration of the SCG DRB and then moved into TM before the TestBody. |
| TTCN module | MAC_SPS_ENDC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; var template (omit) ServingCellConfig v_ServingCellConfig; //@sic R5s221195 sic@ //Init Cell parameters f_ENDC_NR_Init(); <CODE SKIPPED> //@sic R5s221195 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); <CODE SKIPPED>

After Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; //WA# var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; var template (omit) ServingCellConfig v_ServingCellConfig; //@sic R5s221195 sic@ //Init Cell parameters f_ENDC_NR_Init(); <CODE SKIPPED> //@sic R5s221195 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId);