3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230096

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2932 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 6.2.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - RRCRelease message contents are not correclty populated in case of higher EUTRA bands than 64 #Associated proseCR would be raise in RAN5#98 meeting - Need template to have EARFCN for carrierFreq_v9e0 for reselection at step 1 which have band combination higher than 64. - Need a function which can return DL CarrierFreq_v9e0 earfcn values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added if condition for the freuqncy band higher than 64 with new variables v_EARFCN_DL_v9e0, v_RRCConnectionRelease_v890 and modify variable v_IdleModeMobilityControlInfo for higher EUTRA bands. - Added template cs_IdleModeMobCntrlInfo_EUTRA_v9e0 - Added f_EUTRA_CellInfo_GetEARFCN_DL_v9e0 to get the DL arfcn for CarrierFreq_v9e0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_6_2_3_6_NR5GC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | RRCRelease message contents are not correclty populated in case of higher EUTRA bands than 64 #Associated proseCR would be raise in RAN5#98 meeting |
| Summary of change | Added if condition for the freuqncy band higher than 64 with new variables v_EARFCN_DL_v9e0, v_RRCConnectionRelease_v890 and modify variable v_IdleModeMobilityControlInfo for higher EUTRA bands. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#98 |

Before Change:

function fl_TC_6_2_3_6_NR5GC_EUTRA_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on EUTRA_Multi5GS_PTC { var ARFCN_ValueEUTRA v_EARFCN_DL := f_EUTRA_CellInfo_GetEARFCN_DL_r9(eutra_Cell1); var integer v_T1_Cell1_RS_EPRE_FR1 := -95; var integer v_T2_Cell1_RS_EPRE_FR1 := -80; var template (value) IdleModeMobilityControlInfo v_IdleModeMobilityControlInfo; v_IdleModeMobilityControlInfo := cs_IdleModeMobCntrlInfo_EUTRA_NR(cs_FreqPriorityListEUTRA_1Entry( v_EARFCN_DL, 4), {cs_FreqPriorityListNR_1Entry(p_CoOrd_SysInfo.NR[0].Arfcn, 3)}); //@siclog "Step 1" siclog@ // The SS transmmits RRCConnectionRelease message with dedicated priority information and validtity timer to the UE on E-UTRA cell 1. f_EUTRA_RRC_ConnectionRelease_Common(eutra_Cell1, cs_RRCConnectionRelease(tsc_RRC_TI_Def, other, -, v_IdleModeMobilityControlInfo)); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //change power levels also on NR side //@siclog "Step 2" siclog@ //The SS adjusts the E-UTRAN and NR Cell power levels according to row "T1" in table 6.2.3.6.3.2-1. f_EUTRA38_SetCellPower(eutra_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1); //Step3: Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and the UE is camped on NR Cell 1 f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); //@siclog "Step 5" siclog@ // The SS adjusts the E-UTRAN and NR Cell power levels according to row "T2" in table 6.2.3.6.3.2-1. f_EUTRA38_SetCellPower(eutra_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1); //@siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, -, -, -, -, -, -, false); // @sic R5s201004 sic@ } // end of fl_TC_6_2_3_6_NR5GC_EUTRA_TestBody

After Change:

function fl_TC_6_2_3_6_NR5GC_EUTRA_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on EUTRA_Multi5GS_PTC { var ARFCN_ValueEUTRA v_EARFCN_DL := f_EUTRA_CellInfo_GetEARFCN_DL_r9(eutra_Cell1); var integer v_T1_Cell1_RS_EPRE_FR1 := -95; var integer v_T2_Cell1_RS_EPRE_FR1 := -80; var template (value) IdleModeMobilityControlInfo v_IdleModeMobilityControlInfo; var template(omit) RRCConnectionRelease_v890_IEs v_RRCConnectionRelease_v890_IEs := omit; var integer eutra_band := f_EUTRA_CellInfo_GetBand(eutra_Cell1); var ARFCN_ValueEUTRA_v9e0 v_EARFCN_DL_v9e0; if (eutra_band > 64) { // @sic R5-171439 sic@ v_EARFCN_DL_v9e0 := f_EUTRA_CellInfo_GetEARFCN_DL_v9e0(eutra_Cell1); v_IdleModeMobilityControlInfo := cs_IdleModeMobCntrlInfo_EUTRA_NR(cs_FreqPriorityListEUTRA_1Entry( maxEARFCN, 4), {cs_FreqPriorityListNR_1Entry(p_CoOrd_SysInfo.NR[0].Arfcn, 3)}); v_RRCConnectionRelease_v890_IEs := cs_RRCConnectionRelease_All_IEs_v9e0(omit,cs_IdleModeMobCntrlInfo_EUTRA_v9e0(v_EARFCN_DL_v9e0)); } else { v_IdleModeMobilityControlInfo := cs_IdleModeMobCntrlInfo_EUTRA_NR(cs_FreqPriorityListEUTRA_1Entry( v_EARFCN_DL, 4), {cs_FreqPriorityListNR_1Entry(p_CoOrd_SysInfo.NR[0].Arfcn, 3)}); } //@siclog "Step 1" siclog@ // The SS transmmits RRCConnectionRelease message with dedicated priority information and validtity timer to the UE on E-UTRA cell 1. f_EUTRA_RRC_ConnectionRelease_Common(eutra_Cell1, cs_RRCConnectionRelease(tsc_RRC_TI_Def, other, -, v_IdleModeMobilityControlInfo,v_RRCConnectionRelease_v890_IEs)); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //change power levels also on NR side //@siclog "Step 2" siclog@ //The SS adjusts the E-UTRAN and NR Cell power levels according to row "T1" in table 6.2.3.6.3.2-1. f_EUTRA38_SetCellPower(eutra_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1); //Step3: Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and the UE is camped on NR Cell 1 f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); //@siclog "Step 5" siclog@ // The SS adjusts the E-UTRAN and NR Cell power levels according to row "T2" in table 6.2.3.6.3.2-1. f_EUTRA38_SetCellPower(eutra_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1); //@siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed and the UE is camped on E-UTRAN Cell 1? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, -, -, -, -, -, -, false); // @sic R5s201004 sic@ } // end of fl_TC_6_2_3_6_NR5GC_EUTRA_TestBody

## Change 2

| Function name | template cs_IdleModeMobCntrlInfo_EUTRA_v9e0 |
| --- | --- |
| Reason for change | Need template to have EARFCN for carrierFreq_v9e0 for reselection at step 1 which have band combination higher than 64. |
| Summary of change | Added template cs_IdleModeMobCntrlInfo_EUTRA_v9e0 |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Not needed. cs_IdleModeMobCntrlInfo_v9e0_1Entry will be used instead |

Add Function:

template(value) IdleModeMobilityControlInfo_v9e0 cs_IdleModeMobCntrlInfo_EUTRA_v9e0(ARFCN_ValueEUTRA_v9e0 dl_CarrierFreq_v9e0) := { freqPriorityListEUTRA_v9e0 := { { carrierFreq_v9e0 := dl_CarrierFreq_v9e0 } } };

## Change 3

| Function name | function f_EUTRA_CellInfo_GetEARFCN_DL_v9e0() |
| --- | --- |
| Reason for change | Need a function which can return DL CarrierFreq_v9e0 earfcn values. |
| Summary of change | Added f_EUTRA_CellInfo_GetEARFCN_DL_v9e0 to get the DL arfcn for CarrierFreq_v9e0. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#98 |

Add Function:

function f_EUTRA_CellInfo_GetEARFCN_DL_v9e0(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC return ARFCN_ValueEUTRA_v9e0 { var EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); return v_CellInfo.Frequency.UL_DL_Earfcn.dl_CarrierFreq_v9e0; }