3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230110

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2940 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.5.1.1 and EN-DC RRC test case 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the 38.331, the fr1-fr2-Add-UE-NR-Capabilities.fr1-fr2-Add-UE-NR-Capabilities only contains csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework. The current TTCN implementation incorrectly checks others capabilies IEs as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the extra check made for UE-NR-CapabilityAddFRX-Mode from the function fl_Check_NR_Capability_1540(). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_Check_NR_Capability_1540 (UE_NR_Capability p_NRCapability) |
| --- | --- |
| Reason for change | As per the 38.331, the fr1-fr2-Add-UE-NR-Capabilities.fr1-fr2-Add-UE-NR-Capabilities only contains csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework. The current TTCN implementation incorrectly checks others capabilies IEs as well. UE-NR-Capability-v1540 field descriptions. fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.. |
| Summary of change | Remove the check made for UE-NR-CapabilityAddFRX-Mode from the function fl_Check_NR_Capability_1540(). |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_Check_NR_Capability_1540 (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension )) { if (not f_NR_CompareCapability_vs_PICS(pc_as_ReflectiveQoS, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.sdap_Parameters.as_ReflectiveQoS ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_as_ReflectiveQoS not correctly set"); } if(isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) //@sic R5s221147 Ch. 3 sic@ { // Checks on fr1_fr2_Add_UE_NR_Capabilities if (not f_NR_CompareCapability_vs_PICS(pc_ss_SINR_Meas, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.ss_SINR_Meas ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ss_SINR_Meas not correctly set"); // @sic R5-225384 sic@ } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithSSB ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithoutSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithoutSSB ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithoutSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_handoverInterF, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.handoverInterF ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_handoverInterF not correctly set"); } } } // nonCriticalExtension is OPTIONAL else { if (pc_as_ReflectiveQoS or pc_csi_RSRP_AndRSRQ_MeasWithSSB or pc_ss_SINR_Meas or pc_csi_RSRP_AndRSRQ_MeasWithoutSSB or pc_handoverInterF) { f_SetVerdict(inconc,__FILE__, __LINE__, "Setting of UE-NR-Capability-1540 PICS is inconsistent with UE reported capabilities"); } } }; //end of fl_Check_NR_Capability_1540

After Change

function fl_Check_NR_Capability_1540 (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension )) { if (not f_NR_CompareCapability_vs_PICS(pc_as_ReflectiveQoS, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.sdap_Parameters.as_ReflectiveQoS ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_as_ReflectiveQoS not correctly set"); } } // nonCriticalExtension is OPTIONAL else { if (pc_as_ReflectiveQoS) { f_SetVerdict(inconc,__FILE__, __LINE__, "Setting of UE-NR-Capability-1540 PICS is inconsistent with UE reported capabilities"); } } }; //end of fl_Check_NR_Capability_1540