3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230115

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4708 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS test case 11.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 11.3.6 to the LTE_A_PRO ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.3.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	8

2.4	Change 4	9

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm SA515M	10

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.3.6 which is part of the LTE_A_PRO test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.3.6

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SA515M

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_3_6_EUTRA() |
| --- | --- |
| Reason for change | The UE may (or may not) have performed detach procedure during test body |
| Summary of change | Moved postamble handling into test body function (see change 2) |
| TTCN module | IMS_ECall.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_TC_11_3_6_EUTRA() runs on EUTRA_PTC

{ /* eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted */

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c1, -, HANDLE_UM_DATA, USE_BIG_GRANTS);

//Set PLMN identifier to PLMN4

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_CellA, cs_HPLMN_004_31);

// Create cell and send out system information

// Specific SIB1 support for IMS emergency support and eCall support will be set in default functions

f_EUTRA_CellConfig_Def(eutra_CellA);

// Insert USIM for eCall only enabled

// The UE is equipped with a USIM containing values shown in Table 11.3.6.3.1-2.

f_UT_USIM_Insert (UT, "36.508 Table 4.9.3.5-2 and contains the values shown in Table 11.3.6.3.1-2");

// Switch cell on

f_EUTRA_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE);

// The UE is in state Switched OFF (state 1)

f_EUTRA_TestBody_Set(true);

fl_TC_11_3_6_Body();

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_CellA, E1_IDLE); // optional IMS Deregistration is handled in the default, before the UE detaches

}

After change

function f_TC_11_3_6_EUTRA() runs on EUTRA_PTC

{ /* eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted */

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c1, -, HANDLE_UM_DATA, USE_BIG_GRANTS);

//Set PLMN identifier to PLMN4

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_CellA, cs_HPLMN_004_31);

// Create cell and send out system information

// Specific SIB1 support for IMS emergency support and eCall support will be set in default functions

f_EUTRA_CellConfig_Def(eutra_CellA);

// Insert USIM for eCall only enabled

// The UE is equipped with a USIM containing values shown in Table 11.3.6.3.1-2.

f_UT_USIM_Insert (UT, "36.508 Table 4.9.3.5-2 and contains the values shown in Table 11.3.6.3.1-2");

// Switch cell on

f_EUTRA_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE);

// The UE is in state Switched OFF (state 1)

f_EUTRA_TestBody_Set(true);

fl_TC_11_3_6_Body();

// Postamble

// MOVED f_EUTRA_TestBody_Set(false);

// MOVED f_EUTRA_Postamble(eutra_CellA, E1_IDLE); // optional IMS Deregistration is handled in the default, before the UE detaches

// Test ends in fl_TC_11_3_6_Body

}

## Change 2

| Function name | fl_TC_11_3_6_Body() |
| --- | --- |
| Reason for change | - At the beginning of the test case IP handling should be started for the emergency PDN (PDN_2 using DRB 2) - Incorrect EPS bearer ID used for deactivation - In this Limited Service scneario the UE will attach for EPS emergency services, so the attach type used is “EPS emergency attach”. However, during the postamble the detach type evaluated from PIXIT settings may be “combined EPS/IMSI detach”. - DETACH REQUEST message at step 27Aa1 will be received on SRB2 not SRB1 - The t_Watchdog timer is not stopped - The UE may perform detach at step 27Aa2 |
| Summary of change | - Pass PDN_2 and DRB 2 into f_EUTRA_SwitchOnUEandStartIP() - Correct bearer ID to tsc_EpsDedicatedBearerId2 in call to f_EUTRA_DeactivateEPS_BearerContext() - Pass p_ForcedAttach “EPS_Only” in postamble function. - Replace SRB1 with SRB2 for receiving DETACH REQUEST and sending DETACH ACCEPT. - Stop t_Watchdog on receipt of DETACH REQUEST - Add postamble call in this function, with state depending on if the UE already detached during the test body. |
| TTCN module | IMS_ECall.ttcn |
| MCC160 Comment | - Accepted - Accepted - Accepted - Accepted - Accepted - Accepted |

Before change

function fl_TC_11_3_6_Body() runs on EUTRA_PTC

{

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType ();

var NAS_MSG_Indication_Type v_NasInd;

var NAS_AttDetValue_Type v_DetachType;

var GutiParameters_Type v_Guti1Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var float v_EcallDeregisteredInactive := 60.0;

timer t_EcallNoRRCConnReq := 120.0;

timer t_Watchdog := 5.0;

//@siclog "Step 1" siclog@

// The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

// Wait for 60s for the UE to enter EMM-DEREGISTERED.eCALL-INACTIVE state

f_Delay(v_EcallDeregisteredInactive);

//@siclog "Step 3" siclog@

// An eCall to URI for test service is initiated.

f_UT_InitiateECall(UT, TEST);

//@siclog "Step 4" siclog@

// Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds?

t_EcallNoRRCConnReq.start;

alt {

[] t_EcallNoRRCConnReq.timeout {} // Continue the test

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2 - Unexpected RRCConnectionRequest");

}

}

//@siclog "Step 5" siclog@

// A manual eCall is initiated.

f_UT_InitiateECall(UT, MANUAL);

//@siclog "Step 6" siclog@

// Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'?

f_EUTRA_RRC_ConnectionRequest_Def(eutra_CellA, emergency);

//@siclog "Step 7" siclog@

// SS transmits an RRCConnectionSetup message.

f_EUTRA_RRC_ConnectionSetup_Def (eutra_CellA, -, tsc_RRC_TI_Def);

//@siclog "Step 8" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency?

v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA,

cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?),

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cr_508_ATTACH_REQUEST(tsc_EAttachType_Emergency,

v_AdditionalUpdateType),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_Emergency,

f_GetPdnType())));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 9-18" siclog@

// Steps 5 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1).

f_EUTRA_508_IMSEmergencyCallLimitedService_Steps7_16(eutra_CellA, v_NasInd); // @sic R5-227236 sic@

//EXCEPTION: In parallel to the events described in steps 19 to 23 below, the behaviour in table 11.3.6.2-2 occurs.

//@siclog "Step 19-23" siclog@

// Steps 15 to 19 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1).

f_EUTRA_508_IMSEmergencyCallLimitedService_Steps17_19(eutra_CellA); // @sic R5-227236 sic@

//@siclog "Step 24-27" siclog@

// Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33  @sic R5-196019 sic@

//IMS call has been released.

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// The SS deactivates the EPS bearer. [This is the rest of C.33]

f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA,

tsc_EpsDedicatedBearerId); // there is only one PDN

//EXCEPTION: Step 27Aa1 and 27Aa2 describes optional behaviour that depends on the UE implementation.

v_DetachType :=  f_GetEDetachType(NORMAL);

t_Watchdog.start;

alt {

[] t_Watchdog.timeout {} // Continue the test

[] //@siclog "Step 27Aa1" siclog@

// The UE transmits a DETACH REQUEST message with the Detach type IE indicating "EPS detach" to regain normal service.

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellA, //@sic R5-227236 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_DETACH_REQUEST_MO(?,

f_GutiParameters2MobileIdentity(omit, v_Guti1Params),

cr_DetachType( '0'B, v_DetachType))))) {

//@siclog "Step 27Aa2" siclog@

// The SS responds the DETACH ACCEPT message.

SRB.send (cas_SRB1_NasPdu_REQ (eutra_CellA, //@sic R5-227236 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_DETACH_ACCEPT))); };

}

//@siclog "Step 29" siclog@

// The SS releases the RRC connectionafter 5secs of Step 27..

f_EUTRA_RRC_ConnectionRelease(eutra_CellA);

}

After change

function fl_TC_11_3_6_Body() runs on EUTRA_PTC

{

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType ();

var NAS_MSG_Indication_Type v_NasInd;

var NAS_AttDetValue_Type v_DetachType;

var GutiParameters_Type v_Guti1Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var float v_EcallDeregisteredInactive := 60.0;

timer t_EcallNoRRCConnReq := 120.0;

timer t_Watchdog := 5.0;

var boolean v_Detached := false;

//@siclog "Step 1" siclog@

// The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA, -, -, -, PDN_2, tsc_DRB2);

//@siclog "Step 2" siclog@

// Wait for 60s for the UE to enter EMM-DEREGISTERED.eCALL-INACTIVE state

f_Delay(v_EcallDeregisteredInactive);

//@siclog "Step 3" siclog@

// An eCall to URI for test service is initiated.

f_UT_InitiateECall(UT, TEST);

//@siclog "Step 4" siclog@

// Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds?

t_EcallNoRRCConnReq.start;

alt {

[] t_EcallNoRRCConnReq.timeout {} // Continue the test

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2 - Unexpected RRCConnectionRequest");

}

}

//@siclog "Step 5" siclog@

// A manual eCall is initiated.

f_UT_InitiateECall(UT, MANUAL);

//@siclog "Step 6" siclog@

// Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'?

f_EUTRA_RRC_ConnectionRequest_Def(eutra_CellA, emergency);

//@siclog "Step 7" siclog@

// SS transmits an RRCConnectionSetup message.

f_EUTRA_RRC_ConnectionSetup_Def (eutra_CellA, -, tsc_RRC_TI_Def);

//@siclog "Step 8" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency?

v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA,

cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?),

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cr_508_ATTACH_REQUEST(tsc_EAttachType_Emergency,

v_AdditionalUpdateType),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_Emergency,

f_GetPdnType())));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 9-18" siclog@

// Steps 5 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1).

f_EUTRA_508_IMSEmergencyCallLimitedService_Steps7_16(eutra_CellA, v_NasInd); // @sic R5-227236 sic@

//EXCEPTION: In parallel to the events described in steps 19 to 23 below, the behaviour in table 11.3.6.2-2 occurs.

//@siclog "Step 19-23" siclog@

// Steps 15 to 19 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1).

f_EUTRA_508_IMSEmergencyCallLimitedService_Steps17_19(eutra_CellA); // @sic R5-227236 sic@

//@siclog "Step 24-27" siclog@

// Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33  @sic R5-196019 sic@

//IMS call has been released.

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// The SS deactivates the EPS bearer. [This is the rest of C.33]

f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA,

tsc_EpsDedicatedBearerId2); // there is only one PDN

//EXCEPTION: Step 27Aa1 and 27Aa2 describes optional behaviour that depends on the UE implementation.

v_DetachType :=  f_GetEDetachType(EPS_Only);

t_Watchdog.start;

alt {

[] t_Watchdog.timeout {} // Continue the test

[] //@siclog "Step 27Aa1" siclog@

// The UE transmits a DETACH REQUEST message with the Detach type IE indicating "EPS detach" to regain normal service.

SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA, //@sic R5-227236 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_DETACH_REQUEST_MO(?,

f_GutiParameters2MobileIdentity(omit, v_Guti1Params),

cr_DetachType( '0'B, v_DetachType))))) {

t_Watchdog.stop;

//@siclog "Step 27Aa2" siclog@

// The SS responds the DETACH ACCEPT message.

SRB.send (cas_SRB2_NasPdu_REQ (eutra_CellA, //@sic R5-227236 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_DETACH_ACCEPT)));

v_Detached := true;

}

}

//@siclog "Step 29" siclog@

// The SS releases the RRC connectionafter 5secs of Step 27..

f_EUTRA_RRC_ConnectionRelease(eutra_CellA);

f_EUTRA_TestBody_Set(false);

if( v_Detached == false) {

f_EUTRA_Postamble(eutra_CellA, E1_IDLE, EPS_Only);

}

else {

f_EUTRA_Postamble(eutra_CellA, E4_DEREGISTERED);

}

}

## Change 3

| Function name | f_TC_11_3_6_IMS2() |
| --- | --- |
| Reason for change | - Invite request would be received with context A6 (INVITE for creating an emergency session in case of no registration) as per 34.229-1 and section A.2.1, see change 2.4 - Following emergency call release, associated TCP sockets should be closed before IPCAN release. |
| Summary of change | - Removed emergency registration procedure - Added function to close TCP sockets. |
| TTCN module | IMS_ECall_IMS.ttcn |
| MCC160 Comment | - Accepted - Accepted Additional change needed as shown in the MCC160 Implementation |

Before change

function f_TC_11_3_6_IMS2() runs on IMS_PTC

{ /* IMS implementation for PDN2 */

var SipUrl v_ContactUrl;

f_IMS_PTC_Init(PDN_2);

f_IMS_EmergencyRegistration();

f_ECall_CallSetup_AnnexC47(MANUAL);

f_ECall_MSDUpdate_AnnexC47();

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog);

f_IMS_CallReleaseMT(v_ContactUrl);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

After change

function f_TC_11_3_6_IMS2() runs on IMS_PTC

{ /* IMS implementation for PDN2 */

var SipUrl v_ContactUrl;

f_IMS_PTC_Init(PDN_2);

// REMOVED f_IMS_EmergencyRegistration();

f_ECall_CallSetup_AnnexC47(MANUAL);

f_ECall_MSDUpdate_AnnexC47();

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog);

f_IMS_CallReleaseMT(v_ContactUrl);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_TCP_CloseHandling(waitAndClose);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

#### MCC160 Implementation:

As a result of the MCC160 Implementation of change 4 (see below) the TTCN has to be changed as shown hereafter.

After change

function f_TC_11_3_6_IMS2() runs on IMS_PTC

{ /* IMS implementation for PDN2 */

var SipUrl v_ContactUrl;

f_IMS_PTC_Init(PDN_2);

// REMOVED f_IMS_EmergencyRegistration();

f_ECall_CallSetup_AnnexC47(MANUAL, A_2_1_A6 );

f_ECall_MSDUpdate_AnnexC47();

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog);

f_IMS_CallReleaseMT(v_ContactUrl);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_TCP_CloseHandling(waitAndClose);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

## Change 4

| Function name | f_ECall_InitialInvite_AnnexC47() |
| --- | --- |
| Reason for change | According to note 1 in Table 11.3.6.3.2-2: Parallel behaviour, Invite request would be received with context A6 as per 34.229-1 and section A.2.1 |
| Summary of change | Passed the context as A_2_1_A6 as input to the Annex C47 procedure. |
| TTCN module | IMS_eCall_Functions.ttcn |
| MCC160 Comment | Accepted in principle, but added this as a prarameter |

Before change

function f_ECall_InitialInvite_AnnexC47(ECall_Type p_ECallType) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var MIME_Message v_MIME_Message;

var MIME_Part v_MIME_Part;

var SDP_Message v_SdpOffer;

var template (present) SDP_Message v_SDP_Message_Step1;

var template (present) SipUrl v_ECallPSAP_Rx := f_SipUrl_eCall_RX(p_ECallType);

var template (present) MessageHeader v_MessageHeader;

var template (present) AcceptBody_List v_AcceptBodyList := { cr_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), * };    // condition A20, A21

var template (present) SemicolonParam_List v_DispositionParams := { cr_GenericParam("handling", "optional"), * };             // condition A20, A21

var boolean v_A8 := pc_IMS_Geolocation;

var INVITE_A_2_1_Context_Type v_Context := A_2_1_A7;

var charstring v_CidUrl;

var ECall_Type v_ECallType := p_ECallType;

// Step 1. Receive INVITE for an eCall

v_MessageHeader := f_IMS_InviteRequest_MessageHeaderRX(v_ECallPSAP_Rx, -, v_Context, -, v_A8);

v_MessageHeader.accept := cr_Accept(v_AcceptBodyList);                         // condition A20, A21, A25 @sic R5-227567 sic@

v_MessageHeader.callInfo := cr_CallInfo(-, "EmergencyCallData.eCall.MSD");     // condition A20, A21 @sic R5-227567, R5-210324: "EmergencyCallData" instead of "emergencyCallData" sic@

if ((v_ECallType == MANUAL) or (v_ECallType == AUTOMATIC)) { // @sic R5-220971 sic@

v_MessageHeader.recvInfo := cr_RecvInfo_Def("EmergencyCallData.eCall.MSD");    // condition A20, A21

}

v_MessageHeader.contentType := cr_ContentTypeMIME;                             // condition A20, A21, A8, A25 @sic R5-217960 sic@

v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ECallPSAP_Rx, v_MessageHeader));

v_MIME_Message := v_IMS_DATA_REQ.Request.Invite.messageBody.mimeMessageBody;

// Check PIDF

if (v_A8) {

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Pidf);

f_IMS_DecodeAndCheckGeolocation(v_MIME_Part.body.xmlBody);

}

// Check SDP

v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX();   // Build RX SDP template using codecs AMR and AMR_WB

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication);

v_SdpOffer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, v_SDP_Message_Step1);

// Check eCall MSD

v_CidUrl := v_IMS_DATA_REQ.Request.Invite.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_MIME_Message, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl, v_DispositionParams);

return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ, v_SdpOffer);

}

After change

function f_ECall_InitialInvite_AnnexC47(ECall_Type p_ECallType) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var MIME_Message v_MIME_Message;

var MIME_Part v_MIME_Part;

var SDP_Message v_SdpOffer;

var template (present) SDP_Message v_SDP_Message_Step1;

var template (present) SipUrl v_ECallPSAP_Rx := f_SipUrl_eCall_RX(p_ECallType);

var template (present) MessageHeader v_MessageHeader;

var template (present) AcceptBody_List v_AcceptBodyList := { cr_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), * };    // condition A20, A21

var template (present) SemicolonParam_List v_DispositionParams := { cr_GenericParam("handling", "optional"), * };             // condition A20, A21

var boolean v_A8 := pc_IMS_Geolocation;

var INVITE_A_2_1_Context_Type v_Context := A_2_1_A7;

var charstring v_CidUrl;

var ECall_Type v_ECallType := p_ECallType;

if(testcasename()== "TC_11_3_6") {

v_Context := A_2_1_A6;

}

// Step 1. Receive INVITE for an eCall

v_MessageHeader := f_IMS_InviteRequest_MessageHeaderRX(v_ECallPSAP_Rx, -, v_Context, -, v_A8);

v_MessageHeader.accept := cr_Accept(v_AcceptBodyList);                         // condition A20, A21, A25 @sic R5-227567 sic@

v_MessageHeader.callInfo := cr_CallInfo(-, "EmergencyCallData.eCall.MSD");     // condition A20, A21 @sic R5-227567, R5-210324: "EmergencyCallData" instead of "emergencyCallData" sic@

if ((v_ECallType == MANUAL) or (v_ECallType == AUTOMATIC)) { // @sic R5-220971 sic@

v_MessageHeader.recvInfo := cr_RecvInfo_Def("EmergencyCallData.eCall.MSD");    // condition A20, A21

}

v_MessageHeader.contentType := cr_ContentTypeMIME;                             // condition A20, A21, A8, A25 @sic R5-217960 sic@

v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ECallPSAP_Rx, v_MessageHeader));

v_MIME_Message := v_IMS_DATA_REQ.Request.Invite.messageBody.mimeMessageBody;

// Check PIDF

if (v_A8) {

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Pidf);

f_IMS_DecodeAndCheckGeolocation(v_MIME_Part.body.xmlBody);

}

// Check SDP

v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX();   // Build RX SDP template using codecs AMR and AMR_WB

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication);

v_SdpOffer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, v_SDP_Message_Step1);

// Check eCall MSD

v_CidUrl := v_IMS_DATA_REQ.Request.Invite.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_MIME_Message, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl, v_DispositionParams);

return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ, v_SdpOffer);

}

#### MCC160 Implementation:

function f_ECall_CallSetup_AnnexC47(ECall_Type p_ECallType, INVITE_A_2_1_Context_Type p_Context := A_2_1_A7) runs on IMS_PTC

{

f_ECall_CallSetup_Common(p_ECallType, { AMR_16000, AMR_8000 }, p_Context);

}

function f_ECall_CallSetup_Common(ECall_Type p_ECallType,

IMS_SDP_AudioCodecList_Type p_AudioCodecList, INVITE_A_2_1_Context_Type p_Context := A_2_1_A7) runs on IMS_PTC

{ //Allowed to be re-used for 5G with different codec @sic R5-223392 sic@

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

// Step 1. Receive INVITE for an eCall

v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(p_ECallType, p_Context);

f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, p_AudioCodecList); // Split Invite into a separate function @sic R5-227567 sic@

}

function f_ECall_InitialInvite_AnnexC47(ECall_Type p_ECallType, INVITE_A_2_1_Context_Type p_Context := A_2_1_A7) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var MIME_Message v_MIME_Message;

var MIME_Part v_MIME_Part;

var SDP_Message v_SdpOffer;

var template (present) SDP_Message v_SDP_Message_Step1;

var template (present) SipUrl v_ECallPSAP_Rx := f_SipUrl_eCall_RX(p_ECallType);

var template (present) MessageHeader v_MessageHeader;

var template (present) AcceptBody_List v_AcceptBodyList := { cr_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), * };    // condition A20, A21

var template (present) SemicolonParam_List v_DispositionParams := { cr_GenericParam("handling", "optional"), * };             // condition A20, A21

var boolean v_A8 := pc_IMS_Geolocation;

var charstring v_CidUrl;

var ECall_Type v_ECallType := p_ECallType;

// Step 1. Receive INVITE for an eCall

v_MessageHeader := f_IMS_InviteRequest_MessageHeaderRX(v_ECallPSAP_Rx, -, p_Context, -, v_A8);

v_MessageHeader.accept := cr_Accept(v_AcceptBodyList);                         // condition A20, A21, A25 @sic R5-227567 sic@

v_MessageHeader.callInfo := cr_CallInfo(-, "EmergencyCallData.eCall.MSD");     // condition A20, A21 @sic R5-227567, R5-210324: "EmergencyCallData" instead of "emergencyCallData" sic@

if ((v_ECallType == MANUAL) or (v_ECallType == AUTOMATIC)) { // @sic R5-220971 sic@

v_MessageHeader.recvInfo := cr_RecvInfo_Def("EmergencyCallData.eCall.MSD");    // condition A20, A21

}

v_MessageHeader.contentType := cr_ContentTypeMIME;                             // condition A20, A21, A8, A25 @sic R5-217960 sic@

v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(p_Context, v_ECallPSAP_Rx, v_MessageHeader));

v_MIME_Message := v_IMS_DATA_REQ.Request.Invite.messageBody.mimeMessageBody;

// Check PIDF

if (v_A8) {

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Pidf);

f_IMS_DecodeAndCheckGeolocation(v_MIME_Part.body.xmlBody);

}

// Check SDP

v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX();   // Build RX SDP template using codecs AMR and AMR_WB

v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication);

v_SdpOffer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, v_SDP_Message_Step1);

// Check eCall MSD

v_CidUrl := v_IMS_DATA_REQ.Request.Invite.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_MIME_Message, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl, v_DispositionParams);

return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ, v_SdpOffer);

}

# Branches executed

The test case was executed on EUTRA band 4 using ciphering algorithm eea1 and integrity algorithm eia1

# Execution Log Files

## Qualcomm SA515M

The Qualcomm SA515M passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_3_6_LOG.html

- PICS/PIXIT parameter file:

TC_11_3_6_PIXIT.xml

# References

| [1] | R5s230116 : Supporting information for addition of eCall over IMS test case 11.3.6 |
| --- | --- |