3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230118

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2945 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR/5GC: Re-verification of Rel-15 test case 7.1.2.3.10 against a RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Re-verification of Rel-15 test case 7.1.2.3.10 against a RedCap UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.2.3.10 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant Redcap UE will fail the case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	MediaTek MT6835	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.2.3.10 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.2.3.10

ATS Version:	iwd-TTCN3-B2020-09_D22wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	MediaTek MT6835

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | crs_NR_RLC_NACK_SN12_SOs |
| --- | --- |
| Reason for change | At step 21, There is no next NACK SN to be set, so E1 should be 0 instead 1 |
| Summary of change | Added p_E1 with default value as '0'B |
| TTCN module | NR_RLC_Common |
| MCC160 Comment | Accepted |

Before Change

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_SOs(integer p_NackSN, integer p_SOstart, integer p_SOstop) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := 1, /* 1 bit E1 field */ E2 := '1'B, /* 1 bit E2 field */ E3 := '0'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := int2bit(p_SOstart, 16), SOstop := int2bit(p_SOstop, 16), NACKrange := omit };

After Change

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_SOs(integer p_NackSN, integer p_SOstart, integer p_SOstop, B1_Type p_E1 := '0'B) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := p_E1, /* 1 bit E1 field */ E2 := '1'B, /* 1 bit E2 field */ E3 := '0'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := int2bit(p_SOstart, 16), SOstop := int2bit(p_SOstop, 16), NACKrange := omit };

## Change 2

| Function name | crs_NR_RLC_NACK_SN12_Simple |
| --- | --- |
| Reason for change | At step 5, step 10 and step 17, There is no next NACK SN to be set as per prose, so E1 should be 0 instead 1. |
| Summary of change | Added p_E1 with default value as '0'B |
| TTCN module | NR_RLC_Common |
| MCC160 Comment | Accepted |

Before Change

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_Simple(integer p_NackSN) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := p_E1, /* 1 bit E1 field */ E2 := '0'B, /* 1 bit E2 field */ E3 := '0'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := omit, SOstop := omit, NACKrange := omit };

After Change

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_Simple(integer p_NackSN, B1_Type p_E1 := '0'B) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := p_E1, /* 1 bit E1 field */ E2 := '0'B, /* 1 bit E2 field */ E3 := '0'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := omit, SOstop := omit, NACKrange := omit };

Additional Changes

1. Provide crs_NR_RLC_NACK_SN12_Range with E1 as parameter.

To harmonize with the above changes and the corresponding templates for SN18

After Change

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_Range(integer p_NackSN, integer p_NACKrange, B1_Type p_E1 := '0'B) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := p_E1, /* 1 bit E1 field */ E2 := '0'B, /* 1 bit E2 field */ E3 := '1'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := omit, SOstop := omit, NACKrange := int2bit(p_NACKrange, 8) };

2. Correct side-effects of these changes in RLC AM RedCap Test cases 7.1.2.3.6 to 7.1.2.3.10

2.1 crs_NR_RLC_NACK_SN12_SOs

TC 7.1.2.3.9 not yet tested for RedCap - corrected by this change

2.2 crs_NR_RLC_NACK_SN12_Simple

TC 7.1.2.3.6 step 12:

v_NR_NackSN12Bit_1 := crs_NR_RLC_NACK_SN12_Simple (4, '1'B);

TC 7.1.2.3.7,  TC 7.1.2.3.8 and  TC 7.1.2.3.9 not yet tested for RedCap - corrected by this change

2.3 crs_NR_RLC_NACK_SN12_Range

TC 7.1.2.3.6 step 12:

v_NR_NackSN12Bit_2 := crs_NR_RLC_NACK_SN12_Range (5, 2 , '1'B);

TC 7.1.2.3.7 step 40:

v_NR_NackSN12Bit_1 := crs_NR_RLC_NACK_SN12_Range (13, 3, '1'B);

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6835

The MediaTek MT6835 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_2_3_10_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_2_3_10_PIXIT.xml

# References

| [1] | R5s230119: NR5GC FR1 : Supporting information for Re-Verification of NR5GC TC 7.1.2.3.10 against Redcap UE |
| --- | --- |