3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230121

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4709 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implemenation at step 81 the UE transmits a DETACH REQUEST and if the normal detach is performed the SS responds with DETACH ACCPET message in step 81Aa1 which is integrity protected and ciphered. However according to Prose TS 33.401 Clause 6.4 Handling of EPS security contexts : - Any EPS security context shall be deleted from the ME if the UICC is removed from the ME when the ME is in power on state Similary in both steps 86 and 87, Detach and Detach accept procedures are integrity protected and ciphered and do not handle the scenario when the pics pc_USIM_Removal set to true (USIM removal is performed). The current TTCN implementation needs to accommodate this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Detach Accept message at step 81Aa1 replaced the const “tsc_SHT_IntegrityProtected_Ciphered” with “tsc_SHT_NoSecurityProtection” and added detach and detach accept procedures without security protection to handle the scenario when pics pc_USIM_Removal set to true at steps 86 and 87 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f1_TC_22_5_6_Body	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## function f1_TC_22_5_6_Body

| Function name | fl_TC_22_5_6_Body |
| --- | --- |
| Reason for change | In the current TTCN implemenation at step 81 the UE transmits a DETACH REQUEST and if the normal detach is performed the SS responds with DETACH ACCPET message in step 81Aa1 which is integrity protected and ciphered. However according to Prose TS 33.401 Clause 6.4 Handling of EPS security contexts: - Any EPS security context shall be deleted from the ME if the UICC is removed from the ME when the ME is in power on state Similary at both steps 86 and 87, Detach and Detach accept procedures are integrity protected and ciphered and do not handle the scenario when the pics pc_USIM_Removal set to true (USIM removal is performed). The current TTCN implementation needs to accommodate this. |
| Summary of change | In Detach Accept message at step 81Aa1 replaced the const “tsc_SHT_IntegrityProtected_Ciphered” with “tsc_SHT_NoSecurityProtection” and added detach and detach accept procedures without security protection to handle the scenario when pics pc_USIM_Removal set to true at steps 86 and 87 respectively. |
| TTCN module | NBIOT_NAS_Attach.ttcn |
| MCC160 Comment | 1. 36.508 states that Detach Accept (at 81Aa1) should be sent with same protection as Request, therefore need to check incoming Detach Request. 2. 24.304 cl.4.4.4.2 does not include Detach Request in the list of messages the UE will process without passing the integrity check. Therefore, the Detach Request at step 86 CANNOT be sent without security protection. Alternative solution proposed, subject to prose CR approval, is to perform authentication and security before sending the Detach Request at step 86. |

Before Change:

function fl_TC_22_5_6_Body() runs on NBIOT_PTC { const NAS_CauseValue_Type tsc_EmmCause_MessageIncompatibleWithProtocolState := '01100101'B; // @sic R5s180246 sic@ //@sic R5-183208 sic@ var template (value) NB_CellPowerList_Type v_CellPowerList; var NAS_MSG_Indication_Type v_NAS_Ind; var template (present) NAS_UL_Message_Type v_AttachReq; var template (present) NAS_UL_Message_Type v_ESMMessage; var template (present) B3_Type v_PDNTypeRxd := (f_GetPdnType(), '101'B); var B3_Type v_PDNType; var GutiParameters_Type v_Guti_Params_Cell50 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell50); var GutiParameters_Type v_Guti_Params_Cell51 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell51); var float v_T3410plusT3411 := 265.0; <<SKIPPED CODE>> //@siclog "Step 81" siclog@ f_NBIOT_UE_DetachMO(nbiot_Cell50, RRC_CONNECTED, cr_DetachType('?'B, tsc_DetachMO_EpsOnly)); // @sic R5s180638 sic@ // @sic R5s180057 sic@ //@siclog "Step 81Aa1" siclog@ // @sic R5-174464 sic@ SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_508_DETACH_ACCEPT))); <<SKIPPED CODE>> //@siclog "Step 86" siclog@ //The SS initiates Detach procedure with the Detach Type IE "re-attach not required". SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell50, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_508_DETACH_REQUEST_MT(cs_DetachType(tsc_DetachMT_NoReAttach), cs_EMM_Cause_tv(tsc_Cause_EPSServNotAllowed))))); // @sic R5s170770 sic@ //@siclog "Step 87" siclog@ //Check: Does the UE send DETACH ACCEPT message? SRB.receive(car_NB_SRB1bis_NasPdu_IND(nbiot_Cell50, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_DETACH_ACCEPT))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 87"); <<SKIPPED CODE>>

After Change:

function fl_TC_22_5_6_Body() runs on NBIOT_PTC { const NAS_CauseValue_Type tsc_EmmCause_MessageIncompatibleWithProtocolState := '01100101'B; // @sic R5s180246 sic@ //@sic R5-183208 sic@ var template (value) NB_CellPowerList_Type v_CellPowerList; var NAS_MSG_Indication_Type v_NAS_Ind; var template (present) NAS_UL_Message_Type v_AttachReq; var template (present) NAS_UL_Message_Type v_ESMMessage; var template (present) B3_Type v_PDNTypeRxd := (f_GetPdnType(), '101'B); var B3_Type v_PDNType; var GutiParameters_Type v_Guti_Params_Cell50 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell50); var GutiParameters_Type v_Guti_Params_Cell51 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell51); var float v_T3410plusT3411 := 265.0; <<SKIPPED CODE>> //@siclog "Step 81" siclog@ f_NBIOT_UE_DetachMO(nbiot_Cell50, RRC_CONNECTED, cr_DetachType('?'B, tsc_DetachMO_EpsOnly)); // @sic R5s180638 sic@ // @sic R5s180057 sic@ //@siclog "Step 81Aa1" siclog@ // @sic R5-174464 sic@ SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_DETACH_ACCEPT))); //WA# <<SKIPPED CODE>> //@siclog "Step 86" siclog@ //The SS initiates Detach procedure with the Detach Type IE "re-attach not required". //WA# if (pc_USIM_Removal){ SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell50, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_DETACH_REQUEST_MT(cs_DetachType(tsc_DetachMT_NoReAttach), cs_EMM_Cause_tv(tsc_Cause_EPSServNotAllowed))))); // @sic R5s170770 sic@ }else{ SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell50, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_508_DETACH_REQUEST_MT(cs_DetachType(tsc_DetachMT_NoReAttach), cs_EMM_Cause_tv(tsc_Cause_EPSServNotAllowed))))); // @sic R5s170770 sic@ } //@siclog "Step 87" siclog@ //Check: Does the UE send DETACH ACCEPT message? //WA# if (pc_USIM_Removal){ SRB.receive(car_NB_SRB1bis_NasPdu_IND(nbiot_Cell50, cr_NAS_Indication(tsc_SHT_NoSecurityProtection, cr_508_DETACH_ACCEPT))); }else { SRB.receive(car_NB_SRB1bis_NasPdu_IND(nbiot_Cell50, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_DETACH_ACCEPT))); } f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 87"); <<SKIPPED CODE>>

#### MCC160 Implementation

var NB_SRB_COMMON_IND v_ReceivedAsp;

...

if (pc_USIM_Removal) {// @sic R5-174464 sic@

//@siclog "Step 80a1" siclog@ // @sic R5-174464 sic@

f_UT_USIM_Remove(UT, NO_CNF_REQUIRED); // @sic R5s180172 sic@

//The UE will set the SwitchOFF bit to 0 in case of a Normal Detach which happens on USIM removal

//@siclog "Step 81" siclog@

SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell50, ?, cr_NAS_Indication((tsc_SHT_IntegrityProtected, tsc_SHT_IntegrityProtected_Ciphered),

cr_508_DETACH_REQUEST_MO(cr_NAS_KeySetIdentifier (?, ?),

f_GutiParameters2MobileIdentity(omit, v_Guti4_Params),

cr_DetachType('?'B, tsc_DetachMO_EpsOnly))))) -> value v_ReceivedAsp;

// @sic R5s180638 sic@

// @sic R5s180057 sic@

//@siclog "Step 81Aa1" siclog@ // @sic R5-174464 sic@

SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50,

tsc_SRB1bis,

cs_TimingInfo_Now,

cs_NAS_Request(v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.Status,

cs_508_DETACH_ACCEPT)));

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell50); // @sic R5-177054 sic@

//@siclog "Step 82a1" siclog@ // @sic R5-174464 sic@

f_NBIOT_UE_Page_NoReaction(nbiot_Cell50,// @sic R5s180057 sic@

cs_NB_Paging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_Guti1_Params.MME_Code, v_Guti1_Params.M_TMSI)),

30.0);

//@siclog "Step 83a1" siclog@ // @sic R5-174464 sic@

f_UT_USIM_Insert (UT, omit);

}

else {

//@siclog "Step 80b1" siclog@ // @sic R5-174464 sic@

f_UT_SwitchOffUE(UT, true);

//@siclog "Step 81" siclog@

f_NBIOT_UE_DetachMO(nbiot_Cell50, RRC_CONNECTED, cr_DetachType('?'B, tsc_DetachMO_EpsOnly)); // @sic R5s180638 sic@

f_NBIOT_SS_SRBs_DRBs_Reset (nbiot_Cell50, CONTROL_PLANE, delay500ms); // @sic R5s180057 sic@ // @sic R5s180453 sic@

//Step 82 not required here since the UE is switched Off and wont respond to Paging anyway !!

//@siclog "Step 83b1" siclog@ // @sic R5-174464 sic@

f_UT_SwitchOnUE(UT, true);

}

//@siclog "Step 84" siclog@ // @sic R5-174464 sic@

//@siclog "Step 85" siclog@

v_NAS_Ind := fl_TC_22_5_6_ReceiveAttachREQ(nbiot_Cell50);

f_NBIOT_InitialRegistration_Step5_8(nbiot_Cell50,

v_NAS_Ind);

//@siclog "Step 86" siclog@

//The SS initiates Detach procedure with the Detach Type IE "re-attach not required".

SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell50,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_DETACH_REQUEST_MT(cs_DetachType(tsc_DetachMT_NoReAttach), cs_EMM_Cause_tv(tsc_Cause_EPSServNotAllowed))))); // @sic R5s170770 sic@

//@siclog "Step 87" siclog@

//Check: Does the UE send DETACH ACCEPT message?

SRB.receive(car_NB_SRB1bis_NasPdu_IND(nbiot_Cell50,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_DETACH_ACCEPT)));

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 87");

...