3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230123

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4710 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC measurement testcase 8.3.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect implementation of prose CR R5-225419 leads to testcase failure. Report interval of measurement report cannot be calculated after receiving just one measurement report (i.e, first MeasurementReport). Since the variable v_PreviousSFN_1 is initialized with 0, it can lead to incorrect calculation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Report intervals of measurement report is checked only after reception of second measurement report |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport_2Meas_8_3_1_13 |
| --- | --- |
| Reason for change | Incorrect implementation of prose CR R5-225419 leads to testcase failure. Report interval of measurement report cannot be calculated after receiving just one measurement report (i.e, first MeasurementReport). Since the variable v_PreviousSFN_1 is initialized with 0, it can lead to incorrect calculation. |
| Summary of change | Report interval of measurement report is checked only after reception of second measurement report |
| TTCN module | LTE\8_3\RRC_Measurement.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport_2Meas_8_3_1_13(EUTRA_CellId_Type p_CellId,

float p_TotalTimer,

template (present) UL_DCCH_Message p_MeasurementReport_1,

template (present) UL_DCCH_Message p_MeasurementReport_2,

float p_ReportInterval_1 := 1.024,

float p_ReportInterval_2 := 1.024) runs on EUTRA_PTC

{ //@sic R5-225419 sic@

var SRB_COMMON_IND v_ReceivedAsp_1;

var integer v_PreviousSFN_1 := 0;

var integer v_CurrentSFN_1 := 0;

var integer v_SFNLapse_1;

var SRB_COMMON_IND v_ReceivedAsp_2;

var integer v_PreviousSFN_2 := 0;

var integer v_CurrentSFN_2 := 0;

var integer v_SFNLapse_2;

timer t_TotalTimer;

timer t_TimeoutTimer_1;

timer t_TimeoutTimer_2;

t_TotalTimer.start(p_TotalTimer);

t_TimeoutTimer_1.start(5.0);

t_TimeoutTimer_2.start(5.0);

alt {

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, p_MeasurementReport_1)) -> value v_ReceivedAsp_1

{

t_TimeoutTimer_1.stop;

t_TimeoutTimer_1.start(2.0);

v_CurrentSFN_1 := v_ReceivedAsp_1.Common.TimingInfo.SubFrame.SFN.Number;

v_SFNLapse_1 := (v_CurrentSFN_1 - v_PreviousSFN_1) mod 1024;

if ((int2float(v_SFNLapse_1) > p_ReportInterval_1 * 110.0) or   // v_SFNLapse * 10 > p_ReportInterval * 110% * 1000

(int2float(v_SFNLapse_1) < p_ReportInterval_1 * 90.0)) {    // v_SFNLapse * 10 < p_ReportInterval * 90% * 1000

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Abnormal report interval of MeasurementReport_1!");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Preliminary pass: periodical MeasurementReport_1 message received");

}

v_PreviousSFN_1 := v_CurrentSFN_1;

repeat;

}

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, p_MeasurementReport_2)) -> value v_ReceivedAsp_2

{

t_TimeoutTimer_2.stop;

t_TimeoutTimer_2.start(2.0);

v_CurrentSFN_2 := v_ReceivedAsp_2.Common.TimingInfo.SubFrame.SFN.Number;

v_SFNLapse_2 := (v_CurrentSFN_2 - v_PreviousSFN_2) mod 1024;

if ((int2float(v_SFNLapse_2) > p_ReportInterval_2 * 110.0) or   // v_SFNLapse * 10 > p_ReportInterval * 110% * 1000

(int2float(v_SFNLapse_2) < p_ReportInterval_2 * 90.0)) {    // v_SFNLapse * 10 < p_ReportInterval * 90% * 1000

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Abnormal report interval of MeasurementReport_2!");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Preliminary pass: periodical MeasurementReport_2 message received");

}

v_PreviousSFN_2 := v_CurrentSFN_2;

repeat;

}

[] t_TimeoutTimer_1.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Timeout when receiving periodical MeasurementReport_1 message!");

}

[] t_TimeoutTimer_2.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Timeout when receiving periodical MeasurementReport_2 message!");

}

[] t_TotalTimer.timeout

{

t_TimeoutTimer_1.stop;

t_TimeoutTimer_2.stop;

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Periodical MeasurementReport test pass!");

}

}

} //@sic R5-225419 sic@ //end of f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport_2Meas_8_3_1_13

After change:

function f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport_2Meas_8_3_1_13(EUTRA_CellId_Type p_CellId,

float p_TotalTimer,

template (present) UL_DCCH_Message p_MeasurementReport_1,

template (present) UL_DCCH_Message p_MeasurementReport_2,

float p_ReportInterval_1 := 1.024,

float p_ReportInterval_2 := 1.024) runs on EUTRA_PTC

{ //@sic R5-225419 sic@

var SRB_COMMON_IND v_ReceivedAsp_1;

var integer v_PreviousSFN_1 := 0;

var integer v_CurrentSFN_1 := 0;

var integer v_SFNLapse_1;

var boolean v_MeasRpt1stFlag_1 := true;

var SRB_COMMON_IND v_ReceivedAsp_2;

var integer v_PreviousSFN_2 := 0;

var integer v_CurrentSFN_2 := 0;

var integer v_SFNLapse_2;

var boolean v_MeasRpt1stFlag_2 := true;

timer t_TotalTimer;

timer t_TimeoutTimer_1;

timer t_TimeoutTimer_2;

t_TotalTimer.start(p_TotalTimer);

t_TimeoutTimer_1.start(5.0);

t_TimeoutTimer_2.start(5.0);

alt {

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, p_MeasurementReport_1)) -> value v_ReceivedAsp_1

{

t_TimeoutTimer_1.stop;

t_TimeoutTimer_1.start(2.0);

v_CurrentSFN_1 := v_ReceivedAsp_1.Common.TimingInfo.SubFrame.SFN.Number;

if (v_MeasRpt1stFlag_1 == true) {      // the 1st MeasurementReport message

v_MeasRpt1stFlag_1 := false;

}

else {                                 // subsequent MeasurementReport messages

v_SFNLapse_1 := (v_CurrentSFN_1 - v_PreviousSFN_1) mod 1024;

if ((int2float(v_SFNLapse_1) > p_ReportInterval_1 * 110.0) or   // v_SFNLapse * 10 > p_ReportInterval * 110% * 1000

(int2float(v_SFNLapse_1) < p_ReportInterval_1 * 90.0)) {    // v_SFNLapse * 10 < p_ReportInterval * 90% * 1000

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Abnormal report interval of MeasurementReport_1!");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Preliminary pass: periodical MeasurementReport_1 message received");

}

}

v_PreviousSFN_1 := v_CurrentSFN_1;

repeat;

}

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, p_MeasurementReport_2)) -> value v_ReceivedAsp_2

{

t_TimeoutTimer_2.stop;

t_TimeoutTimer_2.start(2.0);

v_CurrentSFN_2 := v_ReceivedAsp_2.Common.TimingInfo.SubFrame.SFN.Number;

if (v_MeasRpt1stFlag_2 == true) {      // the 1st MeasurementReport message

v_MeasRpt1stFlag_2 := false;

}

else {                                 //subsequent MeasurementReport messages

v_SFNLapse_2 := (v_CurrentSFN_2 - v_PreviousSFN_2) mod 1024;

if ((int2float(v_SFNLapse_2) > p_ReportInterval_2 * 110.0) or   // v_SFNLapse * 10 > p_ReportInterval * 110% * 1000

(int2float(v_SFNLapse_2) < p_ReportInterval_2 * 90.0)) {    // v_SFNLapse * 10 < p_ReportInterval * 90% * 1000

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Abnormal report interval of MeasurementReport_2!");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Preliminary pass: periodical MeasurementReport_2 message received");

}

}

v_PreviousSFN_2 := v_CurrentSFN_2;

repeat;

}

[] t_TimeoutTimer_1.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Timeout when receiving periodical MeasurementReport_1 message!");

}

[] t_TimeoutTimer_2.timeout

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Timeout when receiving periodical MeasurementReport_2 message!");

}

[] t_TotalTimer.timeout

{

t_TimeoutTimer_1.stop;

t_TimeoutTimer_2.stop;

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Periodical MeasurementReport test pass!");

}

}

} //@sic R5-225419 sic@ //end of f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport_2Meas_8_3_1_13