3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230126

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2949 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 6.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.523-1 Table 6.1.1.6.3.2-3,UE should send an RRCSetupRequest on NR Cell 13 after 120 seconds, but before 720 seconds in step23. Then in step30, the UE send an RRCSetupRequest on NR Cell 11 after 420 seconds (“MinimumPeriodicSearchTimer”) from step 24. This need 720s+420s= 1140s, the guard timer of 960s may not long enough for some UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the guard timer to 1300s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1  Overview	2

2 Corrections required	2

2.1 Correction to testcase TC_6_1_1_6_NR5GC	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - Correction to testcase TC_6_1_1_6_NR5GC

| Function name | testcase TC_6_1_1_6_NR5GC |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 6.1.1.6.3.2-3,UE should send an RRCSetupRequest on NR Cell 13 after 120 seconds, but before 720 seconds in step23. Then in step30, the UE send an RRCSetupRequest on NR Cell 11 after 420 seconds (“MinimumPeriodicSearchTimer”) from step 24. This need 720s+420s= 1140s, the guard timer of 960s may not long enough for some UE. |
| Summary of change | Change the guard timer to 1300s |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_6_1_1_6_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(960); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_1_1_6_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_6_1_1_6_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_1_1_6_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }