3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230131

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2952 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC testcase 8.2.4.2.1.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add ENDC test case 8.2.4.2.1.2 in FR1 to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to ENDC test case 8.2.4.2.1.2 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody	3

2.2	f_NR_RRC_PSCellChangeCA_InterCell	5

2.3	f_NR_GetPdcpCountInfoListIntraNR_HO	6

2.4	fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody	8

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of ENDC test case 8.2.4.2.1.2 in FR1 which is part of the ENDC test suite in iWD_TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.2.4.2.1.2.ENDC

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon X70 5G Modem-RF system UE

Verification Status:	PASS

# Corrections required

## fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody

| Function name | fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | - The peer signalling message at Step 5 should also have the IE newUE_Identity set to tsc_C_RNTI_Value3 since the local end config has moved to the new C-RNTI and RACH procedure will not be successful if the UE is still on the old C-RNTI - As per the Prose, at Step 5, the condition is EN-DC_PSCell_HO AND RBConfig_NoKeyChange. Hence, the parameter p_RBConfig_KeyChange in function f_NR_RetrieveRadioBearerConfig_PSCellChange should be set to FALSE to communicate to the UE that there is no Key Change. - As per the Prose, at Step 5, the condition is EN-DC_PSCell_HO AND RBConfig_NoKeyChange, this would result in Recover_PDCP being set to TRUE as per TS 36.508 Table 4.6.1-8 However, as per TS 38.323 section 5.4.1: Status reporting Transmit operation For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when: - upper layer requests a PDCP entity re-establishment; - upper layer requests a PDCP data recovery; - upper layer requests a uplink data switching; - upper layer reconfigures the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 38.331 [3]. This would result in the UE sending a PDCP STATUS REPORT and would get at Step 6A as part of the IP Data loopback causing the TC to fail. |
| Summary of change | - The IE newUE_Identity is set to tsc_C_RNTI_Value3 in the RRC Reconfiguration message - The parameter p_RBConfig_KeyChange is set to FALSE - The IE StatusReportRequired is set to OMIT in the PDCP Config |
| TTCN module | RRC_CarrierAggregation_ENDC_NR |
| MCC160 Comment | 1. Accepted 2. Accepted 3. Not needed. PDCP is not configured in transparent mode, the PDCP STATUS REPORT shall not be transmitted to TTCN. |

Before Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? //@sic R5s220809 sic@ f_NR_CellInfo_SetRNTI(p_PSCell2, tsc_C_RNTI_Value3); //@sic R5s221199 R5s221141 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, -, secondary); //@sic R5s220809 sic@ f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); <<SKIPPED CODE>>

After Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? //@sic R5s220809 sic@ f_NR_CellInfo_SetRNTI(p_PSCell2, tsc_C_RNTI_Value3); //@sic R5s221199 R5s221141 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_SCGConfig.spCellConfig.reconfigurationWithSync.newUE_Identity := tsc_C_RNTI_Value3; //WA# v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( false, -, secondary, cs_38508_PDCP_Config(-,-,omit,-)); //@sic R5s220809 sic@ WA# f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); <<SKIPPED CODE>>

## f_NR_RRC_PSCellChangeCA_InterCell

| Function name | f_NR_RRC_PSCellChangeCA_InterCell |
| --- | --- |
| Reason for change | The RACH procedure would be triggered on the Target PSCell due to the reconfigurationWithSync procedure in the peer signalling message at Step 5 and Step 7. Hence, reception of the RACH preamble should be enabled on the Target PSCell. |
| Summary of change | Added the parameter p_TargetPSCellId in call to function f_NR_SendRRCReconfigurationContentsToEUTRA |
| TTCN module | RRC_CarrierAggregation_ENDC_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_RRC_PSCellChangeCA_InterCell(NR_CellId_Type p_SourcePSCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPSCellId, NR_CellId_Type p_TargetSCellId, boolean p_RBConfig_KeyChange := true, template (omit) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_SCGConfig, template (value) NR_RadioBearerList_Type p_RadioBearerToReleaseList ) runs on NR_BASE_PTC { <<SKIPPED CODE>> // Step 9. E-UTRA Cell: Send RRCConnectionReconfiguration. // Step 10. E-UTRA Cell: Receive RRCConnectionReconfigurationComplete. f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, p_SCGConfig, -, -, -, reconfigurationWithSync_CheckRACH); //@sic R5s221199 sic@ <<SKIPPED CODE>>

After Change:

function f_NR_RRC_PSCellChangeCA_InterCell(NR_CellId_Type p_SourcePSCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPSCellId, NR_CellId_Type p_TargetSCellId, boolean p_RBConfig_KeyChange := true, template (omit) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_SCGConfig, template (value) NR_RadioBearerList_Type p_RadioBearerToReleaseList ) runs on NR_BASE_PTC { <<SKIPPED CODE>> // Step 9. E-UTRA Cell: Send RRCConnectionReconfiguration. // Step 10. E-UTRA Cell: Receive RRCConnectionReconfigurationComplete. f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, p_SCGConfig, -, -, -, reconfigurationWithSync_CheckRACH,p_TargetPSCellId); //@sic R5s221199 sic@ //WA# <<SKIPPED CODE>>

## f_NR_GetPdcpCountInfoListIntraNR_HO

| Function name | f_NR_GetPdcpCountInfoListIntraNR_HO |
| --- | --- |
| Reason for change | For the case of RBConfig_NoKeyChange, the DL PDCP SN for the DRB is being incremented un-necessarily, this causes the UE PDCP entity to drop the PDCP SDU packet sent at Step 6A because the received PDCP SN is out of the RX Window. This occurs because the code checks for the trueness of the parameter p_IncrementSRB1 and then just increments the first value in the array assuming it to be SRB1. However, this is not the case for DRB with Recover_PDCP as the parameter v_RadioBearerList in function f_NR_RRC_PSCellChange_InterCell_Steps1To6 has been set to DRB2 |
| Summary of change | Changed the code to ensure that the first entry of the array is an SRB and not a DRB |
| TTCN module | NR_HandoverCommonFunctions |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetPdcpCountInfoListIntraNR_HO(NR_CellId_Type p_SourceCellId, template( value) NR_RadioBearerList_Type p_RadioBearerList, boolean p_RBConfig_KeyChange := true, boolean p_IncrementSRB1 := true //@sic R5-216260 sic@ ) runs on NR_BASE_PTC return NR_PdcpCountInfoList_Type {//@sic R5s220123 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(p_SourceCellId); var integer i; // Acc to 38.300 cl 9.2.3.1: // For DRBs using RLC AM mode, PDCP can either be re-established together with a security key change or initiate a data recovery procedure without a key change. // For DRBs using RLC UM mode and for SRBs, PDCP can either be re-established together with a security key change or remain as it is without a key change if (p_RBConfig_KeyChange) { //No need to retrieve PDCP count for UM and SRBs because they are re-established in the target cell - PDCP SN reset to 0 //Retrieve PDCP count for AM DRBs only for (i := 0; i < lengthof(v_NR_PdcpCountInfoList); i:= i + 1) { if (not(fl_CheckAM_Drb(v_NR_PdcpCountInfoList[i], p_RadioBearerList ))) { v_NR_PdcpCountInfoList[i].DL.Value := int2bit(0,32); v_NR_PdcpCountInfoList[i].UL.Value := int2bit(0,32); } } } else { for (i := 0; i < lengthof(v_NR_PdcpCountInfoList); i:= i + 1) { if (not(fl_CheckAM_Drb(v_NR_PdcpCountInfoList[i], p_RadioBearerList)) and not(fl_CheckSrb(v_NR_PdcpCountInfoList[i])) ) { v_NR_PdcpCountInfoList[i].DL.Value := int2bit(0,32); v_NR_PdcpCountInfoList[i].UL.Value := int2bit(0,32); } } if (p_IncrementSRB1) {//@sic R5-216260 sic@ // DL PDCP count value for SRB1 shall be incremented by 1 because RRCReconfiguration will be sent out on SRB1 of source Cell. //@sic R5s201150 sic@ v_NR_PdcpCountInfoList[0].DL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList[0].DL.Value) + 1), 32); } } return v_NR_PdcpCountInfoList; }

After Change:

function f_NR_GetPdcpCountInfoListIntraNR_HO(NR_CellId_Type p_SourceCellId, template( value) NR_RadioBearerList_Type p_RadioBearerList, boolean p_RBConfig_KeyChange := true, boolean p_IncrementSRB1 := true //@sic R5-216260 sic@ ) runs on NR_BASE_PTC return NR_PdcpCountInfoList_Type {//@sic R5s220123 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(p_SourceCellId); var integer i; // Acc to 38.300 cl 9.2.3.1: // For DRBs using RLC AM mode, PDCP can either be re-established together with a security key change or initiate a data recovery procedure without a key change. // For DRBs using RLC UM mode and for SRBs, PDCP can either be re-established together with a security key change or remain as it is without a key change if (p_RBConfig_KeyChange) { //No need to retrieve PDCP count for UM and SRBs because they are re-established in the target cell - PDCP SN reset to 0 //Retrieve PDCP count for AM DRBs only for (i := 0; i < lengthof(v_NR_PdcpCountInfoList); i:= i + 1) { if (not(fl_CheckAM_Drb(v_NR_PdcpCountInfoList[i], p_RadioBearerList ))) { v_NR_PdcpCountInfoList[i].DL.Value := int2bit(0,32); v_NR_PdcpCountInfoList[i].UL.Value := int2bit(0,32); } } } else { for (i := 0; i < lengthof(v_NR_PdcpCountInfoList); i:= i + 1) { if (not(fl_CheckAM_Drb(v_NR_PdcpCountInfoList[i], p_RadioBearerList)) and not(fl_CheckSrb(v_NR_PdcpCountInfoList[i])) ) { v_NR_PdcpCountInfoList[i].DL.Value := int2bit(0,32); v_NR_PdcpCountInfoList[i].UL.Value := int2bit(0,32); } } if (p_IncrementSRB1 and (not(fl_CheckAM_Drb(v_NR_PdcpCountInfoList[0], p_RadioBearerList)))) {//@sic R5-216260 sic@ WA# // DL PDCP count value for SRB1 shall be incremented by 1 because RRCReconfiguration will be sent out on SRB1 of source Cell. //@sic R5s201150 sic@ v_NR_PdcpCountInfoList[0].DL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList[0].DL.Value) + 1), 32); } } return v_NR_PdcpCountInfoList; }

## fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody

| Function name | fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - As the condition is EN-DC_PSCell_HO AND RBConfig_NoKeyChange at Step 5 and Step 7, the IE sk-Counter-r15 should not be present as per TS 36.508 Table 4.6.1-8. - As there is no key change, the parameter p_SK_Counter in function f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys should not be incremented when calculating the security keys as this would result in Ciphering failure at UE side not being able to decode the PDCP packet sent at Step 6A due to different ciphering keys being used at NW and UE. |
| Summary of change | - sk-Counter-r15 set to omit - parameter p_SK_Counter not incremented so that the same keys are used. |
| TTCN module | RRC_CarrierAggregation_ENDC_EUTRA |
| MCC160 Comment | 1. Accepted 2. Accepted |

Before Change:

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { <<SKIPPED CODE>> //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); <<SKIPPED CODE>>

After Change:

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { <<SKIPPED CODE>> //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR);//WA# f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, omit); //WA# f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); //WA# f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, omit); //WA# f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); <<SKIPPED CODE>>

# Branches executed

## This ENDC TC was executed in Band combination “DC_19A_n78_2A” using EEA1/EIA1 algorithms”.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon X70 5G Modem-RF system UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_8_2_4_2_1_2_ENDC_FR1_QC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230132: Supporting information for addition of ENDC TC 8.2.4.2.1.2 in FR1. |
| --- | --- |