3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230133

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2953 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.3.10 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-01-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.3.10 to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.3.10 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.3.10	4

2.1	fl_TC_11_3_10_TestBody	4

2.2	f_TC_11_3_10_NR5GC_IMS1	6

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm Snapdragon X70 5G Modem-RF System UE	10

5	References	11

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.3.10 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.3.10.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System UE

Verification Status:	PASS

# Corrections required to test case 11.3.10

## fl_TC_11_3_10_TestBody

| Function name | fl_TC_11_3_10_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, after the preamble the Access barring is enabled immediately. However, the Prose is very clear that the IMS registration happens at Steps 1-8, after which the Access barring should be enabled. This needs to be corrected. Also, in the current TTCN implementation, the function f_NR_ModifySysinfo is called after Steps 9A and 11A respectively. This would by default send the paging with state RRC_IDLE, whereas the UE is in connected mode. This also needs to be corrected. |
| Summary of change | Called function f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); before enabling Access barring at Steps 9-9A Instead of calling f_NR_ModifySysinfo(nr_Cell1); called f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); after Steps 9A and 11A respectively. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_11_3_10_TestBody() runs on NR5GC_PTC { //@siclog "Steps 9 - 9A" siclog@ //The SS changes the SIB1 according to 38.508-1[4] Table 11.3.10.3.3-2 and send Short Message on PDCCH using P-RNTI. <-- PDCCH (DCI 1_0): Short Message - - //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(9,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

After Change:

function fl_TC_11_3_10_TestBody() runs on NR5GC_PTC { f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //WA#11_3_10 //@siclog "Steps 9 - 9A" siclog@ //The SS changes the SIB1 according to 38.508-1[4] Table 11.3.10.3.3-2 and send Short Message on PDCCH using P-RNTI. <-- PDCCH (DCI 1_0): Short Message - - //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(9,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); //WA#11_3_10 COMMENTED OUT f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //WA#11_3_10 f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); //WA#11_3_10 COMMENTED OUT f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //WA#11_3_10 f_NR_WaitModificationPeriods(nr_Cell1); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

## f_TC_11_3_10_NR5GC_IMS1

| Function name | f_TC_11_3_10_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation, the timers t_RegExpiremin and t_RegExpireMax are set to 60.0s and 120.0s respectively. At Step 10 delay of 60.0s is called, and after that it is further checked that no re-REGISTER is received until t_RegExpiremin times out. Effectively, it is being checked that the UE is not sending a re-REGISTER for 120s. This is incorrect. When the UE received in 200 OK the “expires” set to 120s, it will start this timer immediately, and after half of it has timed out, the UE could attempt to transmit the re-REGISTER (which is inline with 24.229 cl 5.1.1.3). Thus, after the first 60s are over the UE can try to re-REGISTER. Thus, there is no need to call additional f_Delay(60.0); This needs to be corrected. After the IMS registration is complete (UE has received the 200 OK with “expires” = 120s) and has started this timer, the SS pages the UE for enabling access barring. This takes around approx. 12s. During this, the “expires” is running at the UE side. Thus, when the NR5GC PTC informs the IMS PTC that access barring is enabled, the “expires” has already been running for approx. 12s. Hence, if the timer t_RegExpiremin is set to 60.0s, the testcase will effectively check that the UE does not attempt to send re-REGISTER for 12+60 = 72s, which would be incorrect. Hence it is proposed to set t_RegExpiremin as 50s. The current TTCN also expects that until t_RegExpireMax expires the UE should not send re-REGISTER. This is also incorrect because the UEs are allowed to send a re-REGISTER after half of “expires” times out. The current TTCN also does not check that when half of expires is over, and access barring is still enabled, the UE does not try to send the re-REGISTER. It is proposed to check that after expiry of t_RegExpiremin, for another 20s the UE does not try to transmit the re-REGISTER because of access barring. A Prose CR will be raised to introduce a new step 10A which checks that for 20s after half of “expires” is over the UE does not send re-REGISTER. As a result, it is proposed to set the timer t_RegExpireMax to 70s instead of 120s, and after t_RegExpireMax expires, a coordination message is sent to NR PTC do disable Access barring. The current TTCN calls function v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(); to capture the subsequent re-REGISTER. However, it does not update the NonceCount, and this needs to be addressed. |
| Summary of change | Basically the TTCN has been re-worked. Please see below. A Prose CR will be raised to accommodate Step 10A. |
| TTCN module | UAC_NR5GC_IMS.ttcn |
| MCC160 Comment | Decision postponed until outcome of prose CR at RAN5 |

Before Change:

function f_TC_11_3_10_NR5GC_IMS1()runs on IMS_PTC { var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var REGISTER_Request v_RegisterReq; timer t_RegExpiremin:= 60.0; timer t_RegExpireMax:= 120.0; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(-, -, -, "120000"); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Step 10" siclog@ //Check: Does the UE performs SIP re-registration message in a time span between half of the expiration time and the full expiration time? f_Delay(60.0); t_RegExpiremin.start; t_RegExpireMax.start; alt { [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } [] t_RegExpiremin.timeout { alt { // @sic R5s130266 change 1.2 - MCC160 Implementation sic@ [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } [] t_RegExpireMax.timeout {} } } } //@siclog "Step 12" siclog@ //Check: Does the UE performs SIP REGISTER message? v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //SS responds with 200 OK v_RegisterReq := v_IMS_DATA_REQ.Request.Register; IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change:

function f_TC_11_3_10_NR5GC_IMS1()runs on IMS_PTC { var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var REGISTER_Request v_RegisterReq; timer t_RegExpiremin:= 50.0; //WA#11_3_10 : Run for 50s because 10s are already consumed for Sysinfo Modification enable access barring timer t_RegExpireMax:= 70.0; //WA#11_3_10 : To check that UE does not transmit re-REGISTER for 20s of access barring. f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); v_IMS_DATA_REQ := f_IMS_Registration(-, -, -, "120"); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //WA#11_3_10 f_IMS_IPCAN_WaitForTrigger(IPCAN); //WA#11_3_10 //@siclog "Step 10" siclog@ //Check: Does the UE performs SIP re-registration message in a time span between half of the expiration time and the full expiration time? //WA#11_3_10 COMMENTED OUT f_Delay(60.0); t_RegExpiremin.start; t_RegExpireMax.start; alt { [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } [] t_RegExpiremin.timeout { //WA#11_3_10 New Step 10A alt { [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } [] t_RegExpireMax.timeout { f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //WA#11_3_10 Tell NR PTC to disable Acess barring } } } } //@siclog "Step 12" siclog@ //Check: Does the UE performs SIP REGISTER message? //WA#11_3_10 COMMENTED OUT v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(); v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, 2); //WA#11_3_10 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //SS responds with 200 OK v_RegisterReq := v_IMS_DATA_REQ.Request.Register; IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA2 Integrity and NEA2 ciphering algorithms

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System UE

The Qualcomm Snapdragon X70 5G Modem-RF System UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_11_3_10_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230134: Supporting information for addition of NR5GC testcase 11.3.10 in FR1 |
| --- | --- |