3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230152

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2966 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC UAC test case 11.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case is failing on step5Aa path, as the the RRC Setup Request is not handled. The current TTCN only calls check() on the RRC message before moved on and send rrc setup at step 7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added handling of the RRCSetupRequest message at step step5Aa1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_11_3_7_TestBody |
| --- | --- |
| Reason for change | The test case is failing on step5Aa path, as the the RRC Setup Request is not handled. The current TTCN only calls check() on the RRC message before moved on and send rrc setup at step 7 |
| Summary of change | Added handling of the RRCSetupRequest message at step step5Aa1. |
| TTCN module | UAC_NR5GC.TTCN |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { .................. //@siclog "Step 5" siclog@ //@sic R5-211410, R5-213504, R5s210424, R5-224716 sic@ //Start Timer = 84s (NOTE 2, NOTE 4). //NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention. //NOTE 4: T390 timer value is derived from 38.331[12] T390 = (0.7 + 0.6*rand)*uac-BarringTime where the uac-BarringTime value of 64s is provided in 38.523-1 Table 11.3.7.3.3-1 and where the value of rand is chosen to be 1 as to arrive at the maximum value for T390 of 83.2s rounded up to 84s. //EXCEPTION: Steps 5Aa1-5Ab3 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 5Aa1" siclog@ //@sic R5-224716 sic@ //UE transmits an RRCSetupRequest message with establishmentCause set to 'highPriorityAccess' to establish an emergency call.(NOTE 7) //UE may perform domain selection and stop T390. If UE move back to the NR Cell 1 during 84s timer running, UE may initiate emergency call before 84s timer expire. [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(highPriorityAccess,?)))) { //@siclog "Step 5Aa2" siclog@ //Stop Timer = 84s t_T390.stop; } //@siclog "Step 5Ab1" siclog@ //@sic R5-224716 sic@ //Timer = 84s expires [] t_T390.timeout { //@siclog "Step 5Ab2" siclog@ //Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 5Ab3" siclog@ //UE transmits an RRCSetupRequest message with establishmentCause set to 'highPriorityAccess' to establish an emergency call. f_NR_RRC_SetupRequest_Def(nr_Cell1, highPriorityAccess); } } //@siclog "Step 6" siclog@ .................. }

After Change

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { .................. //@siclog "Step 5" siclog@ //@sic R5-211410, R5-213504, R5s210424, R5-224716 sic@ //Start Timer = 84s (NOTE 2, NOTE 4). //NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention. //NOTE 4: T390 timer value is derived from 38.331[12] T390 = (0.7 + 0.6*rand)*uac-BarringTime where the uac-BarringTime value of 64s is provided in 38.523-1 Table 11.3.7.3.3-1 and where the value of rand is chosen to be 1 as to arrive at the maximum value for T390 of 83.2s rounded up to 84s. //EXCEPTION: Steps 5Aa1-5Ab3 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 5Aa1" siclog@ //@sic R5-224716 sic@ //UE transmits an RRCSetupRequest message with establishmentCause set to 'highPriorityAccess' to establish an emergency call.(NOTE 7) //UE may perform domain selection and stop T390. If UE move back to the NR Cell 1 during 84s timer running, UE may initiate emergency call before 84s timer expire. [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(highPriorityAccess,?)))) { f_NR_RRC_SetupRequest_Def(nr_Cell1, highPriorityAccess);//KS#23xxxx //@siclog "Step 5Aa2" siclog@ //Stop Timer = 84s t_T390.stop; } //@siclog "Step 5Ab1" siclog@ //@sic R5-224716 sic@ //Timer = 84s expires [] t_T390.timeout { //@siclog "Step 5Ab2" siclog@ //Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 5Ab3" siclog@ //UE transmits an RRCSetupRequest message with establishmentCause set to 'highPriorityAccess' to establish an emergency call. f_NR_RRC_SetupRequest_Def(nr_Cell1, highPriorityAccess); } } //@siclog "Step 6" siclog@ .................. }