3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230154

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2968 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.1.2.15a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | STEP 2 onwards UE has to select NR Cell 11 indicating second PLMN in the list but it is found that other parameters on NAS like TAC, Ranac and Cell Identity are not aligned with the corresponding PLMN Identity info parameters of selected tsc_NR_HPLMN_Def, therefore a wrong TAC will be configured in Registration Accept message which can lead a UE to initiate a Mobility registration again as it sees a different TAI info in the received message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected NAS parameters for TAC, Ranac and Cell Identity as per tsc_NR_HPLMN_Def PLMN Identity info parameters for NR Cell 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.15a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	1

2	Corrections required	1

2.1	Correction to the Function f_TC_6_1_2_15a_NR5GC	1

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_6_1_2_15a_NR5GC

| Function name | f_TC_6_1_2_15a_NR5GC |
| --- | --- |
| Reason for change | - STEP 2 onwards UE has to select NR Cell 11 indicating second PLMN in the list but it is found that other parameters on NAS like TAC, Ranac and Cell Identity are not aligned with the corresponding PLMN Identity info parameters of selected tsc_NR_HPLMN_Def, therefore a wrong TAC will be configured in Registration Accept message which can lead a UE to initiate a Mobility registration again as it sees a different TAI info in the received message. |
| Summary of change | - Corrected NAS parameters for TAC, Ranac and Cell Identity as per tsc_NR_HPLMN_Def PLMN Identity info parameters for NR Cell 11. |
| MCC160 Comment | Accepted in principle but implemented differently to be aligned with MCC160’s implementation of R5s230031. See MCC TF160 implementation with R5s230031 and additions from R5s230154 below. |

Before Change:

function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities var template(omit) PLMN_IdentityInfoList v_PLMN_IdentityInfoList := omit; //WA#WI=1041047 f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 //f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_001_02); // Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 //f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_Def, 0, 1); //WA#WI=1041047 v_PLMN_IdentityInfoList := { cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_001_02, int2bit(2,24), 2, '00000000000000000000000110'B & '0000001011'B), cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_Def, int2bit(3,24), // TAI ID to be used is 3 1, '00000000000000000000000101'B & '0000001010'B ) } var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell11); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := tsc_NR_HPLMN_Def; v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList := valueof(v_PLMN_IdentityInfoList); f_NR_CellInfo_Set(nr_Cell11, v_NR_CellInfo); // Table 6.1.2.15a.3.3-2: RAN Area Code //f_NR_CellInfo_SetRANAC(nr_Cell11, 2); //f_NR_CellInfo_SetRANAC(nr_Cell11, 1, 1); // Table 6.1.2.15a.3.3-2: cellIdentity //f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000110'B & '0000001011'B); // f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000101'B & '0000001010'B, 1); // Table 6.1.2.15a.3.3-2: trackingAreaCode //f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(2,24)); //f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(3,24),1); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -82); // WA#WI=1041047 Max Power level - Table 6.1.2.15.3.2-1: // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC

After Change:

function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities var template(omit) PLMN_IdentityInfoList v_PLMN_IdentityInfoList := omit; //WA#WI=1041047 f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 //f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_001_02); // Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 //f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_Def, 0, 1); //WA#WI=1041047 v_PLMN_IdentityInfoList := { cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_001_02, int2bit(2,24), 2, '00000000000000000000000110'B & '0000001011'B), cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_Def, int2bit(3,24), // TAI ID to be used is 3 1, '00000000000000000000000101'B & '0000001010'B ) } var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell11); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := tsc_NR_HPLMN_Def; v_NR_CellInfo.NAS_Parameters.TrackingAreaCode := int2bit(3,24); //WA#WI=1056682 v_NR_CellInfo.NAS_Parameters.Ranac := 1; v_NR_CellInfo.NAS_Parameters.CellIdentity := '00000000000000000000000101'B & '0000001010'B ; v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList := valueof(v_PLMN_IdentityInfoList); f_NR_CellInfo_Set(nr_Cell11, v_NR_CellInfo); // Table 6.1.2.15a.3.3-2: RAN Area Code //f_NR_CellInfo_SetRANAC(nr_Cell11, 2); //f_NR_CellInfo_SetRANAC(nr_Cell11, 1, 1); // Table 6.1.2.15a.3.3-2: cellIdentity //f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000110'B & '0000001011'B); // f_NR_CellInfo_SetCellIdentity(nr_Cell11, '00000000000000000000000101'B & '0000001010'B, 1); // Table 6.1.2.15a.3.3-2: trackingAreaCode //f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(2,24)); //f_NR_CellInfo_SetTAC (nr_Cell11, int2bit(3,24),1); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -82); // WA#WI=1041047 Max Power level - Table 6.1.2.15.3.2-1: // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC

MCC160 implementation:

/* * @desc REFERENCE TS 38.523-1 clause 6.1.2.15a * @status */ function f_TC_6_1_2_15a_NR5GC() runs on NR5GC_PTC {// Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities //@sic R5s230031 sic@ var template (value) PLMN_IdentityInfoList v_PLMN_IdentityInfoList; var NR_CellInfo_Type v_NR_CellInfo; f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR Cells // Set Sys info and cell info for cell 1 & 11 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); // Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 v_PLMN_IdentityInfoList := { cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_001_02, int2bit(2,24), 2, '00000000000000000000000110'B & '0000001011'B), cs_NR_PLMN_IdentityInfo(tsc_NR_HPLMN_Def, int2bit(3,24), // TAI ID 1, '00000000000000000000000101'B & '0000001010'B)}; f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell11, v_PLMN_IdentityInfoList );// Table 6.1.2.15a.3.3-2: SIB1s for NR Cell 11 v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell11); v_NR_CellInfo.NAS_Parameters.TrackingAreaCode := int2bit(3,24); v_NR_CellInfo.NAS_Parameters.Ranac := 1; v_NR_CellInfo.NAS_Parameters.CellIdentity := '00000000000000000000000101'B & '0000001010'B ; f_NR_CellInfo_Set(nr_Cell11, v_NR_CellInfo); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); // update for T0 due to Table 6.1.2.15a.3.2-2 (FR2) f_NR_TestBody_Set(true); f_TC_6_1_2_15and15a_NR5GC_TestBody(); // Acc. to cl. 6.1.2.15a.3.2, Test procedure sequence is same as TC 6.1.2.15.3.2 f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); } //end of f_TC_6_1_2_15a_NR5GC