3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230160

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0896 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS LTE-IRAT test case 19.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the step 5a3 of testcase 19.3.2b, the CS call is released and it is expected that the UE triggers a RRC connection Request procedure to perform Routing Area update procedure. In function f_UTRAN_RRC_ConnRel, it calls f_UTRAN_RestartCRLC_ForNextConnection to perform SS RLC reset procedures. It is possible rrcConnectionRequest is received during this function call which would result in being handled in the default handler and this will result in inconclusive behavior of the testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Parameter ‘p_ConnEstFollows’ is set to true to temporarily block the U_TM port |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_19_3_2b_UTRAN |
| --- | --- |
| Reason for change | During the step 5a3 of testcase 19.3.2b, the CS call is released and it is expected that the UE triggers a RRC connection Request procedure to perform Routing Area update procedure. In function f_UTRAN_RRC_ConnRel, it calls f_UTRAN_RestartCRLC_ForNextConnection to perform SS RLC reset procedures. It is possible rrcConnectionRequest is received during this function call which would result in being handled in the default handler and this will result in inconclusive behavior of the testcase. |
| Summary of change | Parameter ‘p_ConnEstFollows’ is set to true to temporarily block the U_TM port |
| TTCN module | IMS_IRAT\19\IMS_IRAT_19_UTRAN.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_TC_19_3_2b_UTRAN() runs on UTRAN_IRAT_PTC

{ /* Non-UE detectable emergency call / IM CN sends a 380 with unavailable emergency service URN / UE performs normal call via CS domain / UTRAN or GERAN */

f_IMS_CSFB_UTRAN_Init ();    //Initialise test case  @sic R5s160070 sic@

f_UTRAN_TestBody_Set(true);

//@siclog "Step 9" siclog@

f_UTRAN_CS_Fallback_WithRedirection(utran_Cell5, false); // new in 19_3_2b

//@siclog "Step 10" siclog@

//The SS releases the UE connection

f_UTRAN_CC_Disconnect();

f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch);

f_UTRAN_RRC_ConnEst(utran_Cell5);

f_UTRAN_GMM_RAU(utran_Cell5, false);

f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch);

f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-183106 sic@

f_UTRAN_TestBody_Set(false);

// Switch off/power off and release cells

// f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@

f_UTRAN_Postamble(utran_Cell5, U1_IDLE);

// send coordination message for end of test case

f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger);

}

After change:

function f_TC_19_3_2b_UTRAN() runs on UTRAN_IRAT_PTC

{ /* Non-UE detectable emergency call / IM CN sends a 380 with unavailable emergency service URN / UE performs normal call via CS domain / UTRAN or GERAN */

f_IMS_CSFB_UTRAN_Init ();    //Initialise test case  @sic R5s160070 sic@

f_UTRAN_TestBody_Set(true);

//@siclog "Step 9" siclog@

f_UTRAN_CS_Fallback_WithRedirection(utran_Cell5, false); // new in 19_3_2b

//@siclog "Step 10" siclog@

//The SS releases the UE connection

f_UTRAN_CC_Disconnect();

f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch, true);

f_UTRAN_RRC_ConnEst(utran_Cell5);

f_UTRAN_GMM_RAU(utran_Cell5, false);

f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch);

f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-183106 sic@

f_UTRAN_TestBody_Set(false);

// Switch off/power off and release cells

// f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@

f_UTRAN_Postamble(utran_Cell5, U1_IDLE);

// send coordination message for end of test case

f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger);

}