3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230167

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2979 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation of function f_TC_7_1_3_5_2_NR5GC in 22wk49 , - SCG DRB is reconfigured in PDCP Test mode before the testbody . but this implementation can be improved by configuring SCG DRB in test mode during the preamble itself when SCG cell is configured . - To be inline with other PDCP Test mode cases , it is also proposed to make use of the function f_NR_AS_CipheringAlgorithm_Set(nea0); before calling the function f_NR5GC_RRC_ConnectedState3N_NRDC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SCG DRB is configured in PDCP Test mode when SCG configurations are added as part of the function f_NR5GC_RRC_ConnectedState3N_NRDC - To be in line with other PDCP TM mode cases , f_NR_AS_CipheringAlgorithm_Set(nea0); is called before calling the function f_NR5GC_RRC_ConnectedState3N_NRDC() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_7_1_3_5_2_NR5GC()	3

2.2	Correction to the Function f_NR5GC_RRC_ConnectedState3N_NRDC()	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_3_5_2_NR5GC()

| Function name | f_TC_7_1_3_5_2_NR5GC() |
| --- | --- |
| Reason for change | In the current TTCN Implementation of function f_TC_7_1_3_5_2_NR5GC in 22wk49 , - SCG DRB is reconfigured in PDCP Test mode before the testbody . but this implementation can be improved by configuring SCG DRB in test mode during the preamble itself when SCG cell is configured . - To be line with other PDCP Test mode cases , it is also proposed to make use of the function f_NR_AS_CipheringAlgorithm_Set(nea0); before calling the function f_NR5GC_RRC_ConnectedState3N_NRDC. |
| Summary of change | - SCG DRB is configured in PDCP Test mode when SCG configurations are added as part of the function f_NR5GC_RRC_ConnectedState3N_NRDC - To be in line with other PDCP TM mode cases , f_NR_AS_CipheringAlgorithm_Set(nea0); is called before calling the function f_NR5GC_RRC_ConnectedState3N_NRDC() |
| TTCN module | NR5GC\7_1_3\PDCP_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted in principle. Implemented as per R5s230377 change 4. 2. Rejected. not needed as per R5-232206 |

Before Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var integer v_SCG_DRB_Id; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE with TEST mode A on @sic R5s211471 sic@ f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100,- , - ,tsc_NR_DRB3)); //@sic R5s210888 sic@ => Will bring UE to connected state, The SCG DRB is configured in normal mode v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18, v_SCG_DRB_Id); //@sic R5s210888 sic@ //Reconfigure SCG DRB PDCP in Test Mode in SS v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); fl_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_scg; //WA#WI=937281 var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG; //WA#WI=937281 var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG; //WA#WI=937281 var integer v_SCG_DRB_Id; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE with TEST mode A on @sic R5s211471 sic@ f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ f_NR_AS_CipheringAlgorithm_Set(nea0); //WA#WI=937281 //WA#WI=937281 configure DRB with PDCP in TM at the start v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); //WA#WI=937281 v_SS_ConfigMCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_Proxy(cs_RlcBearerRouting_NR(nr_Cell10)), cs_NR_RlcBearerConfig_AM(false, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), cs_SDAP_Configuration_None)}; //WA#WI=937281 v_SS_ConfigSCG := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode,-,cs_SDAP_Configuration_None)}; //WA#WI=937281 f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100,- , - ,tsc_NR_DRB3),v_SS_ConfigMCG,v_SS_ConfigSCG); //@sic R5s210888 sic@ => Will bring UE to connected state, The SCG DRB is configured in normal mode //WA#WI=937281 v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18, v_SCG_DRB_Id); //@sic R5s210888 sic@ //Reconfigure SCG DRB PDCP in Test Mode in SS //v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; //WA#WI=937281 //f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); //WA#WI=937281 f_NR_TestBody_Set(true); fl_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

## Correction to the Function f_NR5GC_RRC_ConnectedState3N_NRDC()

| Function name | f_NR5GC_RRC_ConnectedState3N_NRDC() |
| --- | --- |
| Reason for change | - In the current TTCN implementation , SCG DRB is reconfigured in PDCP Test mode before the testbody . but this implementation can be improved by configuring SCG DRB in test mode during the preamble itself when SCG cell is configured. This can be achieved by passing SCG DRB configurations in the function f_NR5GC_RRC_ConnectedState3N_NRDC(). - Definition of f_NR_RRCReconfig_AddSCG_DRBs_NRDC is already having this place holder for handling SCG DRB configurations |
| Summary of change | - SCG DRB is configured in PDCP Test mode when SCG configurations are added as part of the function f_NR5GC_RRC_ConnectedState3N_NRDC |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted Implemented as per R5s230377 change 6. |

Before Change:

function f_NR5GC_RRC_ConnectedState3N_NRDC(NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, NRDC_DRB_Configuration_Type p_NRDC_DRB_Type, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) PDCP_Config p_PDCP_ConfigSCG_DRB := cs_38508_PDCP_Config ) runs on NR5GC_PTC { //@sic R5-205332 R5-605339 R5-606298 R5s211470 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template(omit) MAC_CellGroupConfig v_MAC_CellGroupConfig := omit; if (f_NR_TestcaseIsL2Testcase()) { //@sic R5s210315 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCG_CellId); f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_MCG_CellId); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_MCG_CellId); } //Steps 1-8: Same as table 4.5.4.2-3, steps 1-8. f_NR_Paging_Def(p_MCG_CellId, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_MCG_CellId); f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_MCG_CellId, v_ReceivedMsg, -, v_MAC_CellGroupConfig, -, -, -, -); //Reconfigure SRB2 + MCG DRBs as per default // Steps 8A to 8C // The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add a new SCG DRB or a new split DRB // The UE transmits an RRCReconfigurationComplete message including nr-SCG-Response. // The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. f_NR_RRCReconfig_AddSCG_DRBs_NRDC(p_MCG_CellId, p_SCG_CellId, p_NRDC_DRB_Type, p_PDCP_ConfigSCG_DRB); // Step 9a1 - 9a2 //IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_MCG_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime); } } case else { // do nothing } } }

After Change:

function f_NR5GC_RRC_ConnectedState3N_NRDC(NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, NRDC_DRB_Configuration_Type p_NRDC_DRB_Type, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) PDCP_Config p_PDCP_ConfigSCG_DRB := cs_38508_PDCP_Config, template(omit) NR_RadioBearerList_Type p_SS_ConfigMCG := omit,//WA#WI=937281 template(omit) NR_RadioBearerList_Type p_SS_ConfigSCG := omit //WA#WI=937281 ) runs on NR5GC_PTC { //@sic R5-205332 R5-605339 R5-606298 R5s211470 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template(omit) MAC_CellGroupConfig v_MAC_CellGroupConfig := omit; if (f_NR_TestcaseIsL2Testcase()) { //@sic R5s210315 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCG_CellId); f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_MCG_CellId); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_MCG_CellId); } //Steps 1-8: Same as table 4.5.4.2-3, steps 1-8. f_NR_Paging_Def(p_MCG_CellId, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_MCG_CellId); f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_MCG_CellId, v_ReceivedMsg, -, v_MAC_CellGroupConfig, -, -, -, -); //Reconfigure SRB2 + MCG DRBs as per default // Steps 8A to 8C // The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add a new SCG DRB or a new split DRB // The UE transmits an RRCReconfigurationComplete message including nr-SCG-Response. // The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. f_NR_RRCReconfig_AddSCG_DRBs_NRDC(p_MCG_CellId, p_SCG_CellId, p_NRDC_DRB_Type, p_PDCP_ConfigSCG_DRB,p_SS_ConfigMCG,p_SS_ConfigSCG); //WA#WI=937281 // Step 9a1 - 9a2 //IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_MCG_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime); } } case else { // do nothing } } }