3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230172

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2982 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.4.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - T0 power level for NR Cell1 is not being configured as per Table 8.1.6.1.4.7.3.2-1. - Caling f_NR_CellInfo_InitMaxReferencePower() for NR Cell 1 after preamble is not required as this is being called earlier before creation of cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Configured NR Cell1 power as -85 as per T0, Table 8.1.6.1.4.7.3.2-1 - Deleted f_NR_CellInfo_InitMaxReferencePower() call after preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_8_1_6_1_4_7_NR5GC	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Correction to the Function f_TC_8_1_6_1_4_7_NR5GC

| Function name | f_TC_8_1_6_1_4_7_NR5GC |
| --- | --- |
| Reason for change | 1. T0 power level for NR Cell1 is not being configured as per Table 8.1.6.1.4.7.3.2-1. 2. Caling f_NR_CellInfo_InitMaxReferencePower() for NR Cell 1 after preamble is not required as this is being called earlier before creation of cells. |
| Summary of change | 1. Configured NR Cell1 power as -85 as per T0, Table 8.1.6.1.4.7.3.2-1. 2. Deleted f_NR_CellInfo_InitMaxReferencePower() call after preamble. |
| TTCN Module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_6_1_4_7_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_4); //System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2. f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A);//The UE is in state 1N-A as defined in TS 38.508-1, subclause 4.4A, on cell 1 f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_TestBody_Set(true); f_TC_8_1_6_1_4_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } // End of f_TC_8_1_6_1_4_7_NR5GC

After Change:

function f_TC_8_1_6_1_4_7_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_4); //System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2. f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A);//The UE is in state 1N-A as defined in TS 38.508-1, subclause 4.4A, on cell 1 //COMMENTED OUT f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 // WA#WI=1061106 f_NR_SetCellPower (nr_Cell1, -85, -85); // update for T0 acc. to Table 8.1.6.1.4.7.3.2-1 // WA#WI=1061106 f_NR_TestBody_Set(true); f_TC_8_1_6_1_4_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } // End of f_TC_8_1_6_1_4_7_NR5GC