3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230190

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0897 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS XCAP test case 15.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for the case of 15.10 a device may activate rules for CFNRc (not reachable) only inconjunction with CFNL (not logged in). In this case the condition to be checked may be in the 2nd rule in a ruleset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For CFNL service allow that additional rules may also have been activated during the service activation. Note : An associated prose CR R5-230876 on 34.229-1 is raised at RAN5#98 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformany UE may not pass this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-1 R5-230876 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_SupplementaryService_Check() |
| --- | --- |
| Reason for change | It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for the case of 15.10 a device may activate rules for CFNRc (not reachable) only inconjunction with CFNL (not logged in). In this case the condition to be checked may be in the 2nd rule in a ruleset. |
| Summary of change | For CFNL service allow that additional rules may also have been activated during the service activation. Note : An associated prose CR R5-230876 on 34.229-1 is raised at RAN5#98 |
| TTCN module | IMS_XCAP_Simserv.ttcn |
| MCC160 Comment | Accepten in principle; implemented according to prose CR R5-230876: The XCAP call forwarding test cases in 34.229-1 and 34.229-5 except 34.229-1 test case 15.10 specify one <cp:rule> element to be contained in the ruleset. R5-230876 allows one additional rule for test case 15.10 to activate/deactivate CFNRc in addition to CFNL. As R5-232010 has been withdrawn the changes shall be for 4G test case only but not for 5G test case 8.9. |

Before Change:

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNL, SS_CFNRc) { v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK;

After Change:

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; var integer i; var boolean v_Rulecheck; var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNRc) { v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); } case (SS_CFNL) { v_RulesetOK := false; v_TargetAddress := px_XCAP_TargetUri; for (i := 0; i < lengthof(p_Ruleset.rule_list); i := i + 1) { // Change for supporting multiple ruleset reception v_Rule := p_Ruleset.rule_list[i]; fl_IMS_XCAP_CheckRuleId(v_Rule); v_Rulecheck := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); if(v_Rulecheck){ v_RulesetOK := true; } } } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK; }

## MCC160 Implementation

type record SupplementaryServiceInfo_Rule_Type { /* to simplify checks */ RuleType SimservsRule, // urn_ietf_params_xml_ns_common_policy.Ruleset SetOfCharString_Type ConditionNames // list of condition names corresponding to the conditions of the Rule }; type record of SupplementaryServiceInfo_Rule_Type SupplementaryServiceInfo_ListOfRules_Type;

type record SupplementaryServiceInfo_Type { /* to simplify checks */ /* @status APPROVED (IMS) */ AbsService_group service, boolean active, SupplementaryServiceInfo_ListOfRules_Type ListOfRules };

template (value) SupplementaryServiceInfo_Type cs_SupplementaryService_Info(AbsService_group p_Service, boolean p_Active, SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules) := { /* @status APPROVED (IMS) */ service := p_Service, active := p_Active, ListOfRules := p_ListOfRules };

function fl_SupplementaryServiceInfo_ListOfRules(Ruleset p_Ruleset) return SupplementaryServiceInfo_ListOfRules_Type { var SupplementaryServiceInfo_ListOfRules_Type v_ListOfRules := {}; var RuleType v_SimservsRule; var integer i; for (i := 0; i < lengthof(p_Ruleset.rule_list); i := i + 1) { v_SimservsRule := p_Ruleset.rule_list[i]; v_ListOfRules[i].SimservsRule := v_SimservsRule; v_ListOfRules[i].ConditionNames := fl_Rule_GetCommonPolicyConditions(v_SimservsRule); } return v_ListOfRules; }

function f_Simservs_GetSpecificService(Simservs p_Simservs, SS_Type p_SupplementaryService) return SupplementaryServiceInfo_Type { var SupplementaryServiceInfo_ListOfRules_Type v_ListOfRules := {}; var AbsService_group v_AbsService; var boolean v_ServiceIsActive; var template boolean v_ServiceActiveTemplate := true ifpresent; /* @sic R5-165883: service needs to be considered to be active too when "active" attribute is absent sic@ */ select (p_SupplementaryService) { case (SS_CLIP) { // 15.1, 8.1.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Originating_identity_presentation); v_ServiceIsActive := match(v_AbsService, cr_Originating_identity_presentation(v_ServiceActiveTemplate)); } case (SS_CLIR) { // 15.2, 8.2.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Originating_identity_presentation_restriction); v_ServiceIsActive := match(v_AbsService, cr_Originating_identity_presentation_restriction(v_ServiceActiveTemplate)); } case (SS_COLP) { // 15.3, 8.4.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Terminating_identity_presentation); v_ServiceIsActive := match(v_AbsService, cr_Terminating_identity_presentation(v_ServiceActiveTemplate)); } case (SS_TIR) { // 15.4, 8.5.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Terminating_identity_presentation_restriction); v_ServiceIsActive := match(v_AbsService, cr_Terminating_identity_presentation_restriction(v_ServiceActiveTemplate)); } case (SS_CFU, // 15.5, 8.7.NR5GC SS_CFNRy, // 15.7, 8.15.NR5GC SS_CFB, // 15.9, 8.11.NR5GC SS_CFNL, // 15.10, 8.9.NR5GC SS_CFNRc) { // 15.10a, 8.13.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Communication_diversion); v_ServiceIsActive := match(v_AbsService, cr_Communication_diversion(v_ServiceActiveTemplate)); v_ListOfRules := fl_SupplementaryServiceInfo_ListOfRules(v_AbsService.communication_diversion.ruleset); } case (SS_BAIC, // 8.17.NR5GC SS_BAIC_ANONYMOUS, // 15.14, 8.19.NR5GC SS_BAIC_EXCEPT_SPECIFIC_USER, // 15.13, 8.18.NR5GC SS_BIC_ROAM) { // 15.14a, 8.25.NR5GC @sic R5-153995 sic@ v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Incoming_communication_barring); v_ServiceIsActive := match(v_AbsService, cr_Incoming_communication_barring(v_ServiceActiveTemplate)); v_ListOfRules := fl_SupplementaryServiceInfo_ListOfRules(v_AbsService.incoming_communication_barring.ruleset); } case (SS_BARRING_OUT_ROAM, // 15.14b, 8.24. SS_BAOC, // 8.21.NR5GC SS_BOIC, // 8.22.NR5GC SS_BOIC_EXHC) { // 8.23.NR5GC v_AbsService := fl_Simservs_GetService(p_Simservs, cr_Outgoing_communication_barring); v_ServiceIsActive := match(v_AbsService, cr_Outgoing_communication_barring(v_ServiceActiveTemplate)); v_ListOfRules := fl_SupplementaryServiceInfo_ListOfRules(v_AbsService.outgoing_communication_barring.ruleset); } case else { FatalError(__FILE__, __LINE__, "unknown supplementary service"); } } return valueof(cs_SupplementaryServiceInfo_Init(v_AbsService, v_ServiceIsActive, v_ListOfRules)); // @sic R5-216570..82 - NOTE: there are no multiple services anymore sic@ }

function fl_ListOfRules_GetRule(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules, integer p_RuleCntMax := 1, template (present) charstring p_ConditionName := ?, template (present) charstring p_RuleId := tsc_XCAP_RuleId_Def) runs on IMS_PTC return SupplementaryServiceInfo_Rule_Type { var integer v_RuleCnt := lengthof(p_ListOfRules); var SupplementaryServiceInfo_Rule_Type v_Rule; var integer i; if (v_RuleCnt > p_RuleCntMax) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "too many rules"); } if (v_RuleCnt == 1) { // @sic R5-216570..82: in general the ruleset has only one rule sic@ v_Rule := p_ListOfRules[0]; } else { // @sic R5-230876: SS_CFNL may come along with SS_CFNRy sic@ for (i := 0; i < v_RuleCnt; i := i + 1) { if (match(p_ListOfRules[i].ConditionNames, superset(p_ConditionName))) { v_Rule := p_ListOfRules[i]; break; } } if (not isbound(v_Rule)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "no rule found for expected condition"); } } if (f_IMS_PTC_RanTypeIsNR5GC() and not match(v_Rule.SimservsRule.id, p_RuleId)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "invalid rule id"); } return v_Rule; }

function f_SupplementaryService_Check(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var integer v_RuleCntMax := 1; var template (present) charstring v_ConditionName := ?; var SupplementaryServiceInfo_Rule_Type v_Rule; var RuleType v_SimservsRule; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNL, SS_CFNRc) { if (not f_IMS_PTC_RanTypeIsNR5GC() and (p_SupplementaryService == SS_CFNL)) { // @sic R5-230876: SS_CFNL may come along with SS_CFNRy sic@ v_RuleCntMax := 2; v_ConditionName := "not-registered"; } v_Rule := fl_ListOfRules_GetRule(p_ListOfRules, v_RuleCntMax, v_ConditionName); v_SimservsRule := v_Rule.SimservsRule; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_SimservsRule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_SimservsRule)); if (v_RulesetOK and (v_RuleCntMax > 1)) { // @sic R5-230876: SS_CFNL may come along with SS_CFNRy sic@ v_Rule := fl_ListOfRules_GetRule(p_ListOfRules, 2, "not-reachable", ?); v_SimservsRule := v_Rule.SimservsRule; v_RulesetOK := (fl_Rule_CheckConditions(SS_CFNRy, v_SimservsRule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_SimservsRule)); } } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := fl_ListOfRules_GetRule(p_ListOfRules); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule.SimservsRule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_ListOfRules) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_ListOfRules) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_ListOfRules))); } } return v_RulesetOK; }

function f_Ruleset_CheckDeactivated(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules, boolean p_RulesetDeactivated) return boolean { // @sic R5-216570..82: Cleanup for IMSoNR5GC XCAP test cases sic@ var SetOfCharString_Type v_Conditions; var boolean v_RuleDeactivated; var integer i; for (i := 0; i < lengthof(p_ListOfRules); i := i + 1) { v_Conditions := p_ListOfRules[i].ConditionNames; v_RuleDeactivated := match(v_Conditions, superset("rule-deactivated")); if (v_RuleDeactivated != p_RulesetDeactivated) { return false; } } return true; }

function fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules, Ruleset p_Ruleset, XSD.AnyURI p_UserId := px_XCAP_TargetUri) return boolean { var integer i; var RuleType v_Rule; for (i := 0; i < lengthof(p_ListOfRules); i := i + 1) { v_Rule := p_ListOfRules[i].SimservsRule; if (fl_BarringRule_CheckAllow(v_Rule, false) and fl_BarringRule_CheckExceptSpecificUser(v_Rule, p_UserId)) { return true; } } return false; }

function fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules, XSD.AnyURI p_UserId := px_XCAP_TargetUri) return boolean { var integer i; var RuleType v_Rule; for (i := 0; i < lengthof(p_ListOfRules); i := i + 1) { v_Rule := p_ListOfRules[i].SimservsRule; if (fl_BarringRule_CheckAllow(v_Rule, true) and fl_BarringRule_CheckOneSpecificUser(v_Rule, p_UserId)) { return true; } } return false; }

function fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(SupplementaryServiceInfo_ListOfRules_Type p_ListOfRules ) return boolean { var integer i; var RuleType v_Rule; for (i := 0; i < lengthof(p_ListOfRules); i := i + 1) { v_Rule := p_ListOfRules[i].SimservsRule; if (fl_BarringRule_CheckAllow(v_Rule, false) and fl_BarringRule_CheckOtherIdentity(v_Rule)) { return true; } } return false; }