3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230199

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2988 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.3.5.4 for a Redcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RedCap UE use 12bit SN for PDCP and as per TS 38.323 reordering window size value equals to 2 ^[pdcp-SN-SizeDL] – 1. Thus, for a redcap UE this needs to be configured as 2048 at Step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added pc_supportOfRedCap_r17 pic based check to configure reordering window size. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-231160 is raised for RAN5#98 Prose CR R5-231533 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_7_1_3_5_4_NR_Step1to17	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_3_5_4_NR_Step1to17

| Function name | f_TC_7_1_3_5_4_NR_Step1to17 |
| --- | --- |
| Reason for change | 1. RedCap UE use 12bit SN for PDCP and as per TS 38.323 reordering window size value equals to 2 ^[pdcp-SN-SizeDL] – 1. Thus, for a redcap UE this needs to be configured as 2048 at Step 1. |
| Summary of change | 1. Added pc_supportOfRedCap_r17 pic based check to configure reordering window size. Note: A prose CR will be raised for this correction. |
| Module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_7_1_3_5_4_NR_Step1to17(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_SubFrameTimingT1; var SubFrameTiming_Type v_SubFrameTimingT2; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var float v_T_reordering; var TimingInfo_Type v_TimingInfo; var integer v_Duration; timer t_Wait := 1.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc ; //@siclog "Step 1" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU #131072 p_NR_PDCP_State.TX_NEXT := 131072; //@sic R5s190227 sic@ f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData); //@siclog "Step 2" siclog@ //Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_Wait.timeout{} } //@siclog "Step 3" siclog@ //The SS sends the PDCP SDU #1 p_NR_PDCP_State.TX_NEXT := 1; <<Skipped Code>> }

After Change:

function f_TC_7_1_3_5_4_NR_Step1to17(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_SubFrameTimingT1; var SubFrameTiming_Type v_SubFrameTimingT2; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var float v_T_reordering; var TimingInfo_Type v_TimingInfo; var integer v_Duration; timer t_Wait := 1.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc ; //@siclog "Step 1" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU #131072 if(pc_supportOfRedCap_r17) //#WA#WI1059756 { p_NR_PDCP_State.TX_NEXT := 2048; //should be 2048 } else{ p_NR_PDCP_State.TX_NEXT := 131072; //@sic R5s190227 sic@ } f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData); //@siclog "Step 2" siclog@ //Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_Wait.timeout{} } //@siclog "Step 3" siclog@ //The SS sends the PDCP SDU #1 p_NR_PDCP_State.TX_NEXT := 1; <<Skipped Code>> }