3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230222

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2994 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcases 7.1.1.4.2.X for a Redcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - TBS size in function fl_NR_GetPdcpSduList_1TB is not implemented as per TS 38.523-1, Table 7.1.1.4.2.5.3.2-2A (Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE) requirements for a Redcap UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated function fl_NR_GetPdcpSduList_1TB as per TS 38.523-1, Table 7.1.1.4.2.5.3.2-2A requirements for a Redcap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function fl_NR_GetPdcpSduList_1TB	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_NR_GetPdcpSduList_1TB

| Function name | fl_NR_GetPdcpSduList_1TB |
| --- | --- |
| Reason for change | 1. TBS size in function fl_NR_GetPdcpSduList_1TB is not implemented as per TS 38.523-1, Table 7.1.1.4.2.5.3.2-2A(Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE) requirements for a Redcap UE. |
| Summary of change | 1. Updated function fl_NR_GetPdcpSduList_1TB as per TS 38.523-1, Table 7.1.1.4.2.5.3.2-2A requirements for a Redcap UE. |
| TTCN module | \NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } } else //Redcap UE { if (p_TBSize <= 12112) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24168) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36224) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48280) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60336) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72392) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84448) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96504) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108560) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120616) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132672) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144728) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156784) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 168840) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 180896) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 192952) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205008) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217064) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229120) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } } return f_NR_Build_N_PDCP_SDUList(v_N, v_PdcpSduSize); }

After Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } else if (p_TBSize <= 24200) { v_N := 2; v_PdcpSduSize := (p_TBSize - 200)/16; } else if (p_TBSize <= 36272) { v_N := 3; v_PdcpSduSize := (p_TBSize - 272)/24; } else if (p_TBSize <= 48344) { v_N := 4; v_PdcpSduSize := (p_TBSize - 344)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60416) { v_N := 5; v_PdcpSduSize := (p_TBSize - 416)/40; } else if (p_TBSize <= 72488) { v_N := 6; v_PdcpSduSize := (p_TBSize - 488)/48; } else if (p_TBSize <= 84560) { v_N := 7; v_PdcpSduSize := (p_TBSize - 560)/56; } else if (p_TBSize <= 96632) { v_N := 8; v_PdcpSduSize := (p_TBSize - 632)/64; } else if (p_TBSize <= 108704) { v_N := 9; v_PdcpSduSize := (p_TBSize - 704)/72; } else if (p_TBSize <= 120776) { v_N := 10; v_PdcpSduSize := (p_TBSize - 776)/80; } else if (p_TBSize <= 132848) { v_N := 11; v_PdcpSduSize := (p_TBSize - 848)/88; } else if (p_TBSize <= 144920) { v_N := 12; v_PdcpSduSize := (p_TBSize - 920)/96; } else if (p_TBSize <= 156992) { v_N := 13; v_PdcpSduSize := (p_TBSize - 992)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 169064) { v_N := 14; v_PdcpSduSize := (p_TBSize - 1064)/112; } else if (p_TBSize <= 181136) { v_N := 15; v_PdcpSduSize := (p_TBSize - 1136)/120; } else if (p_TBSize <= 193208) { v_N := 16; v_PdcpSduSize := (p_TBSize - 1208)/128; } else if (p_TBSize <= 205280) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1280)/136; } else if (p_TBSize <= 217352) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1352)/144; } else if (p_TBSize <= 229424) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1424)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } } else //Redcap UE - Table 7.1.1.4.2.5.3.2-2A: //WA#WI=1066074 { if (p_TBSize <= 12112) { v_N := 1; v_PdcpSduSize := (p_TBSize - 112)/8; // Table 7.1.1.4.2.5.3.2-2A: //WA#WI=1066074 } else if (p_TBSize <= 24168) { v_N := 2; v_PdcpSduSize := (p_TBSize - 168)/16; } else if (p_TBSize <= 36224) { v_N := 3; v_PdcpSduSize := (p_TBSize - 224)/24; } else if (p_TBSize <= 48280) { v_N := 4; v_PdcpSduSize := (p_TBSize - 280)/32; //@sic R5s190927 sic@ } else if (p_TBSize <= 60336) { v_N := 5; v_PdcpSduSize := (p_TBSize - 336)/40; } else if (p_TBSize <= 72392) { v_N := 6; v_PdcpSduSize := (p_TBSize - 392)/48; } else if (p_TBSize <= 84448) { v_N := 7; v_PdcpSduSize := (p_TBSize - 448)/56; } else if (p_TBSize <= 96504) { v_N := 8; v_PdcpSduSize := (p_TBSize - 504)/64; } else if (p_TBSize <= 108560) { v_N := 9; v_PdcpSduSize := (p_TBSize - 560)/72; } else if (p_TBSize <= 120616) { v_N := 10; v_PdcpSduSize := (p_TBSize - 616)/80; } else if (p_TBSize <= 132672) { v_N := 11; v_PdcpSduSize := (p_TBSize - 672)/88; } else if (p_TBSize <= 144728) { v_N := 12; v_PdcpSduSize := (p_TBSize - 728)/96; } else if (p_TBSize <= 156784) { v_N := 13; v_PdcpSduSize := (p_TBSize - 784)/104; //@sic R5-190676 R5s210884 sic@ } else if (p_TBSize <= 168840) { v_N := 14; v_PdcpSduSize := (p_TBSize - 840)/112; } else if (p_TBSize <= 180896) { v_N := 15; v_PdcpSduSize := (p_TBSize - 896)/120; } else if (p_TBSize <= 192952) { v_N := 16; v_PdcpSduSize := (p_TBSize - 952)/128; } else if (p_TBSize <= 205008) { v_N := 17; v_PdcpSduSize := (p_TBSize - 1008)/136; } else if (p_TBSize <= 217064) { v_N := 18; v_PdcpSduSize := (p_TBSize - 1064)/144; } else if (p_TBSize <= 229120) { v_N := 19; v_PdcpSduSize := (p_TBSize - 1120)/152; } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1176)/160; } } return f_NR_Build_N_PDCP_SDUList(v_N, v_PdcpSduSize); }