3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230283

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3022 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC TC 9.1.1.1 against SNPN-only UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Derivation of keys CK' and IK' for EAP-AKA' requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialize and pass the NID for NGC Cell A to the functions building the Authentication Request in Step 5 and Step 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	References	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

Change 1 of R5s230229[1] is also required to pass the TC in SNPN.

## Change 1

| Function name | function f_Get_EAP_OtherAuthenticationReqMsg() |
| --- | --- |
| Reason for change | Derivation of keys CK' and IK' for EAP-AKA' requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |
| Summary of change | Add input parameter p_NID to be passed to f_NG_EAP_AuthenticationInit() where it’s already being handled properly |
| TTCN module | Authentication_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_Get_EAP_OtherAuthenticationReqMsg(inout NG_NAS_SecurityParams_Type p_SecurityParams, NAS_PlmnId p_PLMN, octetstring p_EAPMsg) return template (value) NG_NAS_DL_Message_Type { p_SecurityParams := f_NG_EAP_AuthenticationInit(p_SecurityParams, p_PLMN, hex2str(px_IMSI_Def)); // @sic R5s201021 sic@ return cs_NG_AUTHENTICATION_REQUEST (p_SecurityParams.KSIamf, cs_ABBA (p_SecurityParams.ABBA, omit), -, -, cs_EAP_Message(p_EAPMsg)); }

After Change:

function f_Get_EAP_OtherAuthenticationReqMsg(inout NG_NAS_SecurityParams_Type p_SecurityParams, NAS_PlmnId p_PLMN, octetstring p_EAPMsg, template (omit) hexstring p_NID := omit) return template (value) NG_NAS_DL_Message_Type { p_SecurityParams := f_NG_EAP_AuthenticationInit(p_SecurityParams, p_PLMN, hex2str(px_IMSI_Def), -, -, p_NID); // @sic R5s201021 sic@ return cs_NG_AUTHENTICATION_REQUEST (p_SecurityParams.KSIamf, cs_ABBA (p_SecurityParams.ABBA, omit), -, -, cs_EAP_Message(p_EAPMsg)); }

## Change 2

| Function name | function fl_TC_9_1_1_1_TestBody() |
| --- | --- |
| Reason for change | Derivation of keys CK' and IK' for EAP-AKA' requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |
| Summary of change | Initialize and pass the NID for NGC Cell A to the functions building the Authentication Request in Step 5 and Step 7 |
| TTCN module | Authentication_NR5GC.ttcn |
| MCC160 Comment | Accepted, also passed it into f_NG_EAP_AuthenticationInit at step 7 |

Before Change:

function fl_TC_9_1_1_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NG_NAS_SecurityParams_Type v_NAS_IncorrectSecurityParams; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Steps 2-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_IdentityDef())))); //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_Identity_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ // start with the correct parameters, then change the AUTN v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; // @sic R5s210166 sic@ v_NAS_IncorrectSecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_IncorrectSecurityParams, v_PLMN, hex2str(px_IMSI_Def), -, sqnFailure); // @sic R5s201021 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_IncorrectSecurityParams, v_PLMN, f_NG_EAP_ChallengeReq (v_NAS_IncorrectSecurityParams, v_PLMN, '01'O))))); // @sic R5s201410 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_SyncFailure_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } //@siclog "Steps 9-10" siclog@ f_NR_AuthenticationEAP_AKA(v_NGC_NASCellA, tsc_SHT_IntegrityProtected_Ciphered, tsc_SHT_IntegrityProtected_Ciphered, v_PLMN); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_NAS_SecurityParams := f_NR5GC_Security_Get(); // @sic R5s201021 sic@ //@siclog "Step 11" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_AuthenticationResultMsg(v_NAS_SecurityParams)))); //@siclog "Steps 12-13" siclog@ v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(v_NGC_NASCellA, v_NAS_SecurityParams, Initial_Secure, v_GMM_MobilityInfo); // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // @sic R5s201021 sic@ //@siclog "Steps 14-15" siclog@ // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete //@siclog "Steps 16-17" siclog@ // UE Capability f_NR_Capability(v_NGC_NASCellA); //@siclog "Step 18" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, -,-,-,-,-, v_GMM_MobilityInfo.SessionStatus )))); //@siclog "Step 19" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } //@siclog Step 20 siclog@ f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, TESTMode_OFF, noRrcConnectionRelease); }

After Change:

import from NR_CellInfoInit all; import from NR_Parameters all; function fl_TC_9_1_1_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NG_NAS_SecurityParams_Type v_NAS_IncorrectSecurityParams; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; if(pc_SNPN_only){ v_NID_ASN := f_NR_InitNDI(v_NGC_NASCellA); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Steps 2-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_IdentityDef(), v_NID)))); //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_Identity_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ // start with the correct parameters, then change the AUTN v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; // @sic R5s210166 sic@ v_NAS_IncorrectSecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_IncorrectSecurityParams, v_PLMN, hex2str(px_IMSI_Def), -, sqnFailure, v_NID); // @sic R5s201021 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_IncorrectSecurityParams, v_PLMN, f_NG_EAP_ChallengeReq (v_NAS_IncorrectSecurityParams, v_PLMN, '01'O, v_NID), v_NID)))); // @sic R5s201410 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_SyncFailure_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } //@siclog "Steps 9-10" siclog@ f_NR_AuthenticationEAP_AKA(v_NGC_NASCellA, tsc_SHT_IntegrityProtected_Ciphered, tsc_SHT_IntegrityProtected_Ciphered, v_PLMN); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_NAS_SecurityParams := f_NR5GC_Security_Get(); // @sic R5s201021 sic@ //@siclog "Step 11" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_AuthenticationResultMsg(v_NAS_SecurityParams)))); //@siclog "Steps 12-13" siclog@ v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(v_NGC_NASCellA, v_NAS_SecurityParams, Initial_Secure, v_GMM_MobilityInfo); // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // @sic R5s201021 sic@ //@siclog "Steps 14-15" siclog@ // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete //@siclog "Steps 16-17" siclog@ // UE Capability f_NR_Capability(v_NGC_NASCellA); //@siclog "Step 18" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, -,-,-,-,-, v_GMM_MobilityInfo.SessionStatus )))); //@siclog "Step 19" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } //@siclog Step 20 siclog@ f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, TESTMode_OFF, noRrcConnectionRelease); }

# References

| [1] | R5s230229: NR5GC FR1 : Addition of Rel-16 SNPN test case 9.1.11.3 |
| --- | --- |