3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230290

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3027 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Rel 17 redcap test case 6.1.2.26 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 6.1.2.26 to the IWD_23wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.26 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.26.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A prose CR needs to be raised as well. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_TC_6_1_2_26_NR5GC_TestBody()	4

2.2	Function f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17()	9

3	Branches executed	10

4	Execution Log Files	10

4.1	HiSilicon Balong 5000	10

4.2	MTK MT6835	11

5	References	11

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.1.2.26 which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.1.2.26.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	R&S® 5G Protocol Conformance Test platform, Anritsu Protocol Conformance Test System ME7834NR

UE used:	Hisilicon Balong 5000, MTK6835

Verification Status:	PASS

# Corrections required

## Function f_TC_6_1_2_26_NR5GC_TestBody()

| Function name | f_TC_6_1_2_26_NR5GC_TestBody |
| --- | --- |
| Reason for change | - At T0 level NR Cell 11 is in Off state, thus SS cannot modify SIB1 of NR Cell 11 as per STEP 1 and STEP 2 requirements of the current prose if it is completely in Off state. - At Step 3 once NR Cell 11 is on UE is required to exclude the cell as candidate for selection for 300 sec as specified in TS 38.304 clause 5.3.1. Thus, 300s timer should start after STEP 3 and 60s timer of STEP4 needs to be within that time frame only. |
| Summary of change | 1. Changed the NR Cell 11 T0 power level to a non suitable Cell power as per the 38.508-1 Table 6.2.2.1-3 and Table 6.2.2.2-2 in order to fulfill the STEP 1 and STEP 2 requirements. 2. Moved t_WaitNoRRCSetupRequest.start for 300ms before f_NR_CheckNoRRCSetupReq for 60s function. Note: A prose CR will be riased separately. |
| TTCN module | 6_1_2\Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment | - Accepted in principle but implemented differently without change of prose. As NR Cell 11 is OFF the use of function f_NR_SS_ConfigureSysinfo applies for this case. See MCC160 implementation below - Accepted conditional to prose CR agreement at RAN5#99 |

Before Change:

function f_TC_6_1_2_26_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-231531 sic@ var boolean v_IsFR1; var NR_CellInfo_Type v_NR_CellInfo; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var CellBarredRedCap1Rx_r17_Type v_CellBarredRedCap1Rx := notBarred; var CellBarredRedCap2Rx_r17_Type v_CellBarredRedCap2Rx := notBarred; timer t_WaitNoRRCSetupRequest; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // Table 6.1.2.26.3.3.3-1: SIB1 for NR Cell 11 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell11); if (v_IsFR1) { // FR1 v_CellBarredRedCap1Rx := barred; } else { // FR2 or pc_maxNumberMIMO_LayersPDSCH_twoLayers v_CellBarredRedCap2Rx := barred; } if (pc_maxNumberMIMO_LayersPDSCH_twoLayers) { v_CellBarredRedCap2Rx := barred; } // @siclog "Step 1" siclog@ // The SS changes the SIB1 of NR Cell 11 to set cell Barred status to "true". f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell11, v_CellBarredRedCap1Rx, v_CellBarredRedCap2Rx); f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 2" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell11); // @siclog "Step 3" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T1" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 4" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell11, 60.0); t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // @siclog "Step 5" siclog@ // The SS changes the SIB1 of NR Cell 11 to delete the configuration of cellBarredRedCap-r17 f_NR_Delete_RedCap_ConfigCommonSIB_r17(nr_Cell11); // Table 6.1.2.26.3.3.3-2 f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 300s from Step4? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_WaitNoRRCSetupRequest.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6: UE sent an RRCSetupRequest on NR Cell 11 within 300s"); } [] t_WaitNoRRCSetupRequest.timeout { SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6"); } }

After Change:

function f_TC_6_1_2_26_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-231531 sic@ var boolean v_IsFR1; var NR_CellInfo_Type v_NR_CellInfo; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var CellBarredRedCap1Rx_r17_Type v_CellBarredRedCap1Rx := notBarred; var CellBarredRedCap2Rx_r17_Type v_CellBarredRedCap2Rx := notBarred; timer t_WaitNoRRCSetupRequest; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2) //WA#WI=1077639 }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //WA#WI=1077639 // Table 6.1.2.26.3.3.3-1: SIB1 for NR Cell 11 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell11); if (v_IsFR1) { // FR1 v_CellBarredRedCap1Rx := barred; } else { // FR2 or pc_maxNumberMIMO_LayersPDSCH_twoLayers v_CellBarredRedCap2Rx := barred; } if (pc_maxNumberMIMO_LayersPDSCH_twoLayers) { v_CellBarredRedCap2Rx := barred; } // @siclog "Step 1" siclog@ // The SS changes the SIB1 of NR Cell 11 to set cell Barred status to "true". f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell11, v_CellBarredRedCap1Rx, v_CellBarredRedCap2Rx); f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 2" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell11); // @siclog "Step 3" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T1" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 4" siclog@ // WA#WI=1077639, 60s should be within this 300s period only t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell11, 60.0); // WA#WI=1077639 //t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // @siclog "Step 5" siclog@ // The SS changes the SIB1 of NR Cell 11 to delete the configuration of cellBarredRedCap-r17 f_NR_Delete_RedCap_ConfigCommonSIB_r17(nr_Cell11); // Table 6.1.2.26.3.3.3-2 f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 300s from Step4? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_WaitNoRRCSetupRequest.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6: UE sent an RRCSetupRequest on NR Cell 11 within 300s"); } [] t_WaitNoRRCSetupRequest.timeout { SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6"); } } // @siclog "Step 7" siclog@ // Steps 2-6a1 of generic test procedure in TS 38.508-1 clause 4.9.5 on NR Cell 11. // NOTE: The UE performs a registration for mobility procedure and the RRC connection is released. f_NR5GC_MobilityRegistration(nr_Cell11); // @siclog "Step 8" siclog@ // The SS changes the SIB1 of NR Cell 1 to set intraFreqReselectionRedCap to not present. v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.intraFreqReselectionRedCap_r17 := omit; // Table 6.1.2.26.3.3.3-3 f_NR_CellInfo_Set(nr_Cell1, v_NR_CellInfo); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); // @siclog "Step 10" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 11" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0); // Note 2: The UE finishes in State DEREGISTRED because UE does not camp on any NR Cells after Step 11 }//end of f_TC_6_1_2_26_NR5GC_TestBody

MCC160 implementation:

function f_TC_6_1_2_26_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-231531 sic@ var boolean v_IsFR1; var NR_CellInfo_Type v_NR_CellInfo; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var CellBarredRedCap1Rx_r17_Type v_CellBarredRedCap1Rx := notBarred; var CellBarredRedCap2Rx_r17_Type v_CellBarredRedCap2Rx := notBarred; timer t_WaitNoRRCSetupRequest; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // Table 6.1.2.26.3.3.3-1: SIB1 for NR Cell 11 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell11); if (v_IsFR1) { // FR1 v_CellBarredRedCap1Rx := barred; } else { // FR2 or pc_maxNumberMIMO_LayersPDSCH_twoLayers v_CellBarredRedCap2Rx := barred; } if (pc_maxNumberMIMO_LayersPDSCH_twoLayers) { v_CellBarredRedCap2Rx := barred; } // @siclog "Step 1" siclog@ // The SS changes the SIB1 of NR Cell 11 to set cell Barred status to "true". f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell11, v_CellBarredRedCap1Rx, v_CellBarredRedCap2Rx); f_NR_SS_ConfigureSysinfo(nr_Cell11, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell11)); // @siclog "Step 2" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell11); …

## Function f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17()

| Function name | f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17 |
| --- | --- |
| Reason for change | 1. RedCap_ConfigCommonSIB_r17 template needs to be created and initialized to avoid run time error. |
| Summary of change | 1. Added a RedCap_ConfigCommonSIB_r17 template and assigned it with input parameters. |
| TTCN module | 6_1_2\Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. See MCC160 implementation below. |

Before Change:

function f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17 (NR_CellId_Type p_NR_CellId, CellBarredRedCap1Rx_r17_Type p_CellBarredRedCap1Rx, CellBarredRedCap2Rx_r17_Type p_CellBarredRedCap2Rx) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCap_ConfigCommon_r17.cellBarredRedCap_r17.cellBarredRedCap1Rx_r17 := p_CellBarredRedCap1Rx; v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCap_ConfigCommon_r17.cellBarredRedCap_r17.cellBarredRedCap2Rx_r17 := p_CellBarredRedCap2Rx; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

After Change:

function f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17 (NR_CellId_Type p_NR_CellId, CellBarredRedCap1Rx_r17_Type p_CellBarredRedCap1Rx, CellBarredRedCap2Rx_r17_Type p_CellBarredRedCap2Rx) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) RedCap_ConfigCommonSIB_r17 v_RedCap_ConfigCommonSIB_r17 := omit; //WA#WI=1077639 v_RedCap_ConfigCommonSIB_r17 := { omit, {p_CellBarredRedCap1Rx, p_CellBarredRedCap2Rx}}; // FR1 barred - FR1, Not Barred v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCap_ConfigCommon_r17 := valueof(v_RedCap_ConfigCommonSIB_r17); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

MCC160 implementation:

function f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17 (NR_CellId_Type p_NR_CellId, CellBarredRedCap1Rx_r17_Type p_CellBarredRedCap1Rx, CellBarredRedCap2Rx_r17_Type p_CellBarredRedCap2Rx) runs on NR_BASE_PTC { //@sic R5s230290 sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template (value) RedCap_ConfigCommonSIB_r17 v_RedCap_ConfigCommonSIB_r17; v_RedCap_ConfigCommonSIB_r17.halfDuplexRedCapAllowed_r17 := omit; v_RedCap_ConfigCommonSIB_r17.cellBarredRedCap_r17.cellBarredRedCap1Rx_r17 := p_CellBarredRedCap1Rx; v_RedCap_ConfigCommonSIB_r17.cellBarredRedCap_r17.cellBarredRedCap2Rx_r17 := p_CellBarredRedCap2Rx; v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCap_ConfigCommon_r17 := valueof(v_RedCap_ConfigCommonSIB_r17); // contents of template validates use of valueof here f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

# Branches executed

## This NR TC was executed in NR5GC FR1 Band Combination n41 with NEA1 and NIA1on R&S 5G Conformance Test Platform and  NR band n41 using NR ciphering nea2 and integrity algorithm nia2 on Anritsu Protocol Conformance Test System ME7834NR

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_6_1_2_26_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## MTK MT6835

The Mediatek MT6835 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_26_LOG.xml

- PICS/PIXIT parameter file:

TC_6_1_2_26_PIXIT.txt

# References

| [1] | R5s230291: Supporting information for addition of NR5GC Rel 17 redcap test case 6.1.2.26 in FR1 |
| --- | --- |