3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230295

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3030 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.2.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 23WK12 Implementation of the function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody, v_ConfigDrbSS is updated with {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)} at steps 7-8, Steps 10 – 11, Steps 13 – 14 and Steps 16 – 17. This change is coming from R5s230077 TF160 proposed implementation. When calling Step 4 function f_EUTRA_SS_SRBs_DRBs_Config inside f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5 To configure SRBs and DRBs we are making use of the variables p_DrbConfigList and p_SrbDrbList and making use of consolidated entries from p_DrbConfigList and p_SrbDrbList ( function call flow : f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5 f_EUTRA_SS_SRBs_DRBs_Config ) Point to note down here is that p_DrbConfigList -> is having DRB8 and DRB2 configurations p_SrbDrbList -> is having SRB1 , SRB2 , DRB2 and DRB8 configuration while sending RB configuration to SS :: Configuration will contain below information [0] – SRB1 [1] – SRB2 [2] – DRB2 [3] – DRB8 [4] – DRB 8 :: repeated DRB 8 configuration [5] – DRB2 :: repeated DRB 2 configuration f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5 f_EUTRA_SS_SRBs_DRBs_Config to avoid this conflict , it is proposed to proceed with the actual implementation proposed in R5s230077 to address the actual issue “TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Original changes from R5s230077 are done to address the actual issue Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody

| Function name | fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In 23WK12 Implementation of the function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody, v_ConfigDrbSS is updated with {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)} at steps 7-8, Steps 10 – 11, Steps 13 – 14 and Steps 16 – 17. This change is coming from R5s230077 TF160 proposed implementation. When calling Step 4 function f_EUTRA_SS_SRBs_DRBs_Config inside f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5 To configure SRBs and DRBs we are making use of the variables p_DrbConfigList and p_SrbDrbList and making use of consolidated entries from p_DrbConfigList and p_SrbDrbList Point to note down here is that p_DrbConfigList -> is having DRB8 and DRB2 configurations p_SrbDrbList -> is having SRB1 , SRB2 , DRB2 and DRB8 configuration while sending RB configuration to SS :: Configuration will contaithe n below information [0] – SRB1 [1] – SRB2 [2] – DRB2 [3] – DRB8 [4] – DRB 8 :: repeated DRB 8 configuration [5] – DRB2 :: repeated DRB 2 configuration f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5 f_EUTRA_SS_SRBs_DRBs_Config to avoid this conflict , it is proposed to proceed with the actual implementation proposed in R5s230077 to address the actual issue “TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13” |
| Summary of change | Original changes from R5s230077 are done to address the actual issue Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |
| TTCN module | ttcn/develop/ENDC/8_2_2/RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment | This doesn’t solve the problem that the DRB is not released at other steps. Alternative solution implemented: removed the DRBs from p_SrbDrbList |

Before Change:

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var template (value) RadioBearerList_Type v_ConfigDrbSS; var template (value) RadioBearerList_Type v_SrbDRBList; var template (value) octetstring v_NR_SCGConfig; var template (value) octetstring v_NR_RBConfig; var SubFrameTiming_Type v_Timing; var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs; var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get(); //@sic R5s200317 sic@ // @siclog "Steps 1 - 2" siclog@ // SN to MN terminated SCG f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1); f_EUTRA38_ENDC_Send_eNBKeys(NR); //Change keys on NR side to KeNB f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 4-5" siclog@ // modify MN terminated SCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB // Split RLC/MAC: configured on both EUTRA and NR v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}); v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)}; f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 7 - 8 " siclog@ // modify SN terminated Split DRB with security key s-KgNB to MN terminated Split DRB with security key change to keNB // Split RLC/MAC: configured on both EUTRA and NR v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max()); v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1, -, -, -,{cds_DRB_ToAddMod_r15(tsc_DRB2, tsc_DRB2 + 4, -, cs_508_RLC_Config_DRB_AM, {cs_ReestablishRLC_r15}, cs_508_LogicalChannelConfig_DRB_AM)}); @@ -1018,7 +1019,8 @@ module RRCReconfig_ENDC_EUTRA { @@ -1018,7 +1019,8 @@ module RRCReconfig_ENDC_EUTRA { v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Release(v_NR_RBConfig, v_Security_Params.SK_Counter); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 R5s221053 sic@ f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1, @@ -1042,7 +1044,9 @@ module RRCReconfig_ENDC_EUTRA { @@ -1042,7 +1044,9 @@ module RRCReconfig_ENDC_EUTRA { // RLC/MAC configured on EUTRA only v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ @@ -1076,7 +1080,8 @@ module RRCReconfig_ENDC_EUTRA { @@ -1076,7 +1080,8 @@ module RRCReconfig_ENDC_EUTRA { v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max()); f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1, cs_TimingInfo_EUTRA_NB (v_Timing), v_ConfigDrbSS, //@sic R5s221053 sic@ -, // tsc_C_RNTI_Def2, -, // p_MeasConfig -, // p_Dsr_TransMax := n4, v_RRCConnectionReconfiguration_v890_IEs, v_SrbDRBList); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17"); // @siclog "Step 18" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 19 - 20" siclog@ // modify SN terminated Split DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2); v_ConfigDrbSS := {cs_DRB_Release(2)}; f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated); f_EUTRA38_ENDC_Send_eNBKeys(NR); //Change keys on NR side to KeNB f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 22 - 23" siclog@ // modify MN terminated SCG DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB. v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}); v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)}; v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, v_ConfigDrbSS, //RadioBearerList_Type v_RadioResourceConfigDedicated, //RadioResourceConfigDedicated -, v_NR_RBConfig, -, // p_NAS_MsgList -, // p_SK_Counter -, // p_MaxEUTRA << SKIPPED CODE >> }

After Change:

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var template (value) RadioBearerList_Type v_ConfigDrbSS; var template (value) RadioBearerList_Type v_SrbDRBList; var template (value) octetstring v_NR_SCGConfig; var template (value) octetstring v_NR_RBConfig; var SubFrameTiming_Type v_Timing; var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs; var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get(); //@sic R5s200317 sic@ // @siclog "Steps 1 - 2" siclog@ // SN to MN terminated SCG f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1); f_EUTRA38_ENDC_Send_eNBKeys(NR); //Change keys on NR side to KeNB f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 4-5" siclog@ // modify MN terminated SCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB // Split RLC/MAC: configured on both EUTRA and NR v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}); v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)}; f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 7 - 8 " siclog@ // modify SN terminated Split DRB with security key s-KgNB to MN terminated Split DRB with security key change to keNB // Split RLC/MAC: configured on both EUTRA and NR v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing); //v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ //WA#WI=1078532 v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ //WA#WI=1078532 v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max()); v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1, -, -, -,{cds_DRB_ToAddMod_r15(tsc_DRB2, tsc_DRB2 + 4, -, cs_508_RLC_Config_DRB_AM, {cs_ReestablishRLC_r15}, cs_508_LogicalChannelConfig_DRB_AM)}); @@ -1018,7 +1019,8 @@ module RRCReconfig_ENDC_EUTRA { @@ -1018,7 +1019,8 @@ module RRCReconfig_ENDC_EUTRA { v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Release(v_NR_RBConfig, v_Security_Params.SK_Counter); //v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ //WA#WI=1078532 v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ //WA#WI=1078532 v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 R5s221053 sic@ f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1, @@ -1042,7 +1044,9 @@ module RRCReconfig_ENDC_EUTRA { @@ -1042,7 +1044,9 @@ module RRCReconfig_ENDC_EUTRA { // RLC/MAC configured on EUTRA only v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); //WA#WI=1078532 //v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ //WA#WI=1078532 v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ //WA#WI=1078532 // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ @@ -1076,7 +1080,8 @@ module RRCReconfig_ENDC_EUTRA { @@ -1076,7 +1080,8 @@ module RRCReconfig_ENDC_EUTRA { v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing); //v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_OneDRB_ConfigAM(tsc_DRB2)}; //@sic R5s200171 change 4.2, R5s230077 sic@ //WA#WI=1078532 v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ //WA#WI=1078532 v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max()); f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1, cs_TimingInfo_EUTRA_NB (v_Timing), v_ConfigDrbSS, //@sic R5s221053 sic@ -, // tsc_C_RNTI_Def2, -, // p_MeasConfig -, // p_Dsr_TransMax := n4, v_RRCConnectionReconfiguration_v890_IEs, v_SrbDRBList); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17"); // @siclog "Step 18" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 19 - 20" siclog@ // modify SN terminated Split DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2); v_ConfigDrbSS := {cs_DRB_Release(2)}; f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated); f_EUTRA38_ENDC_Send_eNBKeys(NR); //Change keys on NR side to KeNB f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ // Send trigger to NR to test DRB2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @siclog "Steps 22 - 23" siclog@ // modify MN terminated SCG DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB. v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}); v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)}; v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1, v_ConfigDrbSS, //RadioBearerList_Type v_RadioResourceConfigDedicated, //RadioResourceConfigDedicated -, v_NR_RBConfig, -, // p_NAS_MsgList -, // p_SK_Counter

#### MCC160 Implementation:

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ //@sic R5-198928 sic@

var template (omit)  RadioResourceConfigDedicated v_RadioResourceConfigDedicated;

var template (value) RadioBearerList_Type v_ConfigDrbSS;

var template (value) RadioBearerList_Type v_SrbDRBList;

var template (value) octetstring v_NR_SCGConfig;

var template (value) octetstring v_NR_RBConfig;

var SubFrameTiming_Type v_Timing;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();  //@sic R5s200317 sic@

// @siclog "Steps 1 - 2" siclog@

// SN to MN terminated SCG

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 4-5" siclog@

// modify MN terminated SCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB

// Split RLC/MAC: configured on both EUTRA and NR

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 6" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 7 - 8 " siclog@

// modify SN terminated Split DRB with security key s-KgNB to MN terminated Split DRB with security key change to keNB

// Split RLC/MAC: configured on both EUTRA and NR

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};  //@sic R5s200171 change 4.2, R5s230077 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef};

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max());

v_RadioResourceConfigDedicated := f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell1, -, -, -,{cds_DRB_ToAddMod_r15(tsc_DRB2, tsc_DRB2 + 4, -, cs_508_RLC_Config_DRB_AM, {cs_ReestablishRLC_r15}, cs_508_LogicalChannelConfig_DRB_AM)});

//@sic R5s200171 change 4.2 sic@

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-, -, -,

v_RadioResourceConfigDedicated

);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 9" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 10 - 11 " siclog@

// modify MN terminated Split DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB.

//@sic R5-201101 sic@

// RLC/MAC configured on EUTRA only

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Release(v_NR_RBConfig,

v_Security_Params.SK_Counter);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};  //@sic R5s200171 change 4.2, R5s230077 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef};

//@sic R5s200171 change 4.2 R5s221053 sic@

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-,-, -, -,

cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// @siclog "Step 12" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 13 - 14" siclog@

// SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB

// RLC/MAC configured on EUTRA only

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};  //@sic R5s200171 change 4.2, R5s230077 sic@

// NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef}; // @sic R5s201477 sic@

//@sic R5s200171 change 4.2 sic@

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (-,

v_NR_RBConfig,

v_Security_Params.SK_Counter,

fl_NR_GetP_Max());

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,  //@sic R5s221053 sic@

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList,

-,-, -, -,

cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def) );

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14");

// @siclog "Step 15" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 16 - 17" siclog@

// modify MN terminated MCG DRB with security key keNB to SN terminated Split DRB with security key change to s-KgNB

// Split RLC/MAC: configured on both EUTRA and NR

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing :=  f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_SendTimeToNR(eutra_Cell1,v_Timing);

v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8), cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};  //@sic R5s200171 change 4.2, R5s230077 sic@

v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef};

//@sic R5s200171 change 4.2 sic@

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig, -,fl_NR_GetP_Max());

f_EUTRA38_IntraLTE_HO_IntraCell_PSCellchange( eutra_Cell1,

cs_TimingInfo_EUTRA_NB (v_Timing),

v_ConfigDrbSS,    //@sic R5s221053 sic@

-, // tsc_C_RNTI_Def2,

-, // p_MeasConfig

-, // p_Dsr_TransMax  := n4,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDRBList);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17");

// @siclog "Step 18" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 19 - 20" siclog@

// modify SN terminated Split DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2);

v_ConfigDrbSS := {cs_DRB_Release(2)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

// @siclog "Step 21" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 22 - 23" siclog@

// modify MN terminated SCG DRB with security key keNB to SN terminated MCG DRB with security key change to s-KgNB.

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split});

v_ConfigDrbSS := {cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)};

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA38_508_ENDC_SendReconfiguration( eutra_Cell1,

v_ConfigDrbSS, //RadioBearerList_Type

v_RadioResourceConfigDedicated, //RadioResourceConfigDedicated

-,

v_NR_RBConfig,

-, // p_NAS_MsgList

-, // p_SK_Counter

-, // p_MaxEUTRA

-, // RRC_TransactionIdentifier

-, // MeasConfig p_MeasurementConfiguration

-, // MobilityControlInfo

-, // p_ULNASMsg

cs_TimingInfo_EUTRA_NB(v_Timing),

-,-,-,true);

f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR);  //default security process during PSCell change

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 23");

// @siclog "Step 24" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// @siclog "Steps 25 - 26" siclog@

// modify SN terminated MCG DRB with security key s-KgNB to MN terminated SCG DRB with security key change to keNB.

v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_DRB_Release(tsc_DRB2);

v_ConfigDrbSS := {cs_DRB_Release(2)};

f_EUTRA38_RRC_PSCellChange_IntraCell(eutra_Cell1, v_ConfigDrbSS, v_RadioResourceConfigDedicated);

f_EUTRA38_ENDC_Send_eNBKeys(NR);            //Change keys on NR side to KeNB

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 26");

// @siclog "Step 27" siclog@

// Send trigger to NR to test DRB2

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// Test body now finished

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

}