3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230297

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3032 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR UE capability testcases 8.1.5.1.1 and 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the Current TTCN Implementation of the function fl_NRCap_CommonChecks_r16 , before calling the function fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) , presence of NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension to be checked to avoid the runtime issue - In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ispresent is checked before calling fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) - In the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and fl_Check_MeasAndMobParametersCommon_supportedGapPattern, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_NRCap_CommonChecks_r16	3

2.2	Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH	5

2.3	Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_NRCap_CommonChecks_r16

| Function name | fl_NRCap_CommonChecks_r16 |
| --- | --- |
| Reason for change | In the Current TTCN Implementation of the function fl_NRCap_CommonChecks_r16 , before calling the function fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) , presence of NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension to be checked to avoid the run time issue . |
| Summary of change | ispresent is checked before calling fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_NRCap_CommonChecks_r16(UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; fl_NRCap_CommonChecks_r15(v_Received_NRCapabilityMsg, p_Configuration_Type); fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension); // Input(s) in field UE-NR-Capability-v1610 if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@ fl_Check_sidelinkParameters_r16(v_Received_NRCapabilityMsg); // Only in Test Case 8.1.5.1.1.NR5GC } } // end of fl_NRCap_CommonChecks_r16

After Change:

function fl_NRCap_CommonChecks_r16(UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; fl_NRCap_CommonChecks_r15(v_Received_NRCapabilityMsg, p_Configuration_Type); if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) //WA#WI=1078071 { fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension); // Input(s) in field UE-NR-Capability-v1610 } if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@ fl_Check_sidelinkParameters_r16(v_Received_NRCapabilityMsg); // Only in Test Case 8.1.5.1.1.NR5GC } } // end of fl_NRCap_CommonChecks_r16

## Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH

| Function name | fl_Check_maxNumberMIMO_LayersPDSCH |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not.. |
| Summary of change | In the function definition fl_Check_maxNumberMIMO_LayersPDSCH, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. The check for FR2 is still done using the capabilities signaled by the UE rather than using the PICS set. See MCC160 implementation below. |

Before Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { << SKIPPED CODE >> // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (f_NR_UESupportsFR2Band()) ) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

After Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { << SKIPPED CODE >> // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (/*f_NR_UESupportsFR2Band()*/pc_nrFR2) ) { //WA#WI=1078071 f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

MCC160 implementation:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { << SKIPPED CODE >> // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR > 256)) { // It is standalone NR, simple check for FR2 Band f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } …

## Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern

| Function name | fl_Check_MeasAndMobParametersCommon_supportedGapPattern |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not.. |
| Summary of change | In the function definition fl_Check_MeasAndMobParametersCommon_supportedGapPattern, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. The check for FR2 is still done using the capabilities signaled by the UE rather than using the PICS set. See MCC160 implementation below updating f_NR_UESupportsFR2Band and the calls to this function. |

Before Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2Band())) // This check applies only to option 2 and FR2 Band supported. { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; << SKIPPED CODE >>

After Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (/*f_NR_UESupportsFR2Band()*/pc_nrFR2)) // This check applies only to option 2 and FR2 Band supported. //WA#WI=1078071 { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; << SKIPPED CODE >>

MCC160 implementation:

function f_NR_UESupportsFR2Band(UE_NR_Capability p_NRCapability ) return boolean { //@sic R5s230297 sic@ var boolean v_FoundFR2 := false; var integer i:=0; var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability; while (i < lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) and (v_FoundFR2 == false)) { if ((v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[i].bandNR > 256) and (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[i].bandNR < 262)){ v_FoundFR2 := true; break; } i:=i+1; } return v_FoundFR2; }; function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2Band(p_NRCapability))) // This check applies only to option 2 and FR2 Band supported. { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; << SKIPPED CODE >>