3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230301

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3035 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC MAC test case 7.1.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A conformance UE may fail the test case at step-1 as SS is setting sk-Counter-r15=1 in rrcConnectionReconfiguration message. As per test spec it should should be used with condition EN-DC_PSCell_HO AND RBConfig_NoKeyChange, where the sk-Counter-r15 shall be omit |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set sk-Counter-r15 to omit in rrcConnectionReconfiguration at step 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_1_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC |
| --- | --- |
| Reason for change | A conformance UE may fail the test case at step-1 as SS is setting sk-Counter-r15=1 in rrcConnectionReconfiguration message. As per spec it should should be used with condition EN-DC_PSCell_HO AND RBConfig_NoKeyChange, where the sk-Counter-r15 shall be omit |
| Summary of change | Set sk-Counter-r15 to omit in rrcConnectionReconfiguration at step 1. |
| TTCN module | MAC_ENDC_EUTRA.ttcn. |
| MCC160 Comment | Accepted as improvement |

Before Change

function f_TC_7_1_1_1_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE in state 2A f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE); f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -); f_EUTRA_TestBody_Set(true); // Send trigger to NR to test on SCG bearer f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@sic R5-195336 sic@ //@siclog "Steps 1& 6" siclog@ // Update SgNB Keys for new SK Counter value v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter, false); //@sic R5s190934 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function f_TC_7_1_1_1_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE in state 2A f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_IDLE); f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TESTMode_OFF, -, -); f_EUTRA_TestBody_Set(true); // Send trigger to NR to test on SCG bearer f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@sic R5-195336 sic@ //@siclog "Steps 1& 6" siclog@ // Update SgNB Keys for new SK Counter value v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, omit, false); //@sic R5s190934 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }