3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230308

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3041 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcases 8.1.4.4.X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current 23wk12 iwd has omitted RLC, MAC and spCellConfigDedicated configurations while configuring Target CHO NR Cells R16 reconfiguration message in order to cater R5-231579 for t304 timer modification. In function f_NR5GC_RRCReconfiguration_IntraNR_HO , t304 timer needs to configure by caller methods (Needed because of change above) In function f_NR_GetCellGroupConfigHO, t304 timer needs to configure by caller methods (Needed because of change above) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called f_NR5GC_RRCReconfiguration_IntraNR_HO without CellGroupConfig as it prepares all the required configuration which are already tested in previous IWD versions so just added a new input t304 parameter to use further as per R5-231579 requirement. In function f_NR5GC_RRCReconfiguration_IntraNR_HO, added t304 parameter with a default ms1000 value. In function f_NR_GetCellGroupConfigHO, added t304 parameter with a default ms1000 value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.1.NR5GC, 8.1.4.4.2.NR5GC, 8.1.4.4.3.NR5GC, 8.1.4.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function f_NR_GenerateCondRRCReconfig_r16_Octetstring	4

2.2	Function f_NR5GC_RRCReconfiguration_IntraNR_HO	5

2.3	Function f_NR_GetCellGroupConfigHO	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_NR_GenerateCondRRCReconfig_r16_Octetstring

| Function name | f_NR_GenerateCondRRCReconfig_r16_Octetstring |
| --- | --- |
| Reason for change | 1. Current 23wk12 iwd has omitted RLC, MAC and spCellConfigDedicated configurations while configuring Target CHO NR Cells R16 reconfiguration message inroder to cater R5-231579 for t304 timer modification. |
| Summary of change | 1.Called f_NR5GC_RRCReconfiguration_IntraNR_HO without CellGroupConfig as it prepares all the required configuration which are already tested in previous IWD versions so just added a new input t304 parameter to use further as per R5-231579 requirement. |
| TTCN module | 8_1_4\RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted |

Before Change:

function f_NR_GenerateCondRRCReconfig_r16_Octetstring(NR_CellId_Type p_TargetCellId, ReconfigurationWithSync.t304 p_T304 := ms1000 // @sic R5-231579 sic@ ) runs on NR5GC_PTC return octetstring { var template (value) DL_DCCH_Message v_DL_DCCH_Message; // @sic R5s220271 sic@ var template (value) RRCReconfiguration v_CondRRCReconfig; // @sic R5s220271 sic@ var octetstring v_CondRRCReconfig_Octetstring; v_DL_DCCH_Message := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId, // @sic R5s220271 R5s220375 R5-231579 sic@ -, -, -, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount), -, cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, omit, omit, omit, cs_38508_SpCellConfig(omit, cs_NR_ReconfigurationWithSyncDef(f_NR_GetServingCellConfigCommon(p_TargetCellId),tsc_C_RNTI_Value3,p_T304, omit), omit, f_NR_CellInfo_GetRLF_TimersAndConstants(p_TargetCellId)))); v_CondRRCReconfig := v_DL_DCCH_Message.message_.c1.rrcReconfiguration; // @sic R5s220271 sic@ v_CondRRCReconfig_Octetstring := bit2oct(encvalue(v_CondRRCReconfig)); // String to be encoded return v_CondRRCReconfig_Octetstring; }

After Change:

function f_NR_GenerateCondRRCReconfig_r16_Octetstring(NR_CellId_Type p_TargetCellId, ReconfigurationWithSync.t304 p_T304 := ms1000 // @sic R5-231579 sic@ ) runs on NR5GC_PTC return octetstring { var template (value) DL_DCCH_Message v_DL_DCCH_Message; // @sic R5s220271 sic@ var template (value) RRCReconfiguration v_CondRRCReconfig; // @sic R5s220271 sic@ var octetstring v_CondRRCReconfig_Octetstring; v_DL_DCCH_Message := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId, -, -, -, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount),-,-,-,-,-,-, p_T304 ); // @sic R5s220271 R5s220375 R5-231579 sic@ v_CondRRCReconfig := v_DL_DCCH_Message.message_.c1.rrcReconfiguration; // @sic R5s220271 sic@ v_CondRRCReconfig_Octetstring := bit2oct(encvalue(v_CondRRCReconfig)); // String to be encoded return v_CondRRCReconfig_Octetstring; }

## Function f_NR5GC_RRCReconfiguration_IntraNR_HO

| Function name | f_NR5GC_RRCReconfiguration_IntraNR_HO |
| --- | --- |
| Reason for change | 1. t304 timer needs to configure by caller methods. |
| Summary of change | 1. Added t304 parameter with a default ms1000 value. |
| TTCN module | Common\NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRCReconfiguration_IntraNR_HO(NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MeasConfig p_MeasConfig := omit, template (omit) SIB1 p_SIB1 := omit, template (omit) SystemInformation p_SysInfo := omit, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) CellGroupConfig.sCellToAddModList p_SCellToAddModList := omit, template (omit) CellGroupConfig.sCellToReleaseList p_SCellToReleaseList := omit, template(value) PDCP_Config p_PDCP_Config := cs_38508_PDCP_Config, template (omit) RRCReconfiguration_v1540_IEs p_RRCReconfiguration_v1540_IEs := omit ) runs on NR5GC_PTC return template (value) DL_DCCH_Message { //@sic R5-194821 - R5s200690 change 4: added p_CellGroupConfig sic@ //@sic R5-213445: added p_PDCP_Config sic@ var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(p_RBConfig_KeyChange, p_PDCP_Config); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //Initialise RLC_BearerConfig with re-establishment var template (omit) octetstring v_SIB1 := omit; //@sic R5s190795 sic@ var template (omit) octetstring v_SysInfo := omit; //@sic R5s190795 sic@ //CellGroupCongfig with condition PCell_change if (ispresent(p_CellGroupConfig)) { //@sic R5s200690 change 4 sic@ v_CellGroupConfig := p_CellGroupConfig; // compiler warning may be ignored due to check } else { //CellGroupCongfig with condition PCell_change v_CellGroupConfig := f_NR_GetCellGroupConfigHO ( p_TargetCellId, v_RLC_BearerToAddModList, p_RNTI_Value, p_RACH_ConfigDedicated, p_SCellToAddModList, p_SCellToReleaseList ); } if (isvalue(p_SIB1)) { v_SIB1 := bit2oct(encvalue(p_SIB1)); } if (isvalue(p_SysInfo)) { v_SysInfo := bit2oct(encvalue(p_SysInfo)); } v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)), p_MasterKeyUpdate, v_SIB1, v_SysInfo, -, p_RRCReconfiguration_v1540_IEs); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, p_MeasConfig, v_RRCReconfiguration_v1530); return v_RRCReconfiguration; }

After Change:

function f_NR5GC_RRCReconfiguration_IntraNR_HO(NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MeasConfig p_MeasConfig := omit, template (omit) SIB1 p_SIB1 := omit, template (omit) SystemInformation p_SysInfo := omit, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) CellGroupConfig.sCellToAddModList p_SCellToAddModList := omit, template (omit) CellGroupConfig.sCellToReleaseList p_SCellToReleaseList := omit, template(value) PDCP_Config p_PDCP_Config := cs_38508_PDCP_Config, template (omit) RRCReconfiguration_v1540_IEs p_RRCReconfiguration_v1540_IEs := omit, ReconfigurationWithSync.t304 p_T304 := ms1000 //WA#WI=1079988 ) runs on NR5GC_PTC return template (value) DL_DCCH_Message { //@sic R5-194821 - R5s200690 change 4: added p_CellGroupConfig sic@ //@sic R5-213445: added p_PDCP_Config sic@ var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(p_RBConfig_KeyChange, p_PDCP_Config); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //Initialise RLC_BearerConfig with re-establishment var template (omit) octetstring v_SIB1 := omit; //@sic R5s190795 sic@ var template (omit) octetstring v_SysInfo := omit; //@sic R5s190795 sic@ //CellGroupCongfig with condition PCell_change if (ispresent(p_CellGroupConfig)) { //@sic R5s200690 change 4 sic@ v_CellGroupConfig := p_CellGroupConfig; // compiler warning may be ignored due to check } else { //CellGroupCongfig with condition PCell_change v_CellGroupConfig := f_NR_GetCellGroupConfigHO ( p_TargetCellId, v_RLC_BearerToAddModList, p_RNTI_Value, p_RACH_ConfigDedicated, p_SCellToAddModList, p_SCellToReleaseList, p_T304 //WA#WI=1079988 ); } if (isvalue(p_SIB1)) { v_SIB1 := bit2oct(encvalue(p_SIB1)); } if (isvalue(p_SysInfo)) { v_SysInfo := bit2oct(encvalue(p_SysInfo)); } v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)), p_MasterKeyUpdate, v_SIB1, v_SysInfo, -, p_RRCReconfiguration_v1540_IEs); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, p_MeasConfig, v_RRCReconfiguration_v1530); return v_RRCReconfiguration; }

## Function f_NR_GetCellGroupConfigHO

| Function name | f_NR_GetCellGroupConfigHO |
| --- | --- |
| Reason for change | - t304 timer needs to configure by caller methods. |
| Summary of change | - Added t304 parameter with a default ms1000 value. |
| TTCN module | Common\NR\NR_HandoverCommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetCellGroupConfigHO(NR_CellId_Type p_TargetCellId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerToAddModList, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) CellGroupConfig.sCellToAddModList p_SCellToAddModList := omit, template (omit) CellGroupConfig.sCellToReleaseList p_SCellToReleaseList := omit ) runs on NR_BASE_PTC return template(value) CellGroupConfig { //@sic R5-195323 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId); var template (value) ServingCellConfigCommon v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon(p_TargetCellId); var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_TargetCellId); //@sic R5s201387 sic@ var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId); //HO condition includes reconfigurationWithSync v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig); v_SpCellConfig.reconfigurationWithSync := cs_NR_ReconfigurationWithSyncDef(v_ServingCellConfigCommon, p_RNTI_Value, -, // p_T304 := ms1000, p_RACH_ConfigDedicated ); v_CellGroupConfig := cs_38508_CellGroupConfig(v_CellGroupId, p_RLC_BearerToAddModList, omit, // NR_RLC_BearerToReleaseList_Type v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig, p_SCellToAddModList, p_SCellToReleaseList); return v_CellGroupConfig; }

After Change:

function f_NR_GetCellGroupConfigHO(NR_CellId_Type p_TargetCellId, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerToAddModList, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) CellGroupConfig.sCellToAddModList p_SCellToAddModList := omit, template (omit) CellGroupConfig.sCellToReleaseList p_SCellToReleaseList := omit, ReconfigurationWithSync.t304 p_T304 := ms1000 //WA#WI=1079988 ) runs on NR_BASE_PTC return template(value) CellGroupConfig { //@sic R5-195323 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId); var template (value) ServingCellConfigCommon v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon(p_TargetCellId); var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_TargetCellId); //@sic R5s201387 sic@ var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId); //HO condition includes reconfigurationWithSync v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig); v_SpCellConfig.reconfigurationWithSync := cs_NR_ReconfigurationWithSyncDef(v_ServingCellConfigCommon, p_RNTI_Value, p_T304, // WA#WI=1079988 p_RACH_ConfigDedicated ); v_CellGroupConfig := cs_38508_CellGroupConfig(v_CellGroupId, p_RLC_BearerToAddModList, omit, // NR_RLC_BearerToReleaseList_Type v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig, p_SCellToAddModList, p_SCellToReleaseList); return v_CellGroupConfig; }