3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230315

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3045 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR UE capability test cases 8.1.5.1.1 and 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the Current TTCN Implementation of the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern , Gap Patterns 13,14,17,18 and 19 are expected to be “1” if UE configuration is NR_SA and it supports FR2 . Looks like this implementation is based on 38.306 broader statement “The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2“.More detailed explanation is given in 38.133 As per 38.133 , UE shall set Gap patterns 12-23 only if UE is NR SA FR2. In the current TTCN Implementation of the function f_MIMO_LayersDLParameterSet , else case verdict has to be corrected to match the scope of the function |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern, Gap patterns 13,14,17,18 and 19 are checked if UE is only NR SA FR2 In the function f_MIMO_LayersDLParameterSet , else case verdict is corrected to match the function scope |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern	3

2.2	Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern

| Function name | fl_Check_MeasAndMobParametersCommon_supportedGapPattern |
| --- | --- |
| Reason for change | - In the Current TTCN Implementation of the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern , Gap Patterns 13,14,17,18 and 19 are expected to be “1” if UE configuration is NR_SA and it supports FR2 . Looks like this implementation is based on 38.306 broader statement “The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2“ More detailed explanation is given in 38.133 As per 38.133 , UE shall set Gap patterns 12-23 only if UE is NR SA FR2. . |
| Summary of change | - In the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern, Gap patterns 13,14,17,18 and 19 are checked if UE is only NR SA FR2. For change (1) , R5s230297 is covering part of the correction but which is more intended to replace the check “f_NR_UESupportsFR2Band” with pc_nrFR2 check ; as f_NR_UESupportsFR2Band always returns True |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. The proposal to check strictly “if UE is only NR SA FR2” rather than simply supporting FR2, as stated in the reason for change is accepted but implementation is done according to contents signalled by the UE rather than the PICS set to run the test. Changes are done on top of implementation of R5s230297. See MCC160 implementation below. |

Before Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2Band()) // This check applies only to option 2 and FR2 Band supported. }

After Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (/*f_NR_UESupportsFR2Band()*/((not pc_nrFR1) and pc_nrFR2))) // This check applies only to option 2 and FR2 Band supported. //WA#WI=1078071 //WA#WI=1081777 { << SKIPPED CODE >> }

MCC160 implementation:

/* * @desc Function that returns true only for FR2 Only UEs, i.e. if UE signals FR2 Bands as suppported but no FR1 Band at all * @return boolean * @status APPROVED (ENDC, NR5GC) */ function f_NR_UESupportsFR2BandOnly(UE_NR_Capability p_NRCapability ) return boolean { //@sic R5s230297 R5s230315 Ch. 1 sic@ var boolean v_FoundFR1 := false; var integer i:=0; var UE_NR_Capability v_Received_NRCapabilityMsg := p_NRCapability; while (i < lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) and (v_FoundFR1 == false)) { if ((v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[i].bandNR < 257)){ // Condition for FR1 Band signalled by the UE v_FoundFR1 := true; break; } i:=i+1; } return (not v_FoundFR1); }; … << SKIPPED CODE >> … function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2BandOnly(p_NRCapability))) // This check applies only to NR SA FR2 { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 11) & v_PICS_GapPatterns_13and14_nr & substr(v_SupportedGapPatternsPICS, 11, 9); if (not(match(v_SupportedGapPatternsFromPics, v_UE_supportedGapPattern))){ //safe match due to as isvalue f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by UE supporting FR2 Bands in a NR_SA network"); } } else if ((p_Configuration_Type == NE_DC) or (p_Configuration_Type == NR_SA)){ // This check applies to option 2 or option 4 v_SupportedGapPatternsFromPics_1 := substr(v_SupportedGapPatternsPICS, 0, 11); v_SupportedGapPatternsFromPics_2 := substr(v_SupportedGapPatternsPICS, 11, 9); if ((not(match(v_SupportedGapPatternsFromPics_1, substr(v_UE_supportedGapPattern, 0, 11)))) or (not(match(v_SupportedGapPatternsFromPics_2, substr(v_UE_supportedGapPattern, 11, 9))))) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by the UE"); } } } else if (bit2int(v_SupportedGapPatternsPICS) > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS are set while UE does not signal such capabilities"); } }; //end of fl_Check_MeasAndMobParametersCommon_supportedGapPattern

## Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH

| Function name | f_MIMO_LayersDLParameterSet |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_MIMO_LayersDLParameterSet , else case verdict has to be corrected to match the scope of the function |
| Summary of change | In the function f_MIMO_LayersDLParameterSet , else case verdict is corrected to match the function scope |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_MIMO_LayersDLParameterSet () return integer { var integer v_MIMO_LayersDLParameterSet := 0; if (f_UptoOneParameterSet(pc_maxNumberMIMO_LayersPDSCH_twoLayers, pc_maxNumberMIMO_LayersPDSCH_fourLayers, pc_maxNumberMIMO_LayersPDSCH_eightLayers)) { // Case where as much only one is set if (pc_maxNumberMIMO_LayersPDSCH_twoLayers){ v_MIMO_LayersDLParameterSet := 2; } if (pc_maxNumberMIMO_LayersPDSCH_fourLayers) { v_MIMO_LayersDLParameterSet := 4; } if (pc_maxNumberMIMO_LayersPDSCH_eightLayers){ v_MIMO_LayersDLParameterSet := 8; } } else { // Fail case f_SetVerdict(inconc,__FILE__, __LINE__, "More than one NR Measurement gap patterns PICS are set simultaneously"); } return v_MIMO_LayersDLParameterSet; }

After Change:

function f_MIMO_LayersDLParameterSet () return integer { var integer v_MIMO_LayersDLParameterSet := 0; if (f_UptoOneParameterSet(pc_maxNumberMIMO_LayersPDSCH_twoLayers, pc_maxNumberMIMO_LayersPDSCH_fourLayers, pc_maxNumberMIMO_LayersPDSCH_eightLayers)) { // Case where as much only one is set if (pc_maxNumberMIMO_LayersPDSCH_twoLayers){ v_MIMO_LayersDLParameterSet := 2; } if (pc_maxNumberMIMO_LayersPDSCH_fourLayers) { v_MIMO_LayersDLParameterSet := 4; } if (pc_maxNumberMIMO_LayersPDSCH_eightLayers){ v_MIMO_LayersDLParameterSet := 8; } } else { // Fail case f_SetVerdict(inconc,__FILE__, __LINE__, "More than one MIMO_LayersPDSCH PICS are set simultaneously"); //WA#WI=1081777 } return v_MIMO_LayersDLParameterSet; }