3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230317

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4725 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT MAC testcase 22.3.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN implementation for the MAC test case 22.3.1.8 the PagingCarrier ID is calculated as 0 (Anchor carrier) in function f_NBIOT_UE_Page. However, as per ASP definition for Anchor carrier the ID needs to be omit : type record NB_PagingTrigger_Type { /* CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Calculated paging occasion ControlInfo : CnfFlag:=false; FollowOnFlag:=false */ /* primitive to trigger transmission of a paging on the NPCCH at a calculated paging occasion (TS 36.304, clause 7); * the paging occasion is calculated by TTCN and activation time is applied; * as for BCCH Info acc. to TS 36.331, clause 9.1.1.3 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; * therefore this needs to be done by the system simulator */ NB_PagingMessage_Type Paging, /* paging to be scheduled in the Type1 common search space beginning from the time given in the timing information of the common part of the ASP; the DCI is transmitted on the NPDCCH using P-RNTI */ integer PagingCarrierId optional /* When the UE supports paging on a non-anchor carrier and paging configuration for non-anchor carrier is provided in system information: Id of the DL non-anchor carrier (-> NB_NonAnchorCarrierCommonDL_Type.Id) on which the SS shall do the paging; the SS configuration of the carrier with the given Id corresponds to the entry in dl-ConfigList-r14 of SIB22 which has the same carrier frequency; NOTE: it is up to TTCN implementation to determine the carrier which fulfils the equation given in TS 36.304 clause 7.1 to determine the paging carrier; omit when paging happens on anchor-carrier */ }; This causes the test case to fail, therefore it needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The variable v_PagingCarrierID is assigned to omit for the case of Anchor carrier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_NBIOT_UE_Page	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_NBIOT_UE_Page

| Function name | f _NBIOT_UE_Page |
| --- | --- |
| Reason for change | In the current TTCN implementation for the MAC test case 22.3.1.8 the PagingCarrier ID is calculated as 0 (Anchor carrier) in function f_NBIOT_UE_Page. However, as per ASP definition for the Anchor Carrier the ID needs to be omit This causes the test case to fail, therefore needs to be corrected. |
| Summary of change | The variable v_PagingCarrier ID is assigned to omit |
| TTCN module | NBIOT_Paging.ttcn |
| MCC160 Comment | Accepted, since variable v_PagingCarrierID may have no value if omit was assigned by default, it is proposed to move the check immedeately after the value has been calculated |

Before Change:

function f_NBIOT_UE_Page(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit, template (omit) integer p_PagingCarrierId := omit) runs on NBIOT_PTC { var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; var NB_DefaultPagingCycle_Type v_PagingCycle; var SubFrameTiming_Type v_CurrentTiming; var SubFrameTimingList_Type v_PoList; var hexstring v_Imsi := px_IMSI_Def; var GutiParameters_Type v_GutiParameters; var template (value) PagingRecordList_NB_r13 v_PagingRecordList; var template (omit) integer v_PagingCarrierID := p_PagingCarrierId; var NBIOT_CellInfo_Type v_CellInfo; <<SKIPPED CODE>> if (not(ispresent(p_PagingCarrierId))) {//@sic R5s221226 sic@ v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); if(isvalue(v_CellInfo.Sysinfo.BCCH_Info.SIs[1].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib22_v1430)) { v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, f_NBIOT_CellInfo_GetSIB22(p_CellId), v_Imsi ); } } <<SKIPPED CODE>>

After Change:

function f_NBIOT_UE_Page(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit, template (omit) integer p_PagingCarrierId := omit) runs on NBIOT_PTC { var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; var NB_DefaultPagingCycle_Type v_PagingCycle; var SubFrameTiming_Type v_CurrentTiming; var SubFrameTimingList_Type v_PoList; var hexstring v_Imsi := px_IMSI_Def; var GutiParameters_Type v_GutiParameters; var template (value) PagingRecordList_NB_r13 v_PagingRecordList; var template (omit) integer v_PagingCarrierID := p_PagingCarrierId; var NBIOT_CellInfo_Type v_CellInfo; <<SKIPPED CODE>> if (not(ispresent(p_PagingCarrierId))) {//@sic R5s221226 sic@ v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); if(isvalue(v_CellInfo.Sysinfo.BCCH_Info.SIs[1].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib22_v1430)) { v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, f_NBIOT_CellInfo_GetSIB22(p_CellId), v_Imsi ); } } //WA#WI=1081973 if(valueof(v_PagingCarrierID) == 0) { v_PagingCarrierID := omit; } <<SKIPPED CODE>>