3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230320

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3049 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC DAPS handover test case 8.1.4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, according to R5w230004, time “T” for DAPS handover is set to 700ms. The current implementation gets the timing for T before calling f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(). Somwhow for the TTCN execution, from the start of f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig, it cost about 150ms to reach step 0 (exclusive, ie. all inside TTCN, without ASP call to SS). As a result, T is effectively reduced to 550ms for all SS configurations 2, The parameter p_RBConfig_KeyChange , is not passed on to function call f_NR_GetPdcpCountInfoListIntraNR_HO 3, Step 14B and 14C in f_NR5GC_DAPS_HO_SourceRelease, the NR_SYSTEM_IND and NR_DRB_COMMON_IND may reach TTCN out of order due to race condition 4, With the current TTCN implementation of f_NR5GC_DAPS_HO_SourceRelease, not only PDCP count of DRB, but also PDCP Count of SRBs are transferred at step 17, which is not correct 5, At step8 in test boday. UE discards the PDCP PDU for DRB data, because the PDCP PDU is not ciphered |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, updated to start T before step 1 2, updated call f_NR_GetPdcpCountInfoListIntraNR_HO at step 0 with p_RBConfig_KeyChange passed on 3, updated to allow NR_SYSTEM_IND and NR_DRB_COMMON_IND in any order 4, Corrected to just transfer PDCP count of DRB 5, Activated ciphering for DRB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.1.NR5GC, 8.1.4.3.2.NR5GC, 8.1.4.3.4.NR5GC, 8.1.4.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	8

2.4	Change 4	10

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig |
| --- | --- |
| Reason for change | 1, according to R5w230004, time “T” for DAPS handover is set to 700ms. The current implementation gets the timing for T before calling f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(). Somwhow for the TTCN execution, from the start of f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig, it cost about 150ms to reach step 0 (exclusive, ie. all inside TTCN, without ASP call to SS). As a result, T is effectively reduced to 550ms for all SS configurations 2, this function has parameter p_RBConfig_KeyChange , but it is not passed on to function call f_NR_GetPdcpCountInfoListIntraNR_HO at step 0 |
| Summary of change | 1, get T just before step 1 and return it for future usage 2, updated call f_NR_GetPdcpCountInfoListIntraNR_HO at step 0 with p_RBConfig_KeyChange , passed on |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | - Accepted as improvement. Order of parameters kept as they were and p_Timing is defined as out paremeter. - Accepted as improvement. |

Before Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, // timing based on Source Cell boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return NR_PdcpCountInfoList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_SRBs; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5-230116 sic@ var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s230023 sic@ v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); // @sic R5s221151 sic@ Deleted "v_RACH_ConfigGeneric.preambleTransMax := n200;" if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // Step 0. Source Cell: Get PDCP COUNT for the DAPS DRB and SRBs v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList); //RBConfig_KeyChange:=true, TimingInfo_Now, IncrementSRB1:=true // Step 1. Target Cell: Configuration of DRBs ......... ......... }

After Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(inout SubFrameTiming_Type p_Timing, // timing based on Source Cell NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return NR_PdcpCountInfoList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_SRBs; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5-230116 sic@ var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s230023 sic@ /* MOVED to later v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell */ //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); // @sic R5s221151 sic@ Deleted "v_RACH_ConfigGeneric.preambleTransMax := n200;" if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // Step 0. Source Cell: Get PDCP COUNT for the DAPS DRB and SRBs v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList, p_RBConfig_KeyChange); //RBConfig_KeyChange:=true, TimingInfo_Now, IncrementSRB1:=true p_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell // Step 1. Target Cell: Configuration of DRBs ......... ......... }

## Change 2

| Function name | f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback |
| --- | --- |
| Reason for change | Step 14B and 14C, the NR_SYSTEM_IND and NR_DRB_COMMON_IND may reach TTCN out of order due to race condition |
| Summary of change | Put NR_SYSTEM_IND and NR_DRB_COMMON_IND into interleave |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | Accepted. |

Before Change

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_SourceCellId, charstring p_StepNo := "", NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC //@sic R5s221151 sic@ { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_SourceCellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Step 14A. Source Cell: Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_SourceCellId, v_QosFlowId, {v_IPData})); // Step 14B. Source Cell: Check that the UE transmit a positive HARQ feedback SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_SourceCellId, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); // Step 14C. Target Cell: Receive loopbacked IP packet DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_TargetCellId, v_QosFlowId, {v_IPData})); //@sic R5s221151 sic@ t_Watchdog.stop; }

After Change

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_SourceCellId, charstring p_StepNo := "", NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC //@sic R5s221151 sic@ { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_SourceCellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Step 14A. Source Cell: Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_SourceCellId, v_QosFlowId, {v_IPData})); interleave { // Step 14B. Source Cell: Check that the UE transmit a positive HARQ feedback [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_SourceCellId, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); } // Step 14C. Target Cell: Receive loopbacked IP packet [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_TargetCellId, v_QosFlowId, {v_IPData})) {} //@sic R5s221151 sic@ } t_Watchdog.stop; }

## Change 3

| Function name | f_NR5GC_DAPS_HO_SourceRelease |
| --- | --- |
| Reason for change | With the current TTCN implementation, not only PDCP count of DRB, but also PDCP Count of SRBs are transferred at step 17. |
| Summary of change | Correct to just transfer PDCP count of DRB |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | Accepted. |

Before Change

function f_NR5GC_DAPS_HO_SourceRelease(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, charstring p_StepNo := "", boolean p_AdjustULcount := true) runs on NR_BASE_PTC { //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; // Step 15. Target Cell: Send RRCReconfiguration (daps_SourceRelesase) to release the Source Cell) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_DAPS_HO_ReleaseSource)); // Step 16. Target Cell: Receive RRCReconfigurationComplete) SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); // Step 17. Transfer of the PDCP Count for DAPS DRB from source to target cell: // a) Source Cell: Get PDCP COUNT. // b) Target Cell: Set PDCP COUNT. v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(p_SourceCellId); //if Steps 14A-14C of procedure 7.3.5.3.5 took place, the UL count needs to be adjusted if (p_AdjustULcount) { //For DAPS DRB, the PDCP COUNT is maintained and incremented by one in UL to consider the data exchanged in steps 14A-14C. v_NR_PdcpCountInfoList[2].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList[2].UL.Value) + 1), 32); //@sic R5s230025 sic@ } f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList, cs_TimingInfo_Now); }

After Change

function f_NR5GC_DAPS_HO_SourceRelease(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, charstring p_StepNo := "", boolean p_AdjustULcount := true) runs on NR_BASE_PTC { //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_Source; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_Target; // Step 15. Target Cell: Send RRCReconfiguration (daps_SourceRelesase) to release the Source Cell) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_DAPS_HO_ReleaseSource)); // Step 16. Target Cell: Receive RRCReconfigurationComplete) SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); // Step 17. Transfer of the PDCP Count for DAPS DRB from source to target cell: // a) Source Cell: Get PDCP COUNT. // b) Target Cell: Set PDCP COUNT. //KS R5xxx change_8143x v_NR_PdcpCountInfoList_Source:=f_NR_SS_PdcpCount_Get(p_SourceCellId); v_NR_PdcpCountInfoList_Target:=f_NR_SS_PdcpCount_Get(p_TargetCellId); v_NR_PdcpCountInfoList_Target[2].DL.Value := v_NR_PdcpCountInfoList_Source[2].DL.Value; v_NR_PdcpCountInfoList_Target[2].UL.Value := v_NR_PdcpCountInfoList_Source[2].UL.Value; //if Steps 14A-14C of procedure 7.3.5.3.5 took place, the UL count needs to be adjusted if (p_AdjustULcount) { //For DAPS DRB, the PDCP COUNT is maintained and incremented by one in UL to consider the data exchanged in steps 14A-14C. v_NR_PdcpCountInfoList_Target[2].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_Source[2].UL.Value) + 1), 32); //@sic R5s230025 sic@ } f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_Target, cs_TimingInfo_Now); }

## Change 4

| Function name | fl_TC_8_1_4_3_1And4_TestBody |
| --- | --- |
| Reason for change | 1, same as 1.1 2, At step8. UE discards the PDCP PDU for DRB data, because the PDCP PDU is not ciphered |
| Summary of change | 1, Get T in f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig 2, Active ciphering for DRB |
| TTCN module | RRC_DAPS_Handover_NR5GC |
| MCC160 Comment | - Accepted. Similar change also needed in fl_TC_8_1_4_3_2And5_TestBody - Not needed. Security is already configured in the SS and the use of PdcpCount.Set does not imply a reset of the SS AS security configuration, especially when the set PDCP SN is higher that the one used in AS_Security.StartRestart. |

Before Change

function fl_TC_8_1_4_3_1And4_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { //This function follows the RRC procedure described in TS 38.523-3, 7.3.5.3.5 var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms v_Timing3 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); // time for sending IP packets to source cell //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform //DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. v_NR_PdcpCountInfoList_DRB := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //TS 38.523-3, 7.3.5.3.5, Steps 1-8, R5w230004: this function now returns v_NR_PdcpCountInfoList instead of v_RadioBearerList //@siclog "Step 2" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable // of exchanging IP data on DRB#n of the first PDU session on NR Cell 1? f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 2", cs_TimingInfo_NR(v_Timing3)); //TS 38.523-3, 7.3.5.3.5, Steps 9-10 //After data exchange, transfer count to the target cell //f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, v_RadioBearerList, -, cs_TimingInfo_Now, false); //@sic R5s221151 sic@ v_NR_PdcpCountInfoList_DRB[0].DL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].DL.Value) + 1), 32); v_NR_PdcpCountInfoList_DRB[0].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].UL.Value) + 1), 32); f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A //@siclog "Steps 4-5" siclog@ f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, //TS 38.523-3, 7.3.5.3.5, Steps 11-14 "Step 5", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Steps 5A-5B" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(p_SourceCellId, "Step 5A", p_TargetCellId); //@sic R5s221151 sic@ //@siclog "Steps 6-7" siclog@ f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 7"); //TS 38.523-3, 7.3.5.3.5, Steps 15-17, @sic R5-230116 sic@ //@siclog "Step 8" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 8"); //TS 38.523-3, 7.3.5.3.5, Step 18 //@siclog "Steps 9-10" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_NoReply(p_SourceCellId); //TS 38.523-3, 7.3.5.3.5, Step 18 f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); //TS 38.523-3, 7.3.5.3.5, Steps 19-20 }

After Change

function fl_TC_8_1_4_3_1And4_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { //This function follows the RRC procedure described in TS 38.523-3, 7.3.5.3.5 var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; var NR_SecurityParams_Type v_RRC_SecurityParams; //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform //DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. v_NR_PdcpCountInfoList_DRB := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(v_Timing, p_SourceCellId, p_TargetCellId); //TS 38.523-3, 7.3.5.3.5, Steps 1-8, R5w230004: this function now returns v_NR_PdcpCountInfoList instead of v_RadioBearerList v_Timing3 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); // time for sending IP packets to source cell //@siclog "Step 2" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable // of exchanging IP data on DRB#n of the first PDU session on NR Cell 1? f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 2", cs_TimingInfo_NR(v_Timing3)); //TS 38.523-3, 7.3.5.3.5, Steps 9-10 //After data exchange, transfer count to the target cell //f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, v_RadioBearerList, -, cs_TimingInfo_Now, false); //@sic R5s221151 sic@ v_NR_PdcpCountInfoList_DRB[0].DL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].DL.Value) + 1), 32); v_NR_PdcpCountInfoList_DRB[0].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].UL.Value) + 1), 32); f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A //@siclog "Steps 4-5" siclog@ f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, //TS 38.523-3, 7.3.5.3.5, Steps 11-14 "Step 5", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Steps 5A-5B" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(p_SourceCellId, "Step 5A", p_TargetCellId); //@sic R5s221151 sic@ //@siclog "Steps 6-7" siclog@ f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 7"); //TS 38.523-3, 7.3.5.3.5, Steps 15-17, @sic R5-230116 sic@ v_RRC_SecurityParams:= f_NR_Security_Get(); v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_TargetCellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_TargetCellId, v_RRC_SecurityParams.AS_Integrity, v_RRC_SecurityParams.AS_Ciphering); //@siclog "Step 8" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 8"); //TS 38.523-3, 7.3.5.3.5, Step 18 //@siclog "Steps 9-10" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_NoReply(p_SourceCellId); //TS 38.523-3, 7.3.5.3.5, Step 18 f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); //TS 38.523-3, 7.3.5.3.5, Steps 19-20 }