3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230322

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3051 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MAC test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, As per change done in R5-231905, restablishPDCP to true in RRCReconfiguration so that UE doesn`t retransmit the data even after the reconfiguration with sync procedure. As per 38.323 section 5.4.1 PSR shall be triggered in this case which is causing the TC to fail. 2, SS needs to ignore following scheduling requests before step 20, and 29, sometimes unexpected SR indication reach TTCN after DRB is received and SR handler is disabled. This is due to racing condition on the DRB indication and SR indication |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, PSR is made omit in PDCP config so that UE doesn’t trigger PSR. 2, Added 100ms delay before stop handler for SR, to improve the stability of the testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | As per change done in R5-231905, restablishPDCP to true in RRCReconfiguration so that UE doesn`t retransmit the data even after the reconfiguration with sync procedure. As per 38.323 section 5.4.1 PSR shall be triggered in this case which is causing the TC to fail |
| Summary of change | PSR is made omit in PDCP config so that UE doesn’t trigger this. Note: This need a prose change in PSR config. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement |

Before Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ ......... ......... //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 R5s220795 R5-231905 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit), v_SS_Drb_ConfigList, true, v_CellGroupConfig, true);//@sic R5-213132 sic@ // @siclog "Step 4" sigloc@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); //@siclog "Steps 16A-21" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID); //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit), v_SS_Drb_ConfigList,true, v_CellGroupConfig, true); //@sic R5-204418 R5s210138 R5-227518 R5-23231905 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_Drb_ConfigList[0]}); }

After Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ ......... ......... //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 R5s220795 R5-231905 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList, true, v_CellGroupConfig, true);//@sic R5-213132 sic@ // @siclog "Step 4" sigloc@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); //@siclog "Steps 16A-21" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID); //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList,true, v_CellGroupConfig, true); //@sic R5-204418 R5s210138 R5-227518 R5-23231905 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_Drb_ConfigList[0]}); }

## Change 2

| Function name | function f_TC_7_1_1_9_1_Step16Ato21 |
| --- | --- |
| Reason for change | SS needs to ignore following scheduling requests before step 20, sometimes unexpected SR indication reach TTCN after DRB is received and SR handler is disabled, due to racing condition on the DRB indication and SR indication |
| Summary of change | A delay of 100ms is added before SR handler to be disabled, to improve the stability of the testing |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as improvement, implemented differently as using a delay timer is not acceptable. See MCC160 proposed implementation below: The default handler a_NR5GC_SR_Handler can remain activated as there is no requirement in the prose to check that UE doesn't send SR. |

Before Change

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID) runs on NR_BASE_PTC { //@sic R5-206320 sic@ ......... ......... //@siclog "Step 20A" siclog@ //@sic R5s201457 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id); // @sic R5s220015 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ }

After Change

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID) runs on NR_BASE_PTC { //@sic R5-206320 sic@ ......... ......... //@siclog "Step 20A" siclog@ //@sic R5s201457 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id); // @sic R5s220015 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_Delay(0.1); deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@ }

MCC160 proposed implementation

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID) runs on NR_BASE_PTC

{ //@sic R5-206320 sic@

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@

//@sic R5s210896 R5s230261 sic@

// @siclog "Step 16 void" sigloc@

// @siclog "Step 16A" sigloc@

//@sic R5-204418 sic@

//The SS ignores scheduling requests and does not allocate any uplink grant.

v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1));

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 17" siclog@

//PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B)

//@sic R5s201457 R5s210896 R5s220687 sic@

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(2,crs_NR_PDCP_SDU_37B)));  //@sic R5-227518 sic@

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now,

{cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1)));

//@siclog "Step 18 " siclog@

//Indication of Scheduling Request

SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq));

//@siclog "Step 19" siclog@

//The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0.

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits

f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1));

//@siclog "Step 20" siclog@

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any,

{cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND;

//@siclog "Step 20A" siclog@

//@sic R5s201457 sic@

f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id); // @sic R5s220015 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

// The default handler a_NR5GC_SR_Handler can remain activated as there is no requirement in the prose to check that UE doesn't send SR

// deactivate(v_IgnoreNRSR); @sic R5-204418 R5s230322 sic@

}

/*

* @desc      Test Body 7.1.1.9.1 Step 24 to Step 28

* @param     p_NR_DRB_Id

* @param     p_LCID

* @param     p_NR_RadioBearerList

* @param     p_NR_HarqProcessConfig_UL (default value: omit)

* @status    APPROVED (ENDC, NR5GC)

*/

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig_UL:= omit) runs on NR_BASE_PTC

{ //@sic R5s200855 R5-206320 sic@

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);//@sic R5s200855 sic@

//@siclog "Step 24A" siclog@

//The SS ignores scheduling requests and does not allocate any uplink grant.

//@sic R5s201457 sic@

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

// v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); @sic R5-204418 R5s230322 sic@

//@siclog "Step 25" siclog@

//The SS transmits MAC PDU using same HARQ process and NDI as in step 17.

//PDCP SDU = 37B + 3B PDCP Header + 3B RLC Header + 2B MAC Header = 45B = 360 bits (an allowed TB size as per 38.523-3 Annex B)

//@sic R5s201457 sic@

f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(1)); //@sic R5s211710 sic@

v_EncodedRlcPdu1:= f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(3,crs_NR_PDCP_SDU_37B)));  //@sic R5-227518 sic@

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now,

{cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)})); //@sic R5s211710 sic@

//@siclog "Step 26 " siclog@

//Indication of Scheduling Request

SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq));

//@siclog "Step 27" siclog@

//The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0.

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits  @sic R5s201457 sic@

f_NR_OneULGrantTransmission_NDI(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1),-, false);// @sic R5s201457 R5s220669 sic@

// @siclog "Step 28" siclog@

DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any,

{cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28");

// @siclog "Step 29" siclog@

//The SS transmits a STATUS PDU.

// Restrict the SS to use of the HARQ proccess id used by the test case

f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@

f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications.

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications.

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -,-,-,-,p_NR_HarqProcessConfig_UL); //Default Grant configuration.

// The default handler a_NR5GC_SR_Handler can remain activated as there is no requirement in the prose to check that UE doesn't send SR

//deactivate(v_IgnoreNRSR); @sic R5-204418  R5s220687 R5s230322 sic@

}

## Change 3

| Function name | function f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | SS needs to ignore following scheduling requests before step 28, sometimes unexpected SR indication reach TTCN after DRB is received and SR handler is disabled , due to racing condition on the DRB indication and SR indication. |
| Summary of change | A delay of 100ms is added before SR handler to be disabled, to improve the stability of the testing |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | See change 2.2 |

Before Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig_UL:= omit) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ ......... ......... // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -,-,-,-,p_NR_HarqProcessConfig_UL); //Default Grant configuration. deactivate(v_IgnoreNRSR);//@sic R5-204418 R5s220687 sic@ }

After Change

function f_TC_7_1_1_9_1_Step24to29(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID,template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, template (omit) NR_HarqProcessConfig_Type p_NR_HarqProcessConfig_UL:= omit) runs on NR_BASE_PTC { //@sic R5s200855 R5-206320 sic@ ......... ......... // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); //@sic R5-204418 R5s201457 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //Disable SR indications. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //Disable UL HARQ indications. f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -,-,-,-,p_NR_HarqProcessConfig_UL); //Default Grant configuration. f_Delay(0.1); deactivate(v_IgnoreNRSR);//@sic R5-204418 R5s220687 sic@ }