3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230326

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3053 | rev | - | Current version: | 17.6.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-16 MDT test case 8.1.6.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 8.1.6.2.2to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.6.2.2required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	3

3	Branches executed	12

4	Execution Log Files	13

4.1	Samsung SM-A546	13

5	References	13

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.2.2which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.6.2.2

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung SM-A546

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_8_1_6_2_2_NR5GC_EUTRA () |
| --- | --- |
| Reason for change | With current co-ordination function SS will be not passing EUTRA cell ID to NR component, SO to pass EUTRA cell ID SS has to add EUTRA_CellId on coordination message. |
| Summary of change | Added EUTRA cell ID variable on coordination message before passing it to NR componenent. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_EUTRA |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_6_2_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting f_EUTRA38_IRAT_Init(c31); f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

After Change

function f_TC_8_1_6_2_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting var template (value) B28_Type v_EUTRA_CellId; var template (value) CoOrd_EUTRASysInfo_Typev_CoOrd_SysInfo; f_EUTRA38_IRAT_Init(c31); //f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT v_CoOrd_SysInfo := f_EUTRA_CoOrdSysInfo(eutra_Cell1); v_EUTRA_CellId := int2bit(f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1),28); v_CoOrd_SysInfo.CellId := v_EUTRA_CellId; f_EUTRA_NR_SendSysinfoCoOrd(NR, cs_EUTRA_NR_CoOrdSysInfo_Eutra({v_CoOrd_SysInfo})); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_8_1_6_2_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

  - Change 2

| Function name | f_TC_8_1_6_2_2_NR5GC () |
| --- | --- |
| Reason for change | As per prose 38.523-1, Table 8.1.6.2.2.3.3-2: SIB5 of NR Cell 1, SIB5 must have only entry for CarrierFreqEUTRA[1], but current TTCN is allowing more than one, SO UE may take more time to search different EUTRA frequencies |
| Summary of change | Added first entry only on SIB5 using v_CarrierFreqListEUTRA[0]. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_6_2_2_NR5GC() runs on NR5GC_PTC { //Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting var EUTRA_NR_CoOrd_SysInfo_Typev_CoOrd_SysInfo; f_NR5GC_IRAT_Init(NR_6); // System information combination NR-6, acc. to TS 38.508-1 clause 4.4.3.1.3 is used in NR Cell v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //Table 8.1.6.2.2.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.2.3.3-2: SIB5 of NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -82);//@sic R5-216498 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell1, -78, -82); //@sic R5-216498 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_6_2_2_TestBody(v_CoOrd_SysInfo); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); // Tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function f_TC_8_1_6_2_2_NR5GC() runs on NR5GC_PTC { //Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting var EUTRA_NR_CoOrd_SysInfo_Typev_CoOrd_SysInfo; f_NR5GC_IRAT_Init(NR_6); // System information combination NR-6, acc. to TS 38.508-1 clause 4.4.3.1.3 is used in NR Cell v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var CarrierFreqListEUTRAv_CarrierFreqListEUTRA; //Table 8.1.6.2.2.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.2.3.3-2: SIB5 of NR Cell 1 //f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); v_CarrierFreqListEUTRA := f_NR5GC_CellInfo_GetSIB5_CarrierFreqListEUTRA(nr_Cell1); f_NR5GC_CellInfo_SetSIB5_CarrierFreqListEUTRA(nr_Cell1, {v_CarrierFreqListEUTRA[0]}); f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -82);//@sic R5-216498 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell1, -78, -82); //@sic R5-216498 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_6_2_2_TestBody(v_CoOrd_SysInfo); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); // Tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

  - Change 3

| Function name | fl_TC_8_1_6_2_2_TestBody () |
| --- | --- |
| Reason for change | - At step 4, UE requires at least 35 sec to detect and measure EUTRA cell and populate its first entry with EUTRA measurement on logMeasInfoList-r16 at step13. NR cell have configured cell specific DRX for 128RF and as per Spec 38.133 Table 4.2.2.4-1, DRX with 1.28sec Time to detect EUTRA will start 32 seconds onwards. #associatedproseCR raised during RAN5#99 - At step 8, Function f_NR_RRC_ConnEst_DefWithNas can handle RRC setup request and RRC setup, so no need to add separate functions for it on test body. - At step 9-11, UE will be triggering service request instead registration request. - At step 13 UE will use SRB2 instead SRB1 for Loggedmeasurment reporting. - At step 13 UE will have multiple entries for LogMeasInfo_r16 on ueInformationResponse as UE will be performing logging for 35 seconds at step4. - UE can have one or more entry on any of logMeasInfoList-r16 .Sofor evaluation of EUTRA measurementment only on first entry might cause unnecessary failure of the case. |
| Summary of change | - Modified delay at step 4 to 35 seconds. - Commented f_NR_RRC_SetupRequest_Def and f_NR_RRC_Setup_Def. - Modified f_Check_NG_ServiceReqMsg. - Modified SRB2 for UEinformationresponse reception. - Removed if condition to check one entry on v_FieldCnt. - Added for loop to check first logged EUTRA measurement on any of the logMeasInfoList-r16 entries. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR |
| MCC160 Comment | - Accepted, conditional to prose CR agreement at RAN5#99 - Accepted - Accepted - Accepted - Accepted, conditional to prose CR agreement at RAN5#99 - Accepted, conditional to prose CR agreement at RAN5#99 |

Before Change

function fl_TC_8_1_6_2_2_TestBody(EUTRA_NR_CoOrd_SysInfo_Typep_CoOrd_SysInfo) runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (present) MeasQuantityResultsv_MeasQuantityResults; var NG_NAS_MSG_Indication_Typev_ReceivedMsg; var GMM_MobilityInfo_Typev_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var EUTRA_PhysCellIdv_EUTRA_CellId; var integer v_FieldCnt; //@siclog "Step 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType set to periodical on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms2560)))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //The SS changes cell power levels according to the row "T1" in table 8.1.6.2.2.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -78, -82); //FFS for FR2 @sic R5-221556 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait 10 seconds for UE performing the logging at regular time intervals. f_Delay(10.0); //@siclog "Step 5" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 6" siclog@ //The UE transmits an RRCRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The SS transmit an RRCSetup message. f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 8" siclog@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? //@sic R5-225387 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_))); // receive RRC Connection Setup Complete as per Table 8.1.6.2.2.3.3-4 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 9 - 11" siclog@ //Steps 6 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 13" siclog@ //Check: Does the UE send an UEInformationResponse message including logMeasReport with E-UTRAN Cell1? v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt> 1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: More than one entries") } v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[0]; v_EUTRA_CellId := bit2int (p_CoOrd_SysInfo.Eutra[0].CellId); if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def( -, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, {cr_MeasResult2EUTRA_r16(p_CoOrd_SysInfo.Eutra[0].Arfcn,{cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId,v_MeasQuantityResults)})} ))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.6.2.2 Steps 13"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } }//End of f_TC_8_1_6_2_2_TestBody()

After Change

function fl_TC_8_1_6_2_2_TestBody(EUTRA_NR_CoOrd_SysInfo_Typep_CoOrd_SysInfo) runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (present) MeasQuantityResultsv_MeasQuantityResults; var NG_NAS_MSG_Indication_Typev_ReceivedMsg; var GMM_MobilityInfo_Typev_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var EUTRA_PhysCellIdv_EUTRA_CellId; var NG_NAS_GutiParameters_Typev_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentityv_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer i:= 0 ; var integer v_FieldCnt ; var booleank := false ; //@siclog "Step 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType set to periodical on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms2560)))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //The SS changes cell power levels according to the row "T1" in table 8.1.6.2.2.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -78, -82); //FFS for FR2 @sic R5-221556 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait 10 seconds for UE performing the logging at regular time intervals. f_Delay(35.0); //@siclog "Step 5" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 6" siclog@ //The UE transmits an RRCRequest message. //f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The SS transmit an RRCSetup message. //f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 8" siclog@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? //@sic R5-225387 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_))); // receive RRC Connection Setup Complete as per Table 8.1.6.2.2.3.3-4 if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 9 - 11" siclog@ //Steps 6 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS send a UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 13" siclog@ //Check: Does the UE send an UEInformationResponse message including logMeasReport with E-UTRAN Cell1? v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); /* if (v_FieldCnt> 1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13: More than one entries") } */ for (i := 0; i<v_FieldCnt ; i:= i + 1) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; v_EUTRA_CellId := bit2int (p_CoOrd_SysInfo.Eutra[0].CellId); if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16_Def( -, -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, {cr_MeasResult2EUTRA_r16(p_CoOrd_SysInfo.Eutra[0].Arfcn,{cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId,v_MeasQuantityResults)})} ))) { k := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); break; } } if (k == false) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueInformationResponse-r16 Message Failed"); } }//End of f_TC_8_1_6_2_2_TestBody()

- Branches executed

The test case was executed on NR band n78using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - SamsungSM-A546

The SamsungSM-A546UEpassed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_6_2_2_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_6_2_2_PIXIT.xml

- References

| [1] | R5s230327: Supporting information for addition of Rel-16 MDT test case 8.1.6.2.2 in FR1 |
| --- | --- |