3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230330

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3055 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC SON MDT test case 8.1.6.1.2.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per R5-230757, at step 25, “Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension is present” But the current TTCN implementation expect the nonCriticalExtension always present |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated step 35 to accept nonCriticalExtention IE as any or omit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | f_TC_8_1_6_1_2_12_TestBody() |
| --- | --- |
| Reason for change | As per R5-230757, at step 35, “Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension is present” But the current TTCN implementation expect the nonCriticalExtension always present |
| Summary of change | Updated step 35 to accept nonCriticalExtention IE as any or omit |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_6_1_2_12_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 33" siclog@ //The UE transmits an RRCRequest message on NR Cell 1. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits an RRCSetup message on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 35" siclog@ //@sic R5-230757 sic@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? SRB.receive(car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-,?),?)) -> value v_ReceivedAsp; v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if (ispresent (v_ReceivedAsp.Signalling.Rrc.Dcch.message_.c1.rrcSetupComplete.criticalExtensions.rrcSetupComplete.nonCriticalExtension )) { if (not ispresent (v_ReceivedAsp.Signalling.Rrc.Dcch.message_.c1.rrcSetupComplete.criticalExtensions.rrcSetupComplete.nonCriticalExtension.ue_MeasurementsAvailable_r16.logMeasAvailable_r16)) { f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35: UE transmit an RRCSetupComplete message with logMeasAvailable IE not present") } }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); } //@siclog "Step 36-40" siclog@ //Steps 5 to 9 of the generic procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); }//End of f_TC_8_1_6_1_2_12_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_12_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 33" siclog@ //The UE transmits an RRCRequest message on NR Cell 1. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits an RRCSetup message on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 35" siclog@ //@sic R5-230757 sic@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? SRB.receive(car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, *),?)) -> value v_ReceivedAsp; v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if (ispresent (v_ReceivedAsp.Signalling.Rrc.Dcch.message_.c1.rrcSetupComplete.criticalExtensions.rrcSetupComplete.nonCriticalExtension )) { if (not ispresent (v_ReceivedAsp.Signalling.Rrc.Dcch.message_.c1.rrcSetupComplete.criticalExtensions.rrcSetupComplete.nonCriticalExtension.ue_MeasurementsAvailable_r16.logMeasAvailable_r16)) { f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35: UE transmit an RRCSetupComplete message with logMeasAvailable IE not present") } }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); } //@siclog "Step 36-40" siclog@ //Steps 5 to 9 of the generic procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); }//End of f_TC_8_1_6_1_2_12_NR5GC_TestBody