3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230345

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0902 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for IMS 5GS call control test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the implementation of the Prose CR R5-231496, function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 () is called in TCs 7.31 and 7.32 test bodies at step 9. The issue is that after step 10 of A.5.1a, a prompt must be given to the user to answer the MT call, which is step 10A in A.5.1a (and also A.5.1). In the prose, the procedure is mentioned till step 10 of A.5.1a only, where as, it should actually be step 10A, .i.e. the step 19 of the test body of these TCs should actually be calling step 10A of A.5.1a which is missing at the moment (This is correctly done in TC 7.34). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added additional step, step 18A, is added in these two test cases to call step 10A of A.5.1a, so user prompt can be given to answer the MT call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.31.NR5GC, 7.32.NR5GC, 7.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_31_NR5GC_IMS1 |
| --- | --- |
| Reason for change | With the implementation of the Prose CR R5-231496, function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 () is called in TCs 7.31 and 7.32 test bodies at step 9. The issue is that after step 10 of A.5.1a, a prompt must be given to the user to answer the MT call, which is step 10A in A.5.1a (and also A.5.1). In the prose, the procedure is mentioned till step 10 of A.5.1a only, where as, it should actually be step 10A, .i.e. the step 19 of the test body of these TCs should actually be calling step 10A of A.5.1a which is missing at the moment (This is correctly done in TC 7.34). |
| Summary of change | An additional step, step 18A, is added in these two test cases to call step 10A of A.5.1a , so a user prompt can be given to answer the MT call. NOTE: A supporting Prose CR will be raised in the RAN5#99 meeting. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted subject to RAN5 approval |

Before Change

function f_TC_7_31_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end supports but does not send Session-Expires / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(); // @sic R5-231496: steps 1 - 10 of A.5.1a sic@ v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; … … … }

After Change

function f_TC_7_31_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end supports but does not send Session-Expires / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(); // @sic R5-231496: steps 1 - 10 of A.5.1a sic@ v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; … … … }

## Change 2

| Function name | f_TC_7_32_NR5GC_IMS1 |
| --- | --- |
| Reason for change | See change 1. |
| Summary of change | See change 1. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted subject to RAN5 approval |

Before Change

function f_TC_7_32_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(cs_SessionExpires(v_DeltaSec)); // @sic R5-231497: steps 1 - 10 of A.5.1a sic@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; … … … }

After Change

function f_TC_7_32_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(cs_SessionExpires(v_DeltaSec)); // @sic R5-231497: steps 1 - 10 of A.5.1a sic@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; … … … }

## Change 3

| Function name | f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A |
| --- | --- |
| Reason for change | There is an issue in the implementation of the new function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A() that it actually calls itself, resulting the test case execution to be stuck in an infinite recursion. |
| Summary of change | Changed the function called within to function should be changed to call to f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(). |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231498: A.5.1a instead of A.5.1 sic@ */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1 - 10 v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(p_SessionExpires, p_Allow, p_OptionTagsForSupported); // Step 10A f_UT_AnswerCall(MMI); return v_InviteRequestWithSdp; }

After Change

function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231498: A.5.1a instead of A.5.1 sic@ */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1 - 10 v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(p_SessionExpires, p_Allow, p_OptionTagsForSupported); // Step 10A f_UT_AnswerCall(MMI); return v_InviteRequestWithSdp; }